3GPP TSG-RAN WG4 Meeting # 98bis-e	R4-2107129

# Electronic Meeting, 12 – 20 April 2021

Agenda item:	9.1.6

Source:	Keysight Technologies

Title:	Draft LS to RAN5 on Test Time Reduction

Document for:	Approval

# Introduction

This contribution provides the background for the enclosed LS to RAN5 on Test Time Reduction based on the declaration of the antenna configuration. The introductory/background portion of this contribution will be removed in a revision of this LS.

# Discussion

In RAN4#98-e, three companies provided simulations for the minimum required number of grid points for the beam peak search with the antenna array configuration relaxed from 8x2 to 4x2 to determine the level of test time reduction [1][2][3]. The results generally showed that the number of grid points could be reduced by ~1/3. Additionally, some test time improvements can be expected for TRP and spherical coverage tests as well.

As documented in the WF [4], it was proposed to send an LS to RAN5 with the intention to

- Consider a vendor declaration related to the antenna configuration to allow relaxation of measurement grids and thus test time improvements

- Leave all system-related assumptions (and thus MTSU) unchanged, i.e., based on the original 8x2 antenna assumption

Enclosed is a draft LS to RAN5 taking the decisions from the last meeting into account.

Proposal 1: Send the draft LS enclosed to RAN5 regarding the test time reduction based on the antenna configuration.

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Send the draft LS enclosed to RAN5 regarding the test time reduction based on the antenna configuration.

# References

- R4-2100895, Discussion on FR2 test time reduction, Samsung, 3GPP TSG-RAN WG4 Meeting #98-e, February 2021

- R4-2102401, Analysis on reducing test time, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #98-e, February 2021

- R4-2102618, On Test Time Enhancements based on different Antenna Array Assumptions, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #98-e, February 2021

- R4-2103920, WF on ETC (objective4) and test time reduction(objective6), vivo, 3GPP TSG-RAN WG4 Meeting #98-e, February 2021

3GPP TSG-RAN WG4 Meeting # 98bis-e	R4-210XXXX

# Electronic Meeting, 12 – 20 April 2021

Title:	LS on Test Time Reduction based on Antenna Configuration Declaration

Release:	Rel-17

Work Item:	FS_FR2_enhTestMethods

Source:	RAN WG4

To:	RAN WG5

CC:	-

Contact Person:

#### Name: Thorsten Hertel

#### E-mail Address:	thorsten dot hertel @ keysight dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Background Information

The enhanced testability SI (Acronym: FS_FR2_enhTestMethods; Latest approved SID: RP-210633) includes one topic/objected related to test time reduction

5. Study testability enhancements to reduce test time - Including RF test method enhancement with reduced test time, and possible test time saving approach for UE Demodulation test and RRM test

One suitable approach to reduce overall test time for conformance testing was related to the antenna array assumption; other approaches are still under discussion. In RAN4, antenna array configurations ranging from 4x1 to 8x2 were considered for testability and preliminary MU assumptions. In the end, the 8x2 antenna array assumptions were selected as worst case which was also adopted in RAN5 to finalize MUs, the Maximum Test System Uncertainty (MTSU), and Test Tolerances (TTs).

Based on feedback from OEMs in RAN4, many PC3 UE implementations utilize antenna array configurations with fewer elements than 8x2, e.g., one OEM highlighted that “only 4 antenna elements are the dominant configuration in commercial PC3 UE.” Thus, measurement grid analyses were performed for the beam peak searches, a pre-requisite for all NR FR2 UE RF and many RRM test cases and a large contributor in terms of overall test time, to quantify the test time reduction impact of the relaxed 4x2 array configuration. Those results for the minimum number of grid points of the TX/RX beam peak searches are summarized below; clearly, the reduction of grid points by ~1/3 yields a significant improvement in test time.

| Antenna Assumption Grid Type | 8x2 | 4x2 | Factor of Improvement |
| --- | --- | --- | --- |
| Constant-Step Size | 1106 | 366 | 3.0 |
| Constant-Density | 800 | 275 | 2.9 |

Since RAN5 has finalized maximum test system uncertainties (MTSUs) and test tolerances (TTs) for many test cases already, it is not suggested to change the baseline antenna array assumptions at this point as this will have significant impact in RAN5 and industry since changes in MU/MTSU could have impact on certifications and test platform validations. It is furthermore proposed for RAN5 to keep all system-related assumptions, e.g., related to max antenna aperture of D=5cm for PC3, based on the 8x2 antenna array assumptions.

Given the improvement in test time, it is suggested for RAN5 to support a relaxation of the beam peak search measurement grid requirements for the beam peak searches, TRP, and spherical coverage based on an optional vendor declaration.

2. Actions

To RAN WG5

RAN4 respectfully asks RAN5 to take the above information into consideration to support test time reduction efforts and to determine the min required number of grid points for TRP, spherical coverage, beam peak search for the 4x2 antenna configuration.

3. Date of Next TSG-RAN4 Meetings:

TSG RAN WG4 Meeting #99-e                			19 – 27 May 2021         			E-meeting

TSG RAN WG4 Meeting #100-e                	August 2021							E-meeting