3GPP TSG RAN WG4 Meeting #98bis-e	R4-2107136

Electronic Meeting, April 12-20, 2021

Agenda Item:	5.3.3.4

Source: 	Ericsson

Title:	Analysis of CSI-RS based RLM tests for LA IAB-MT

Document for:	Discussion

- Introduction

In the contribution we analyze the following CSI-RS based RLM test cases assigned according to the work split [1].

| RRM Test cases | Related RRM Requirements | Applicability | Company |
| --- | --- | --- | --- |
| RLM OOS with CSI-RS in FR1 | 12.3.1.3 Requirements for CSI-RS based radio link monitoring | 1-H LA | Ericsson |
| RLM OOS with CSI-RS in FR2 |  | 2-O LA |  |
| RLM IS with CSI-RS in FR1 |  | 1-H LA |  |
| RLM IS with CSI-RS in FR2 |  | 2-O LA |  |

# Analysis of RLM test cases

The aim of the CSI-RS based RLM test cases is to very the out of sync and in-sync requirements for CSI-RS based RLM defined for IAB-MT in section 12.3.1.3 of TS 38.174.

The methodology of the existing CSI-RS based RLM test cases defined for the UE in annex A.6 of TS 38.133 (i.e. tests in clauses A.6.5.1.5- A.6.5.1.8) can be reused for IAB-MT. However, the RLM core requirements for IAB-MT are diffent compared to that of the UE. Therefore, test parameters such as test times need to be adapted for IAB-MT. This in turn depends on in-sync and out-of-sync evaluation periods. They can be derived from the core requirements in TS 38.174 and based on parameters used in the test cases:

Table 12.3.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 [TS 38.174]

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200 K1, Ceil(Mout×P K1)×TCSI-RS) | Max(100 K1, Ceil(Min×P K1) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. |  |  |

Table 12.3.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 [TS 38.174]

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200 K2, Ceil(Mout×P K2)×TCSI-RS) | Max(100 K2, Ceil(Min×P K2) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. |  |  |

In the above requirements the scaling factors are defined as follows:

- for FR1 with scaling factor K1 = 5.

- for FR2 with scaling factor K2 = 3.

- Mout = 20 and Min = 10

For FR1, P=1. In FR2, P=1 assuming the RLM-RS resource does not overlap with SMTC occasion.

In both FR1 and FR2 tests, TCSI-RS = 20 ms.

Based on the above parameters the OOS and IS evaluation periods in the tests can be derived as shown in table 1:

Table 1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 and FR2 in tests

| FR | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| FR1 | 2000 ms | 1000 ms |
| FR2 | 1200 ms | 600 ms |

- Observation 1: The OOS/IS evaluation periods for FR2 are shorter than those for FR1. This is counterintuitive given that in FR1 the IAB-MT needs to perform beam sweeping before measurement sampling.

- Observation 2: The beam sweeping factor (N) is missing for TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 in the CSI-RS based RLM core requirements in TS 38.174.

- Proposal 2: The CSI-RS RLM tests for IAB-MT are based on the same methodology used in corresponding CSI-RS based RLM tests defined for the UE in TS 38.133.

- Proposal 2: The test times CSI-RS RLM tests for IAB-MT are kept TBD until the core requirements for FR2 are clarified.

- Summary

In this paper we have analyzed the CSI-RS RLM test cases for IAB-MT. The main observation and proposals are as follows:

- Observation 1: The OOS/IS evaluation periods for FR2 are shorter than those for FR1. This is counterintuitive given that in FR1 the IAB-MT needs to perform beam sweeping before measurement sampling.

- Observation 2: The beam sweeping factor (N) is missing for TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 in the CSI-RS based RLM core requirements in TS 38.174.

- Proposal 2: The CSI-RS RLM tests for IAB-MT are based on the same methodology used in corresponding CSI-RS based RLM tests defined for the UE in TS 38.133.

- Proposal 2: The test times CSI-RS RLM tests for IAB-MT are kept TBD until the core requirements for FR2 are clarified.

CR to 38.174 on defining the CSI-RS based RLM test cases is provided in [1].

- References

- R4-2104091, WF on test cases for IAB-MTs, ZTE.

- R4-2107137, CSI-RS based RLM tests for LA IAB-MT in TS 38.174, Ericsson