3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107178

# Electronic Meeting, Apr. 12-20, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Link simulation results for SRS-RSRP accuracy

Agenda item:	5.5.2.3.2

Document for:	Discussion

Introduction

At RAN4 #98-e, further discussion was carried out on how to specify gNB accuracy requirements for NR positioning in TS 38.133 and agreements were captured in the WF [1].

Following agreements were achieved in view of simulating link performance:

- Accuracy requirements for gNB measurements shall be defined for the following Ês/Iot values:

  - Ês/Iot1 [dB] (High Ês/Iot) = 3 dB for FR1 and FR2

  - Ês/Iot2 [dB] (Low Ês/Iot) = -13 dB for FR1 and FR2.

- gNB accuracy requirements shall be defined for group of SRS BWs

  - grouping of SRS BWs will be decided based on link simulation results

- Baseline SRS-RSRP measurement accuracy without margin is based on link simulation results

- Link simulations are based on assumptions in R4-2012142 and agreements in this meeting (e.g. on propagation channel).

This contribution lists our baseline simulation results for SRS-RSRP accuracy for agreed SRS configurations in [2].

Simulation assumptions

The SRS configurations including the settings for SRS comb size and # SRS symbols for FR1 and FR 2 in [2] were simulated. In addition, due to the agreement on SRS BW grouping, bandwidth grouping based on same FFT size has been assumed as depicted in [3], which is slightly modified according to available SRS PRB numbers for SCS=15 kHz in Table 1 with a lower bound named PRB start in terms of number of PRBs, and with an upper bound, named PRB end in terms of number of PRBs. The simulation was carried out for the lower bound for each SRS BW group.

Table 1: Proposed SRS BW grouping for FR1, SCS= 15 kHz.

| SCS (kHz) | PRB num | BW (MHz) |
| --- | --- | --- |
| 15 | 24-40 | 5-7.5 |
|  | 44-84 | 10-15 |
|  | 88-168 | 15-30 |
|  | 176-264 | 30-50 |

In addition, the number of measurement samples was varied: 1, 2 and 4 samples were taken and then averaged. AWGN radio channel was selected (same as for evaluating gNB Rx-Tx time accuracy). No RF and implementation impairments are included.

Simulation results

Simulation results for SRS-RSRP accuracy for the SRS configurations agreed in [2] are depicted below for FR1 and FR2. The results are reported for the symmetric 90 percentile CDF, i.e. from 5% of the CDF to 95% of the CDF for given Ês/Iot ratios (+3 dB, -6 dB. -13 dB) of the SRS-RSRP error with

SRS-RSRP error = estimated SRS-RSRP – ideal SRS-RSRP (based on perfect channel knowledge).

The maximum absolute SRS-RSRP error from the 5% and 95% CDF points is determined according to [2] and reported in dB scale. Results are derived from the lower bound of each SRS BW group.

FR1

SCS = 15 kHz

The SRS-RSRP accuracy for AWGN channel, derived from the symmetric 90-percentile of the SRS-RSRP error CDF according to [2], is listed in Tables 1a, 1b and 1c for 1, 2 and 4 samples, respectively.

Table 1a: SRS-RSRP accuracy in dB (AWGN, SCS= 15 kHz), 1 sample.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 24 - 40 / 5 - 7.5 | 512 | 2 /2 | 0.33 | 1.31 | 3.67 |
|  |  | 4 / 4 | 0.33 | 1.25 | 3.68 |
|  |  | 8 / 8 | 0.33 | 1.27 | 3.62 |
|  |  | 8 / 12 | 0.27 | 1.06 | 3.17 |
| 44 - 84 / 10 - 15 | 1024 | 2 /2 | 0.23 | 0.92 | 2.87 |
|  |  | 4 / 4 | 0.24 | 0.91 | 2.84 |
|  |  | 8 / 8 | 0.23 | 0.92 | 2.87 |
|  |  | 8 / 12 | 0.20 | 0.76 | 2.45 |
| 88 - 168 / 15 - 30 | 2048 | 2 /2 | 0.17 | 0.66 | 2.19 |
|  |  | 4 / 4 | 0.17 | 0.66 | 2.16 |
|  |  | 8 / 8 | 0.17 | 0.65 | 2.18 |
|  |  | 8 / 12 | 0.14 | 0.51 | 1.94 |
| 176 - 264 / 30 – 50 | 4096 | 2 /2 | 0.12 | 0.45 | 1.67 |
|  |  | 4 / 4 | 0.12 | 0.45 | 1.69 |
|  |  | 8 / 8 | 0.12 | 0.46 | 1.72 |
|  |  | 8 / 12 | 0.10 | 0.37 | 1.46 |

Table 1b: SRS-RSRP accuracy in dB (AWGN, SCS= 15 kHz), 2 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 24 - 40 / 5 - 7.5 | 512 | 2 /2 | 0.23 | 0.85 | 3.14 |
|  |  | 4 / 4 | 0.23 | 0.83 | 3.14 |
|  |  | 8 / 8 | 0.23 | 0.82 | 3.09 |
|  |  | 8 / 12 | 0.19 | 0.70 | 2.62 |
| 44 - 84 / 10 - 15 | 1024 | 2 /2 | 0.17 | 0.64 | 2.35 |
|  |  | 4 / 4 | 0.17 | 0.62 | 2.35 |
|  |  | 8 / 8 | 0.17 | 0.62 | 2.40 |
|  |  | 8 / 12 | 0.14 | 0.52 | 2.00 |
| 88 - 168 / 15 - 30 | 2048 | 2 /2 | 0.12 | 0.44 | 1.73 |
|  |  | 4 / 4 | 0.12 | 0.46 | 1.66 |
|  |  | 8 / 8 | 0.12 | 0.44 | 1.66 |
|  |  | 8 / 12 | 0.10 | 0.36 | 1.36 |
| 176 - 264 / 30 – 50 | 4096 | 2 /2 | 0.08 | 0.31 | 1.23 |
|  |  | 4 / 4 | 0.09 | 0.31 | 1.23 |
|  |  | 8 / 8 | 0.08 | 0.32 | 1.19 |
|  |  | 8 / 12 | 0.06 | 0.26 | 0.97 |

Table 1c: SRS-RSRP accuracy in dB (AWGN, SCS= 15 kHz), 4 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 24 - 40 / 5 - 7.5 | 512 | 2 /2 | 0.16 | 0.62 | 2.71 |
|  |  | 4 / 4 | 0.16 | 0.61 | 2.72 |
|  |  | 8 / 8 | 0.16 | 0.60 | 2.68 |
|  |  | 8 / 12 | 0.13 | 0.50 | 2.20 |
| 44 - 84 / 10 - 15 | 1024 | 2 /2 | 0.12 | 0.44 | 1.97 |
|  |  | 4 / 4 | 0.11 | 0.43 | 1.94 |
|  |  | 8 / 8 | 0.12 | 0.49 | 2.01 |
|  |  | 8 / 12 | 0.10 | 0.38 | 1.60 |
| 88 - 168 / 15 - 30 | 2048 | 2 /2 | 0.08 | 0.32 | 1.37 |
|  |  | 4 / 4 | 0.09 | 0.32 | 1.35 |
|  |  | 8 / 8 | 0.08 | 0.32 | 1.37 |
|  |  | 8 / 12 | 0.07 | 0.26 | 1.07 |
| 176 - 264 / 30 – 50 | 4096 | 2 /2 | 0.06 | 0.23 | 0.93 |
|  |  | 4 / 4 | 0.06 | 0.21 | 0.89 |
|  |  | 8 / 8 | 0.06 | 0.22 | 0.93 |
|  |  | 8 / 12 | 0.05 | 0.18 | 0.75 |

SCS = 30 kHz

The SRS-RSRP accuracy for AWGN channel, derived from the symmetric 90-percentile of the SRS-RSRP error CDF according to [2], is listed in Tables 2a, 2b and 2c for 1, 2 and 4 samples, respectively.

Table 2a: SRS-RSRP accuracy in dB (AWGN, SCS= 30 kHz), 1 sample.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 48 - 84 / 20 - 30 | 1024 | 2 /2 | 0.23 | 0.89 | 2.72 |
|  |  | 4 / 4 | 0.23 | 0.89 | 2.73 |
|  |  | 8 / 8 | 0.23 | 0.88 | 2.78 |
|  |  | 8 / 12 | 0.19 | 0.73 | 2.31 |
| 88 - 168 / 30 - 60 | 2048 | 2 /2 | 0.17 | 0.65 | 2.17 |
|  |  | 4 / 4 | 0.17 | 0.66 | 2.25 |
|  |  | 8 / 8 | 0.17 | 0.64 | 2.17 |
|  |  | 8 / 12 | 0.14 | 0.51 | 1.87 |
| 176 - 272 / 60 – 100 | 4096 | 2 /2 | 0.12 | 0.46 | 1.74 |
|  |  | 4 / 4 | 0.12 | 0.44 | 1.67 |
|  |  | 8 / 8 | 0.12 | 0.45 | 1.71 |
|  |  | 8 / 12 | 0.10 | 0.37 | 1.44 |

Table 2b: SRS-RSRP accuracy in dB (AWGN, SCS= 30 kHz), 2 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 48 - 84 / 20 - 30 | 1024 | 2 /2 | 0.16 | 0.61 | 2.26 |
|  |  | 4 / 4 | 0.16 | 0.59 | 2.23 |
|  |  | 8 / 8 | 0.16 | 0.60 | 2.24 |
|  |  | 8 / 12 | 0.13 | 0.49 | 1.87 |
| 88 - 168 / 30 - 60 | 2048 | 2 /2 | 0.12 | 0.46 | 1.71 |
|  |  | 4 / 4 | 0.12 | 0.46 | 1.67 |
|  |  | 8 / 8 | 0.12 | 0.45 | 1.72 |
|  |  | 8 / 12 | 0.10 | 0.37 | 1.42 |
| 176 - 272 / 60 – 100 | 4096 | 2 /2 | 0.08 | 0.32 | 1.25 |
|  |  | 4 / 4 | 0.08 | 0.32 | 1.23 |
|  |  | 8 / 8 | 0.08 | 0.31 | 0.19 |
|  |  | 8 / 12 | 0.07 | 0.26 | 0.99 |

Table 2c: SRS-RSRP accuracy in dB (AWGN, SCS= 30 kHz), 4 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 48 - 84 / 20 - 30 | 1024 | 2 /2 | 0.11 | 0.44 | 1.87 |
|  |  | 4 / 4 | 0.12 | 0.43 | 1.85 |
|  |  | 8 / 8 | 0.11 | 0.42 | 1.88 |
|  |  | 8 / 12 | 0.10 | 0.34 | 1.51 |
| 88 - 168 / 30 - 60 | 2048 | 2 /2 | 0.08 | 0.33 | 1.40 |
|  |  | 4 / 4 | 0.08 | 0.32 | 1.31 |
|  |  | 8 / 8 | 0.08 | 0.32 | 1.37 |
|  |  | 8 / 12 | 0.07 | 0.27 | 1.14 |
| 176 - 272 / 60 – 100 | 4096 | 2 /2 | 0.06 | 0.22 | 0.97 |
|  |  | 4 / 4 | 0.06 | 0.23 | 0.94 |
|  |  | 8 / 8 | 0.06 | 0.22 | 0.91 |
|  |  | 8 / 12 | 0.05 | 0.19 | 0.77 |

FR2

SCS = 120 kHz

The SRS-RSRP accuracy for AWGN channel, derived from the symmetric 90-percentile of the SRS-RSRP error CDF according to [2], is listed in Tables 3a, 3b and 3c for 1, 2 and 4 samples, respectively.

Table 3a: SRS-RSRP accuracy in dB (AWGN, SCS= 120 kHz), 1 sample.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 32 - 40 / 50 - 60 | 512 | 2 /2 | 0.64 | 1.50 | 3.11 |
|  |  | 4 / 4 | 0.68 | 1.54 | 3.12 |
|  |  | 8 / 8 | 0.77 | 1.69 | 3.04 |
|  |  | 8 / 12 | 0.72 | 1.45 | 2.75 |
| 44 - 84 / 60 - 120 | 1024 | 2 /2 | 0.60 | 1.33 | 2.65 |
|  |  | 4 / 4 | 0.66 | 1.41 | 2.64 |
|  |  | 8 / 8 | 0.68 | 1.45 | 2.66 |
|  |  | 8 / 12 | 0.63 | 1.23 | 2.62 |
| ≥ 88 / ≥ 120 | 2048 | 2 /2 | 0.54 | 1.07 | 2.57 |
|  |  | 4 / 4 | 0.59 | 1.10 | 2.60 |
|  |  | 8 / 8 | 0.62 | 1.17 | 2.66 |
|  |  | 8 / 12 | 0.60 | 1.02 | 2.51 |

Table 3b: SRS-RSRP accuracy in dB (AWGN, SCS= 120 kHz), 2 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 32 - 40 / 50 - 60 | 512 | 2 /2 | 0.55 | 1.10 | 2.57 |
|  |  | 4 / 4 | 0.58 | 1.15 | 2.56 |
|  |  | 8 / 8 | 0.67 | 1.22 | 2.49 |
|  |  | 8 / 12 | 0.65 | 1.13 | 2.00 |
| 44 - 84 / 60 - 120 | 1024 | 2 /2 | 0.53 | 1.02 | 2.14 |
|  |  | 4 / 4 | 0.59 | 1.10 | 2.11 |
|  |  | 8 / 8 | 0.61 | 1.23 | 2.10 |
|  |  | 8 / 12 | 0.57 | 0.98 | 1.67 |
| ≥ 88 / ≥ 120 | 2048 | 2 /2 | 0.49 | 0.85 | 1.67 |
|  |  | 4 / 4 | 0.54 | 0.88 | 1.64 |
|  |  | 8 / 8 | 0.58 | 0.82 | 1.70 |
|  |  | 8 / 12 | 0.56 | 0.85 | 1.62 |

Table 3c: SRS-RSRP accuracy in dB (AWGN, SCS= 120 kHz), 4 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 32 - 40 / 50 - 60 | 512 | 2 /2 | 0.50 | 0.87 | 2.15 |
|  |  | 4 / 4 | 0.53 | 0.91 | 2.13 |
|  |  | 8 / 8 | 0.62 | 1.01 | 2.07 |
|  |  | 8 / 12 | 0.59 | 0.90 | 1.59 |
| 44 - 84 / 60 - 120 | 1024 | 2 /2 | 0.46 | 0.77 | 1.73 |
|  |  | 4 / 4 | 0.53 | 0.85 | 1.72 |
|  |  | 8 / 8 | 0.55 | 0.90 | 1.72 |
|  |  | 8 / 12 | 0.53 | 0.80 | 1.31 |
| ≥ 88 / ≥ 120 | 2048 | 2 /2 | 0.45 | 0.69 | 1.12 |
|  |  | 4 / 4 | 0.50 | 0.73 | 1.07 |
|  |  | 8 / 8 | 0.53 | 0.78 | 1.12 |
|  |  | 8 / 12 | 0.53 | 0.72 | 1.11 |

Discussion

From the simulation results reported above, we observe a clear dependency of the SRS-RSRP accuracy on the Ês/Iot ratio in the considered range +3 dB …-13 dB, whilst performance impact due to increased SRS bandwidth is rather limited and there is no dependency neither on the SRS comb size nor on the number of continuous SRS symbols. Thus, following observation is made:

The SRS-RSRP accuracy for all SRS configurations depends majorly on the Ês/Iot ratio in the considered range +3 dB …-13 dB, and lesser on the SRS BW (a clear dependency is observed for low Ês/Iot condition only), whilst no dependency on SRS comb size and number of continuous SRS symbols is observed.

It is also observed that the number of samples has an impact on the SRS-RSRP accuracy, especially for the low Ês/Iot condition (-13 dB), where an improvement is of around 0.5 dB is seen for 2 samples and around 0.9 dB for 4 samples.

The SRS-RSRP accuracy can be improved for the low Ês/Iot condition adopting multiple shots (e.g. 2 or 4) compared to single shot by around 0.5 dB (two shots) and 0.9 dB (four shots).

Following proposal is made:

The provided SRS-RSRP accuracy results are taken into account in the discussion on SRS BW grouping and other SRS configuration parameter grouping and for identifying the number of shots.

Conclusion

This contribution lists our baseline simulation results for SRS-RSRP accuracy for agreed SRS configurations in [2] as evaluated in section 3.

Based on above evaluations, following observations are made:

- The SRS-RSRP accuracy for all SRS configurations depends majorly on the Ês/Iot ratio in the considered range +3 dB …-13 dB, and lesser on the SRS BW (a clear dependency is observed for low Ês/Iot condition only), whilst no dependency on SRS comb size and number of continuous SRS symbols is observed.

The SRS-RSRP accuracy can be improved for the low Ês/Iot condition adopting multiple shots (e.g. 2 or 4) compared to single shot by around 0.5 dB (two shots) and 0.9 dB (four shots).

Following proposal for agreement is made:

- The provided SRS-RSRP accuracy results are taken into account in the discussion on SRS BW grouping and other SRS configuration parameter grouping and for identifying the number of shots.

References

- R4-2103587, "WF on gNB positioning measurement requirements", source: Ericsson

- R4-2012142, "Link simulation assumptions for deriving positioning SRS configurations", source: Nokia

- R4-2102780, “Discussion on gNB Rx-Tx time difference requirements”, source: Huawei, HiSilicon