3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107179

# Electronic Meeting, Apr. 12-20, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Link simulation results for gNB Rx-Tx time difference accuracy

Agenda item:	5.5.2.3.3

Document for:	Discussion

Introduction

At RAN4 #98-e, further discussion was carried out on how to specify gNB accuracy requirements for NR positioning in TS 38.133 and agreements were captured in the WF [1].

Following agreements were achieved in view of simulating link performance:

- Accuracy requirements for gNB measurements shall be defined for the following Ês/Iot values:

  - Ês/Iot1 [dB] (High Ês/Iot) = 3 dB for FR1 and FR2

  - Ês/Iot2 [dB] (Low Ês/Iot) = -13 dB for FR1 and FR2.

- gNB accuracy requirements shall be defined for group of SRS BWs

  - grouping of SRS BWs will be decided based on link simulation results

- Baseline timing measurement accuracy without margin is based on link simulation results

- gNB Rx-Tx measurement accuracy requirements apply in AWGN.

- Link simulations are based on assumptions in R4-2012142 and agreements in this meeting (e.g. on propagation channel).

This contribution lists our baseline simulation results for gNB Rx-Tx time difference accuracy for agreed SRS configurations in [2].

Simulation assumptions

The SRS configurations including the settings for SRS comb size and # SRS symbols for FR1 and FR 2 in [2] were simulated. In addition, due to the agreement on SRS BW grouping, bandwidth grouping based on same FFT size has been assumed as depicted in [3], which is slightly modified according to available SRS PRB numbers for SCS=15 kHz in Table 1 with a lower bound named PRB start in terms of number of PRBs, and with an upper bound, named PRB end in terms of number of PRBs. The simulation was carried out for the lower bound for each SRS BW group.

Table 1: Proposed SRS BW grouping for FR1, SCS= 15 kHz.

| SCS (kHz) | PRB num | BW (MHz) |
| --- | --- | --- |
| 15 | 24-40 | 5-7.5 |
|  | 44-84 | 10-15 |
|  | 88-168 | 15-30 |
|  | 176-264 | 30-50 |

In addition, the number of measurement samples was varied: 1, 2 and 4 samples were taken and then averaged. No RF and implementation impairments are included.

Simulation results

Simulation results for gNB Rx-Tx time difference accuracy for the SRS configurations agreed in [2] are depicted below for FR1 and FR2. The results are reported for 90 percentile CDF of the timing error, i.e. 90% of measurements for given Ês/Iot ratios (+3 dB, -6 dB. -13 dB) experience a timing error less than the given figure. Results are derived for the lower bound of each SRS BW group.

FR1

SCS = 15 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the 90-percentile of the CDF for measured timing error according to [2], is listed in Tables 1a, 1b and 1c for 1, 2 and 4 samples, respectively.

Table 1a: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 15 kHz), 1 sample.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 24 - 40 / 5 - 7.5 | 512 | 2 /2 | 82 | 82 | 82 |
|  |  | 4 / 4 | 46 | 82 | 82 |
|  |  | 8 / 8 | 18 | 46 | 82 |
|  |  | 8 / 12 | 18 | 46 | 46 |
| 44 - 84 / 10 - 15 | 1024 | 2 /2 | 46 | 46 | 46 |
|  |  | 4 / 4 | 18 | 18 | 46 |
|  |  | 8 / 8 | 18 | 18 | 18 |
|  |  | 8 / 12 | 18 | 18 | 18 |
| 88 - 168 / 15 - 30 | 2048 | 2 /2 | 18 | 18 | 18 |
|  |  | 4 / 4 | 14 | 18 | 18 |
|  |  | 8 / 8 | 2 | 2 | 14 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| 176 - 264 / 30 – 50 | 4096 | 2 /2 | 14 | 14 | 18 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 2 |

Table 1b: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 15 kHz), 2 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 24 - 40 / 5 - 7.5 | 512 | 2 /2 | 82 | 82 | 82 |
|  |  | 4 / 4 | 46 | 46 | 82 |
|  |  | 8 / 8 | 18 | 18 | 50 |
|  |  | 8 / 12 | 18 | 18 | 50 |
| 44 - 84 / 10 - 15 | 1024 | 2 /2 | 46 | 46 | 46 |
|  |  | 4 / 4 | 18 | 18 | 18 |
|  |  | 8 / 8 | 14 | 18 | 18 |
|  |  | 8 / 12 | 14 | 18 | 18 |
| 88 - 168 / 15 - 30 | 2048 | 2 /2 | 18 | 18 | 18 |
|  |  | 4 / 4 | 14 | 14 | 18 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 6 |
| 176 - 264 / 30 – 50 | 4096 | 2 /2 | 14 | 14 | 14 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |

Table 1c: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 15 kHz), 4 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 24 - 40 / 5 - 7.5 | 512 | 2 /2 | 82 | 82 | 82 |
|  |  | 4 / 4 | 46 | 46 | 50 |
|  |  | 8 / 8 | 18 | 18 | 46 |
|  |  | 8 / 12 | 18 | 18 | 34 |
| 44 - 84 / 10 - 15 | 1024 | 2 /2 | 46 | 46 | 46 |
|  |  | 4 / 4 | 18 | 18 | 18 |
|  |  | 8 / 8 | 14 | 14 | 14 |
|  |  | 8 / 12 | 14 | 14 | 14 |
| 88 - 168 / 15 - 30 | 2048 | 2 /2 | 18 | 18 | 18 |
|  |  | 4 / 4 | 14 | 14 | 14 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| 176 - 264 / 30 – 50 | 4096 | 2 /2 | 14 | 14 | 14 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |

SCS = 30 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the 90-percentile of the CDF for measured timing error according to [2], is listed in Tables 2a, 2b and 2c for 1, 2 and 4 samples, respectively.

Table 2a: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 30 kHz), 1 sample.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 48 - 84 / 20 - 30 | 1024 | 2 /2 | 18 | 18 | 18 |
|  |  | 4 / 4 | 14 | 18 | 18 |
|  |  | 8 / 8 | 2 | 2 | 14 |
|  |  | 8 / 12 | 2 | 2 | 14 |
| 88 - 168 / 30 - 60 | 2048 | 2 /2 | 14 | 14 | 18 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 6 |
| 176 - 272 / 60 – 100 | 4096 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |

Table 2b: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 30 kHz), 2 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 48 - 84 / 20 - 30 | 1024 | 2 /2 | 18 | 18 | 18 |
|  |  | 4 / 4 | 14 | 14 | 18 |
|  |  | 8 / 8 | 2 | 2 | 10 |
|  |  | 8 / 12 | 2 | 2 | 6 |
| 88 - 168 / 30 - 60 | 2048 | 2 /2 | 14 | 14 | 14 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| 176 - 272 / 60 – 100 | 4096 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |

Table 2c: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 30 kHz), 4 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 48 - 84 / 20 - 30 | 1024 | 2 /2 | 18 | 18 | 18 |
|  |  | 4 / 4 | 14 | 14 | 14 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 6 |
| 88 - 168 / 30 - 60 | 2048 | 2 /2 | 14 | 14 | 14 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| 176 - 272 / 60 – 100 | 4096 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |

FR2

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the 90-percentile of the CDF for measured timing error according to [2], is listed in Tables 3a, 3b and 3c for 1, 2 and 4 samples, respectively.

SCS = 120 kHz

Table 3a: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 120 kHz), 1 sample.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 32 - 40 / 50 - 60 | 512 | 2 /2 | 14 | 14 | 18 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 6 |
| 44 - 84 / 60 - 120 | 1024 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 6 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| ≥ 88 / ≥ 120 | 2048 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 0 | 0 | 2 |
|  |  | 8 / 12 | 0 | 0 | 0 |

Table 3b: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 120 kHz), 2 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 32 - 40 / 50 - 60 | 512 | 2 /2 | 14 | 14 | 14 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 6 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| 44 - 84 / 60 - 120 | 1024 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| ≥ 88 / ≥ 120 | 2048 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 0 | 0 | 1 |
|  |  | 8 / 12 | 0 | 0 | 1 |

Table 3c: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 120 kHz), 4 samples.

| PRB start – end / BW (MHz) | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -6 dB | Ês/Iot = -13 dB |
| 32 - 40 / 50 - 60 | 512 | 2 /2 | 14 | 14 | 14 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 4 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| 44 - 84 / 60 - 120 | 1024 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 2 | 2 | 2 |
|  |  | 8 / 12 | 2 | 2 | 2 |
| ≥ 88 / ≥ 120 | 2048 | 2 /2 | 2 | 2 | 2 |
|  |  | 4 / 4 | 2 | 2 | 2 |
|  |  | 8 / 8 | 0 | 0 | 0.5 |
|  |  | 8 / 12 | 0 | 0 | 0.5 |

Discussion

From the simulation results reported above, we observe a dependency of the gNB Rx-Tx time difference accuracy on the SRS bandwidth, on SRS comb size and number of continuous SRS symbols, whilst performance impact due to Ês/Iot ratio in the considered range +3 dB …-13 dB is rather limited. Thus, following observation is made:

The gNB Rx-Tx time difference accuracy for all SRS configurations depends majorly on the SRS bandwidth, on SRS comb size and number of continuous SRS symbols.

It is also observed that the number of samples has an impact on the gNB Rx-Tx time difference accuracy, especially for the low Ês/Iot condition (-13 dB), where an improvement is seen.

The gNB Rx-Tx time difference accuracy can be improved for the low Ês/Iot condition adopting multiple shots (e.g. 2 or 4) compared to single shot.

Following proposal is made:

The provided gNB Rx-Tx time difference accuracy results are taken into account in the discussion on SRS BW grouping and other SRS configuration parameter grouping and for identifying the number of shots.

Conclusion

This contribution lists our baseline simulation results for gNB Rx-Tx time difference accuracy for agreed SRS configurations in [2] as evaluated in section 3.

Based on above evaluations, following observations are made:

- The gNB Rx-Tx time difference accuracy for all SRS configurations depends majorly on the SRS bandwidth, on SRS comb size and number of continuous SRS symbols.

- The gNB Rx-Tx time difference accuracy can be improved for the low Ês/Iot condition adopting multiple shots (e.g. 2 or 4) compared to single shot.

Following proposal for agreement is made:

- The provided gNB Rx-Tx time difference accuracy results are taken into account in the discussion on SRS BW grouping and other SRS configuration parameter grouping and for identifying the number of shots.

References

- R4-2103587, "WF on gNB positioning measurement requirements", source: Ericsson

- R4-2012142, "Link simulation assumptions for deriving positioning SRS configurations", source: Nokia

- R4-2102780, “Discussion on gNB Rx-Tx time difference requirements”, source: Huawei, HiSilicon