3GPP TSG-RAN WG4 Meeting #98bis-e				  									  R4-2107195 Electronic Meeting, Apr. 12-20, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	HAPS adjacent channel coexistence simulation results

Agenda item: 	8.8.2.2 Simulation results	[NR_NTN_solutions-Core]

Document for:	Discussion and Approval

# Introduction

The NTN WI has been started in RAN4#98-e. The discussions of the general aspect of NTN, including use cases, deployment scenarios, architecture, frequency bands, are summarized in [1]. The discussions on NTN coexistence study are summarized in [2]. Agreed way forward for issues in the general aspect and coexistence study are respectively documented in [3] and [4]. With respect to HAPS, the agreements in [3][4] are listed below :

Furthermore, the chairman’s note [5] captures the agreed scenarios of initial NTN coexistence simulations as follows:

Agreements: RAN4 agreed to take following scenarios for initial simulation alignment purpose in Q2 2021: TN deployment: NR only with Rural, Urban Macro NTN deployment: GEO, LEO-600, LEO-1200, HAPS Satellite Set: Set1 The detailed set as following: No.; Frq.; TN; TN scenario; NTN; Prioritize (Selected set for simulator alignment in Q2’ 2021). 1; 2GHz; NR; Rural; GEO; THALES, Nokia. 2; 2GHz; NR; Rural; LEO 600km; THALES, Nokia. 3; 2GHz; NR; Rural; LEO 1200km. 4; 2GHz; NR; Urban macro; GEO; THALES, Nokia. 5; 2GHz; NR; Urban macro; LEO 600km; THALES, Nokia. 6; 2GHz; NR; Urban macro; LEO 1200km. 25; 2GHz; NR; Rural; HAPS; Nokia. 26; 2GHz; NR; Urban macro; HAPS; Nokia. No.; Frq.; NTN; NTN. 1; 2GHz; GEO; GEO. 2; 2GHz; GEO; LEO 600km. 5; 2GHz; LEO 600km; LEO 600km. 9; 2GHz; LEO 1200km; LEO 1200km. 10; 2GHz; HAPS; HAPS. Companies are encouraged to bring simulation results at least for above cases in Q2’ 2021 for simulator alignment purpose.

and noted that the simulation assumption provided in [5] “was for simulation alignment purpose; companies are encouraged to follow such simulation assumption to provide results in April RAN4 meeting.”

In this contribution, we present the initial results of HAPS coexistence simulations.

# Simulation results

Coexistence simulations were conducted for the scenarios illustrated in Figure 1. The two coexistence systems can be (a) terrestrial NR network and HAPS network; (b) HAPS network and HAPS network. The terrestrial NR network has a layout of 19 sites/57 sectors (3 sectors/site) and the HAPS network consists of 7 sectors in two layers as described in [7]. At 2 GHz frequency, HAPS has a coverage radius of 100 Km. Uniformly distributed UEs are dropped in the coverage area of each network with the density of 10 UEs per sector. Details of the simulation assumption are given in [7].

Adjacent channel interference (ACI) is modelled, consistent with [8], by attenuating the aggressor network signals by ACIR (Adjacent Channel Interference Ratio). Since NR UE is assumed for the HAPS network, the maximum ACIR is limited by the NR UE  requirement of ACLR or ACS according to

| ACIR=11ACLR+1ACS . | (1) |
| --- | --- |

Throughput calculation is based on the instantaneous SINR following the methodology in [8]. The average throughput and cell-edge (5%-tile) throughput degradation of the victim network due to ACI is tabulated with various ACIR and inter-system distances (center-to-center distance of two networks). Varying the distance between the two systems allow us to understand the effect of both path loss and HAPS antenna gain.

| (a) | (b) |
| --- | --- |

Figure 1. Coexistence scenarios of (a) HAPS and TN, (b) HAPS and HAPS.

## FDD DL HAPS coexistence simulation results

For downlink simulations, we consider one HAPS network as the aggressor in order to study HAPS transmitter’s ACLR requirement. The victim network can be a terrestrial NR network in TN+HAPS coexistence or can be another HAPS network in HAPS+HAPS coexistence. The simulations evaluate the impact of HAPS ACI on the victim network UEs. Maximum ACIR in this case is limited by the UE’s ACS of 33 dB.

#### HAPS DL interfering with Urban Macro NR DL

Table 1. Degradation of DL average throughput in UMa NR system

| ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| 0km ISDCC | 6.7% | 3.2% | 1.4% | 0.5% | 0.0% | 0.0% |
| 10km ISDCC | 2.9% | 1.2% | 0.4% | 0.0% | 0.1% | 0.0% |
| 20km ISDCC | 0.9% | 0.4% | 0.0% | 0.0% | 0.0% | 0.0% |
| 30km ISDCC | 2.0% | 1.0% | 0.4% | 0.0% | 0.0% | 0.0% |
| 40km ISDCC | 2.1% | 0.8% | 0.4% | 0.1% | 0.0% | 0.2% |
| 50km ISDCC | 1.3% | 0.7% | 0.3% | 0.2% | 0.1% | 0.0% |

Table 2. Degradation of DL cell-edge throughput in UMa NR system

| ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| 0km ISDCC | 29.4% | 14.0% | 3.8% | 0.0% | 0.0% | 0.0% |
| 10km ISDCC | 15.4% | 6.5% | 2.0% | 0.0% | 0.0% | 0.0% |
| 20km ISDCC | 3.6% | 2.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| 30km ISDCC | 9.9% | 3.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| 40km ISDCC | 8.8% | 1.6% | 0.0% | 0.0% | 0.0% | 0.0% |
| 50km ISDCC | 4.2% | 2.2% | 0.0% | 0.0% | 0.0% | 0.0% |

#### HAPS DL interfering with Rural NR DL

Table 3. Degradation of DL average throughput in RMa NR system

| ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| 0km ISDCC | 1.0% | 0.5% | 0.1% | 0.0% | 0.0% | 0.0% |
| 10km ISDCC | 0.6% | 0.3% | 0.0% | 0.1% | 0.0% | 0.0% |
| 20km ISDCC | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| 30km ISDCC | 0.8% | 0.4% | 0.1% | 0.1% | 0.1% | 0.1% |
| 40km ISDCC | 0.9% | 0.3% | 0.1% | 0.0% | 0.0% | 0.0% |
| 50km ISDCC | 0.8% | 0.4% | 0.1% | 0.1% | 0.0% | 0.0% |

Table 4. Degradation of DL cell-edge throughput in RMa NR system

| ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| 0km ISDCC | 1.9% | 2.5% | 0.0% | 0.0% | 0.0% | 0.0% |
| 10km ISDCC | 1.7% | 1.2% | 0.0% | 1.2% | 0.8% | 0.5% |
| 20km ISDCC | 1.8% | 0.8% | 1.3% | 0.0% | 1.3% | 0.0% |
| 30km ISDCC | 2.3% | 2.0% | 0.0% | 0.0% | 1.3% | 3.0% |
| 40km ISDCC | 2.0% | 0.9% | 0.0% | 0.0% | 0.0% | 0.0% |
| 50km ISDCC | 1.6% | 2.3% | 1.6% | 1.1% | 0.0% | 1.5% |

#### HAPS DL interfering with HAPS DL

Table 5. Degradation of DL average throughput in HAPS system

| ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| 0km ISDCC | 39.2% | 22.1% | 10.4% | 4.2% | 1.6% | 0.7% |
| 10km ISDCC | 38.8% | 21.6% | 10.1% | 4.0% | 1.5% | 0.5% |
| 20km ISDCC | 38.4% | 21.4% | 9.9% | 4.0% | 1.6% | 0.7% |
| 30km ISDCC | 34.3% | 18.8% | 8.5% | 3.3% | 1.1% | 0.3% |
| 40km ISDCC | 34.4% | 19.0% | 8.7% | 3.4% | 1.1% | 0.3% |
| 50km ISDCC | 31.6% | 17.5% | 8.2% | 3.3% | 1.2% | 0.5% |

Table 6. Degradation of DL cell-edge throughput in HAPS system

| ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| 0km ISDCC | N/A | N/A | 43.5% | 20.1% | 7.6% | 1.8% |
| 10km ISDCC | N/A | N/A | 44.0% | 26.2% | 13.7% | 3.8% |
| 20km ISDCC | N/A | N/A | 46.8% | 27.5% | 11.4% | 4.9% |
| 30km ISDCC | N/A | N/A | 45.8% | 25.5% | 9.4% | 3.4% |
| 40km ISDCC | N/A | N/A | 48.7% | 19.9% | 6.3% | 0.0% |
| 50km ISDCC | N/A | N/A | 47.8% | 16.8% | 2.3% | 1.3% |
| Note: “N/A” indicates that the victim network’s cell-edge throughput is 0. |  |  |  |  |  |  |

## Simulation results summary

The agreed protection criteria for adjacent channel interference is 5% degradation in victim network throughput. Table 7 shows the worst case DL ACIR for 5% loss in average throughput and cell edge throughput of the victim network for each simulation scenario. For FDD DL, it appears that TN+HAPS coexistence has much lower ACI than HAPS+HAPS coexistence. In HAPS+HAPS coexistence, the cell edge throughput suffers a higher degradation than the average throughput. Considering the existing 33 dB ACS for the UE, the required ACLR for HAPS should not be more than 33 dB (using 30 dB ACIR based on Table 7 and the ACIR model (1)).

Table 7. Worst case downlink ACIR for 5% throughput loss in simulated scenarios

| Simulation scenario | ACIR for 5% average throughput loss (dB) | ACIR for 5% cell-edge throughput loss (dB) |
| --- | --- | --- |
| Urban macro NR + HAPS | 7.4 | 14.4 |
| Rural macro NR + HAPS | < 5 | < 5 |
| HAPS + HAPS | 19.4 | 29.9 |

Observation 1: For FDD DL, terrestrial NR + HAPS coexistence scenarios have lower adjacent channel interference than HAPS + HAPS scenario.

Observation 2: For FDD DL, the victim network suffers a higher degradation in cell-edge throughput than in average throughput.

Observation 3: Based on simulation data, HAPS ACLR requirement should not be more than 33 dB.

# Conclusion

We presented HAPS coexistence simulation results for FDD DL scenarios based on the assumption detailed in [7]. From the initial results, we have observed that for FDD DL coexistence: (1) the impact of adjacent channel interference (ACI) is mainly in the HAPS+HAPS scenario and no tangible effect in TN+HAPS scenarios; (2) for HAPS+HAPS coexistence, ACI causes a larger degradation in cell edge throughput than in average throughput.

Observation 1: For FDD DL, terrestrial NR + HAPS coexistence scenarios have lower adjacent channel interference than HAPS + HAPS scenario.

Observation 2: For FDD DL, the victim network suffers a higher degradation in cell-edge throughput than in average throughput.

Observation 3: Based on simulation data, HAPS ACLR requirement should not be more than 33 dB.

# References

- R4-2103948, Moderator (Thales), “Email discussion summary for [98e][310] NTN_Solutions_Part1,”

- R4-2103949, Moderator (Samsung), “Email discussion summary for [98e][311] NTN_Solutions_Part2,”

- R4-2103877, Moderator (Thales), “WF for NTN General Part,” 3GPP TSG-RAN WG4 Meeting #98-e, 21st Jan – 5th Feb, 2021.

- R4-2103878, Moderator (Samsung), “Way Forward for NTN Coexistence Study,” 3GPP TSG-RAN WG4 Meeting #98-e, 21st Jan – 5th Feb, 2021.

- DRAFT Meeting Report for TSG RAN WG4 meeting: 98-e, 3GPP TSG-RAN WG4 Meeting #98-e, 21st Jan – 5th Feb, 2021.

- R4-2103998, CATT, “Simulation assumption for NTN co-existence study,” 3GPP TSG-RAN WG4 Meeting #98-e, 21st Jan – 5th Feb, 2021.

- R4-2107194, Nokia, Nokia Shanghai Bell, “HAPS simulation assumptions for coexistence study,” 3GPP TSG-RAN WG4 Meeting #98bis-e, 12-20 Apr, 2021.

- 3GPP TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.”