3GPP TSG-RAN4 Meeting #99-e	R4-2108601

# Electronic meeting, 19th – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to 38.176-1: IAB-MT performance tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2011-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Performance tests for the IAB-MT are needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performance tests and performance requirements are introduced. The performance requirements are simplified according to the selected requirements and simplifications of previous agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete IAB-MT specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.2	IAB-MT Performance requirements

### 8.2.1	General

#### 8.2.1.1	Scope and definitions

Conducted performance requirements specify the ability of the IAB-MT to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the TAB connector(s).

Conducted performance requirements for the IAB-MT are specified for the fixed reference channels and the propagation conditions defined in TS 38.174 [2] annex A and annex J, respectively. The requirements only apply to those FRCs that are supported by the IAB-MT.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for an IAB-MT supporting CA are defined in terms of single carrier requirements.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in a slot on a single TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

### 8.2.2	Demodulation performance requirements

#### 8.2.2.1	General

{Editors note: Applicability of requirements to be added }

#### 8.2.2.2	Performance requirements for PDSCH

##### 8.2.2.2.1	Definition and applicability

The performance requirement of PDSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

##### 8.2.2.2.2	Minimum requirement

The minimum requirement is in TS 38.174 [2] clause 8.2.2.1.2.

##### 8.2.2.2.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

##### 8.2.2.2.4	Method of test

##### 8.2.2.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

##### 8.2.2.2.4.2	Procedure

1)	Connect the IAB tester generating the wanted signal, multipath fading simulators and AWGN generators to all IAB-MT antenna connectors for diversity reception via a combining network as shown in annex D.x.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 8.2.2.2.4.2-1.

Table 8.2.2.2.4.2-1: Test parameters for testing PDSCH

| Parameter |  |  |  | Unit |  |  | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  |  |  |  | TDD |  |
| Active BWP index |  |  |  |  |  |  | 1 |  |
| Default TDD UL-DL pattern (Note 1) |  |  |  |  |  |  |  | 7D1S2U, S=6D:4G:4U |
| PDSCH transmission scheme |  |  |  |  |  |  | Transmission scheme 1 |  |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | RBs |  |  | 0 |  |
|  | Subcarrier spacing |  |  | kHz |  |  | 30 |  |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  |  | Normal |  |
|  | RB offset |  |  | RBs |  |  | 0 |  |
|  | Number of contiguous PRB |  |  | PRBs |  |  | 106 |  |
| PDSCH DMRS configuration | Antenna ports indexes |  |  |  |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |  |
|  | Position of the first DMRS for PDSCH mapping type A |  |  |  |  |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  |  |  |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DMRS Type |  |  |  |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  |  |  |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  |  |  | 1 |
| PDSCH configuration |  | Mapping type |  |  |  | Type A |  |  |
|  |  | k0 |  |  |  | 0 |  |  |
|  |  | Starting symbol (S) |  |  |  | 2 |  |  |
|  |  | Length (L) |  |  |  | Specific to each Reference channel |  |  |
|  |  | PDSCH aggregation factor |  |  |  | 1 |  |  |
|  |  | PRB bundling type |  |  |  | Static |  |  |
|  |  | PRB bundling size |  |  |  | 2 |  |  |
|  |  | Resource allocation type |  |  |  | Type 0 |  |  |
|  |  | RBG size |  |  |  | Config2 |  |  |
|  |  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |  |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |  |
| PT-RS configuration |  |  |  |  |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  |  |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  |  |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: Point A coincides with minimum guard band as specified in TS 38.174 [2] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in tables 8.2.2.2.5.1-1, 8.2.2.2.5.1-2 , 8.2.2.2.5.1-3 or 8.2.2.2.5.2-1-4 (as applicable) is achieved at the IAB-MT input.

6)	For each of the reference channels in tables 8.2.2.2.5.1-1, 8.2.2.2.5.1-2 , 8.2.2.2.5.1-3 or 8.2.2.2.5.2-1-4 applicable for the IAB-MT, measure the throughput.

##### 8.2.2.2.5	Test requirement

The throughput measured according to clause 8.2.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.2.2.2.5-1, 8.2.2.2.5-2, 8.2.2.2.5-3 and 8.2.2.2.5-4.

Table 8.2.2.2.5-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | M-FR1-A.3.3-1 | 40 / 30 | 256QAM, 0.82 | TDLA30-10 | 2x4, ULA Low | 70 | [22.5] |
| 2 | M-FR1-A.3.1-1 | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 30 | [-0.1] |

Table 8.2.2.2.5-2: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | M-FR1-A.3.1-1 | 40 / 30 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | [14.6] |

Table 8.2.2.2.5-3: Minimum performance for Rank 3

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4 | M-FR1-A.3.1-3 | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | [12.4] |

Table 8.2.2.2.5-4: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | M-FR1-A.3.1-4 | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | [16.4] |

#### 8.2.2.3	Performance requirements for PDCCH

##### 8.2.2.3.1	Definition and applicability

The performance requirement of PDCCH is determined by a maximum allowed missed detection rate for a given SNR. The required missed detection rate is expressed for the FRCs listed in annex A.

##### 8.2.2.3.2	Minimum requirement

The minimum requirement is in TS 38.174 [2] clause 8.2.2.2.

##### 8.2.2.3.3	Test purpose

The test shall verify the receiver's ability to achieve missed detection rate under multipath fading propagation conditions for a given SNR.

##### 8.2.2.3.4	Method of test

##### 8.2.2.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

##### 8.2.2.3.4.2	Procedure

1)	Connect the IAB tester generating the wanted signal, multipath fading simulators and AWGN generators to all IAB-MT antenna connectors for diversity reception via a combining network as shown in annex D.x.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 8.2.2.3.4.2-1.

Table 8.2.2.3.4.2-1: Test parameters for testing PDCCH

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 6 |  |  |
| Shift Index |  | 0 |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in tables 8.2.2.3.5.1-1, 8.2.2.3.5.2-2, 8.2.2.3.6.1-3, 8.2.2.3.6.2-4 (as applicable) is achieved at the IAB-MT input.

6)	For each of the reference channels in table 8.2.2.3.5.1-1, 8.2.2.3.5.2-2, 8.2.2.3.6.1-3, 8.2.2.3.6.2-4 applicable for the IAB-MT, measure the missed detection.

##### 8.2.2.3.5	Test requirement

For the parameters specified in Table 8.2.2.3.4.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.3.5-1.

Table 8.2.2.3.5-1: Minimum performance for PDCCH

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | TBC | TDLA30-10 | 1x4 Low | 1 | [2.1] |
| 2 | 40 | 102 | 1 | 4 | TBC | TDLA30-10 | 1x4 Low | 1 | [-0.9] |
| 3 | 40 | 90 | 1 | 8 | TBC | TDLA30-10 | 2x4 Low | 1 | [-4.3] |