3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110657

# Electronic Meeting, May. 19-27, 2021

Source: 	Huawei, HiSilicon

Title: 	On asymmetric Uplink /Downlink scenarios

Agenda Item:	8.28.2

Document for:	Approval

Introduction

On UE REFSENS for 35 MHz and 45 MHz, one remaining issue is to define the asymmetric Uplink /Downlink scenarios. In this contribution we provide some consideration on the open issue.

Discussion

Per way forward on UE REFSENS for 35 MHz and 45 MHz [1], the asymmetric uplink/downlink scenarios is to be further investigated,

- For asymmetric  Uplink /Downlink scenarios

  - Option 1

    - To add a new entry in  "Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations" for bands n8, n25 and n71.

    - The impact of each UL-DL scenario on the default Tx-Rx frequency separation.

  - Option 2

    - To state that the REFSENS requirement is met for a specific duplex spacing in the general REFSENS section

  - Other options are not excluded

In existing 38.101-1, FDD asymmetric UL and DL channel bandwidth combinations are defined for the band which has asymmetric UL and DL spectrum, e.g. n70. And the Tx-Rx separation is defined as a deviation to the default center frequency separation. Hence option 1 may have 2 issues. One is the impact to Tx-Rx separation and the other is the supported channel bandwidths should be explicitly defined. Hence our preference is option2.

Proposal 1: The following note is added for the REFSENS for band n8/n71 35MHz, and n2/n25 45MHz

NOTE: the requirement is met for a default TX–RX frequency separation specified in Table 5.4.4-1.

Proposal 2: The widest channel bandwidth is applicable only to downlink should be clarified in Table Table 5.3.5-1 with NOTE3.

NOTE 3:	This UE channel bandwidth is applicable only to downlink.

Conclusions

In the contribution, we provide discussion on asymmetric Uplink /Downlink scenarios.

Proposal 1: The following note is added for the REFSENS for band n8/n71 35MHz, and n2/n25 45MHz

NOTE: the requirement is met for a default TX–RX frequency separation specified in Table 5.4.4-1.

Proposal 2: The widest channel bandwidth is applicable only to downlink should be clarified in Table Table 5.3.5-1 with NOTE3.

NOTE 3:	This UE channel bandwidth is applicable only to downlink.

References

R4-2105364, WF on UE REFSENS for 35 MHz and 45 MHz,  Huawei, HiSilicon