3GPP TSG-RAN WG4 Meeting #99e	R4-2110998

Electronic Meeting, 19th - 27th May, 2021

Source:	ZTE Corporation

Title:	Evaluation on intra-cell inter-user interference

Agenda Item:	9.11.2.2

Document for:	Discussion

# Introduction

In RAN4#98bis-e, a way forward has been agreed regarding on MMSE-IRC for intra-cell inter-user interference evaluation [1]. In principle, there are three types of parameters to be discussed:

- Scenario for modeling intra-cell inter-user interference - Reference receiver - PDSCH generation

In this contribution, we further discuss possible options for these parameters and share our views and preference.

# Discussion

Scenarios for modelling intra-cell inter-user interference

For paired UE number, we think that during the initial stage for evaluation, we can focus on 2Tx and 4Tx. More than 4Tx or more interference UEs can be studied after the initial stage.

Regarding ranks for both target and interference PDSCH, both rank 1 and rank 2 per UE should be covered, but it is not necessary to exploit all possible combinations, that is Option 2A : [1+1], [2+2] for both UEs.

Proposal 1: Take Option 2A for rank for target and interference PDSCH.

Regarding PMI and precoding matrix generation, the following options were discussed:

Actually BS behaviour could be different in practice: it may choose an optimized precoding matrix based on the feedbacks from both paired UEs with minimized cross-interference among the paired UEs. Here the discussion on PMI selection and precoding matrix generation is more or less for the sake of TE, which is very different from that in real fields. In this case, the option which facilitates most TE would be a good option,  i.e., Option 3.

Proposal 2: Take Option 3 to facilitate TE for PMI and precoding matrix generation.

For DMRS ports selection, the options now are shown below:

As seen in Proposal 1, both rank 1 and rank 2 transmission should be considered and not full combination is needed, e.g., [2,2] is selected, then the target UE can select DMRS port 0/1, and the interference UE port 2/3.

Proposal 3: DMRS port 0/1 for target UE, and port 2/3 for the interference UE.

For DMRS pattern and sequence, three options were discussed:

It is convenient for BS to treat all co-scheduled UEs as one “virtual super UE” with multiple layers, so the same DMRS type, additional position, scrambling ID and cell ID would be preferred.

Proposal 4: Take Option 1 for DMRS pattern and sequence.

Reference receiver

For all open issues on reference receiver, such as Interference estimation for cases with 2 DMRS CDM group, Interference estimation granularity, UE vendors should have some flexibility, and the difference among vendors can be covered by different implementation margin, so we prefer to leave to UE implementation.

Proposal 5: Take Option 3 (up to UE implementation) for interference estimation.

And for network assistance for the receiver, it can be regarded as a kind of “indirect coordination” among co-scheduled UEs, and besides the required UE-wise signaling overhead, we could focus on no network assistance.

Proposal 6: No network assistance is considered at this stage.

PDSCH generation

The main open issue on PDSCH generation is the selection of channel bandwidth:

| Channel bandwidth | Option 1 - For FDD 15kHz SCS: Cover 10MHz and 50MHz CBW - For TDD 30kHz SCS: Cover 40MHz and 100MHz CBW Option 2: - For FDD 15kHz SCS: Cover 10MHz - For TDD 30kHz SCS: Cover 40MHz Option 3: - For FDD 15kHz SCS: Cover 10MHz and 40MHz CBW - For TDD 30kHz SCS: Cover 40MHz and 100MHz CBW |
| --- | --- |

A reasonable assumption is that a UE with advanced receiver may have higher cost, i.e., a high-end UE is more likely to have advanced receiver support, thus a better user experience, e.g., higher data rate is expected, so among all three optons, Option 1 may better fit for this purpose.

Proposal 7: Take Option 1 for the PDSCH channel bandwidth.

# Conclusion

In this paper, we have the following proposals for intra-cell inter-UE interference evaluation:

Proposal 1: Take Option 2A for rank for target and interference PDSCH.

Proposal 2: Take Option 3 to facilitate TE for PMI and precoding matrix generation.

Proposal 3: DMRS port 0/1 for target UE, and port 2/3 for the interference UE.

Proposal 4: Take Option 1 for DMRS pattern and sequence.

Proposal 5: Take Option 3 (up to UE implementation) for interference estimation.

Proposal 6: No network assistance is considered at this stage.

Proposal 7: Take Option 1 for the PDSCH channel bandwidth.

# Reference

R4-2106118, Way Forward on MMSE-IRC for intra-cell inter-user interference, Huawei