3GPP TSG-RAN WG4 Meeting #99-e                                          R4-2111446

Electronic Meeting, May 19-27, 2021

Source: 	Huawei, HiSilicon

Title: 	Further system level simulation for FDD HPUE

Agenda Item:	10.5.4

Document for:	Approval

# Introduction

WF on simulation for system performance evaluation was agreed in last RAN4 meeting [1]. For system level simulation, there are some issues to be further evaluated.

This contribution provides additional results based on initial simulation results provided in [2].

# Discussion

## Simulation assumptions

Table 2.1-1 provides the simulation assumptions based on previous WF.

Table 2.1-1: Simulation assumptions

| Configuration parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| ISD | 500 m |
| Duplexing | FDD |
| Carrier frequency | 1.8 GHz, 2.1GHz |
| Modulation | Up to 64QAM |
| Numerology | 15 kHz |
| Simulation bandwidth | 40 MHz |
| Transmission scheme | SU-MIMO |
| Codebook | For 2Tx, codebook [1 1]T is used for transmit diversity |
| SU dimension | 1 layer |
| Antenna configuration at TRxP | 4Rx, (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) |
| Antenna configuration at UE | 1Tx, (M,N,P,Mg, Ng) = (1,1,1,1,1; 1,1), |
| UE maximal transmit power | For 1Tx, 23 dBm for each TXRU For 1Tx, 26 dBm for each TXRU (High power UE) For 2Tx, 23 dBm for each TXRU (High power UE) |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Power control parameter | P0 = -76, alpha = 0.6 P0=-76, alpha = 0.8 |
| TRxP number per site | 3 |
| TRxP number | 21 |
| Channel model | UMa following TR 38.901 |
| Electronic tilt | 102° |
| Traffic model | FTP3, packet size: 100k/10k Byte, arrival rate: 1 packet / 200ms. |
| Uplink duty cycle | 50%（PC 2UE）, 100%（PC 3 UE） |

## Simulation results

According to the WF in [1], in this section the simulation for P0 = -76, alpha = 0.8 under BS 4R UE 1T and P0 = -76, alpha = 0.6 is further performed.

Table 2.2-1 and 2.2-2 provide the simulation results.

Table 2.2-1: Simulation results for P0 = -76, alpha = 0.8

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 167.50% | 109.1% |

Table 2.2-2: Simulation results for P0 = -76, alpha = 0.6

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 153.33% | 112.50 % |

Figure 1 shows the CDF curve of the UE. From Figure 1, although the interference increases with the increase of transmit power of PC2 UE, there is still 1dB gain compared with the PC3 UE. And Figure 2 shows the actual number of scheduled RB for the PC2 UE and PC3 UE sorted by UE throughput. For the cell edge UE (5% UPT UE), the number of the scheduled RB for PC2 UE is more than twice of the PC3 UE. Therefore, even though the PC2 UE has 50% restriction, due to the gain of the SINR and the more scheduled RB for PC2 UE, the average and cell edge cases could have an obvious performance gain.

Figure 1 SINR CDF of PC2 UE (50% duty cycle) and PC3 UE (100% duty cycle)

Figure 2 Number of scheduled RB per UE sorted by the UE throughput

Observation 1: From the simulation results, performance gain can be observed for both cell average and cell edge cases. And extremely obvious performance gain on cell average throughput is observed.

# Conclusion

This contribution provides additional simulation results on system performance evaluation for FDD HPUE according to the WF in last RAN4 meeting. Based on the results, we still see obvious performance gain.

Observation 1: From the simulation results, performance gain can be observed for both cell average and cell edge cases. And extremely obvious performance gain on cell average throughput is observed.

Proposal 1: It is proposed to make a conclusion on system performance gain for FDD HUPE and focus on other remaining issues for the SI.

# References

[1] R4-2105425, WF on initial agreements on simulation result, China Unicom

[2] R4-2107300, System performance evaluation for FDD HPUE, Huawei, HiSilicon