3GPP TSG-RAN WG4 (Radio) Meeting #73-AH-UE-RF	R4-73AH-0017

Oulu, Finland, 13 – 16 January 2015

Agenda Item:	4.5.2

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for TR 36.851: LTE Advanced Carrier Aggregation of Band Combination (25 + 41)

Document for:	Approval

# 1.	Introduction

The WI to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 was completed in Rel-12 [1-3]. But the related text proposals [4-5] were approved into the Rel-13 TR [6]. In this contribution, we provide a text proposal to put the corresponding materials into the Rel-12 TR [7] to avoid unnecessary confusion.

# 2.	Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation under Rel-12 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation requirements for the Rel-12 band combinations in table 1-1.

Table 1-1: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B1_B7 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 7 | A3 |
| LTE_CA_B2_B4 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 4 | A4 |
| LTE_CA_B23_B29 | LTE Advanced inter-band Carrier Aggregation of Band 23 and Band 29 | A1 |
| LTE_CA_B3_B26 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 26 | A1 |
| LTE_CA_B3_B19 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 19 | A1 |
| LTE_CA_B1_B8 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 8 | A1 |
| LTE_CA_B3_B28 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 28 | A1 |
| LTE_CA_B1_B26 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 26 | A1 |
| LTE_CA_B39_B41 | LTE Advanced inter-band Carrier Aggregation of Band 39 and Band 41 | A3 |
| LTE_CA_B2_B12 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 12 | A1 |
| LTE_CA_B2_B13 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 13 | A1 |
| LTE_CA_B19_B21 | LTE Advanced inter-band Carrier Aggregation of Band 19 and Band 21 | A5 |
| LTE_CA_B12_B25 | LTE Advanced inter-band Carrier Aggregation of Band 12 and Band 25 | A1 |
| LTE_CA_B2_B5 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 5 | A1 |
| LTE_CA_B1_B18 | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 18 | A1 |
| LTE_CA_B7_B28 | LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 28 | A1 |
| LTE_CA_B5_B25 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 25 | A1 |
| LTE_CA_B1_B11 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 11 | A5 |
| LTE_CA_B8_B11 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 11 | A5 |
| LTE_CA_B5_B7 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 7 | A1 |
| LTE_CA_B1_B3 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 3 | A3 |
| LTE_CA_B1_B28 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 28 | A2 |
| LTE_CA_B4_B27 | LTE Advanced inter-band Carrier Aggregation of Band 4 and Band 27 | A1 |
| LTE_CA_B3_B27 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 27 | A1 |
| LTE_CA_B8_B27 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 27 | A3 |
| LTE_CA_B3_B20_BWset | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 20 | A1 |
| LTE_CA_B7_B20_BWset | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 20 | A1 |
| LTE_CA_B41_B42 | LTE Advanced inter-band Carrier Aggregation of Band 41 and Band 42 | A5 |
| LTE_CA_B5_B13 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 13 | A3 |
| LTE_CA_B7_B22 | LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 22 | A5 |
| LTE_CA_B2_B5_BWset | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 5 | A1 |

The scope of the report has been further extended [6] to cover the 2 DL fallback modes for the 3 DL Carrier Aggregation combinations with single UL configuration. These 3 DL/1UL combinations are listed in table 1-2. In addition, the report also covers the LTE TDD-FDD joint operation including Carrier Aggregation combinations [7], captured in Table 1-3.

Table 1-2: Release 12 inter-band carrier aggregation combinations (3 DL/1UL)

| WI code | WI title |
| --- | --- |
| LTE_CA_B2_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 5 and Band 30 |
| LTE_CA_B2_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 29 and Band 30 |
| LTE_CA_B1_B5_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 5 and Band 7 |
| LTE_CA_B2_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30 |
| LTE_CA_B1_B3_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 20 |
| LTE_CA_B1_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 7 and Band 20 |
| LTE_CA_B7_B8_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 7, Band 8 and Band 20 |
| LTE_CA_B1_B18_B28 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 18 and Band 28 |
| LTE_CA_3DL_FB_BW | Additional bandwidth combinations for LTE Advanced inter-band Carrier Aggregation to support 3DL fallback |
| LTE_CA_B4_B7_B12 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 4, Band 7 and Band 12 |

Table 1-3: Release 12 inter-band carrier aggregation combinations (LTE TDD-FDD joint operation)

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_TDD_FDD | LTE TDD-FDD joint operation including Carrier Aggregation (Band 1 and Band 41) | A3 |
| LTE_CA_TDD_FDD | LTE TDD-FDD joint operation including Carrier Aggregation (Band 1 and Band 42) | A3 |
| LTE_CA_TDD_FDD | LTE TDD-FDD joint operation including Carrier Aggregation (Band 3 and Band 40) | A3 |
| LTE_CA_TDD_FDD | LTE TDD-FDD joint operation including Carrier Aggregation (Band 8 and Band 40) | A1 |
| LTE_CA_TDD_FDD | LTE TDD-FDD joint operation including Carrier Aggregation (Band 3 and Band 42) | A2 |
| LTE_CA_B26_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 26, Band 41 and Band 41 | - |
| LTE_CA_B3_B38 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 38 | A3 |
| LTE_CA_B19_B42_B42 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 19, Band 42 and Band 42 | A1 |
| LTE_CA_B25_B41_B41 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 25, Band 41 and Band 41 | - |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

### 7.2.X	LTE Advanced Carrier Aggregation of Band 25 and Band 41

CA_25-41 is designed to operate in the operating bands in table 7.2.X-1.

Table 7.2.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_25-41 | 25 | 1850 MHz | – | 1915 MHz | 5, 10, 15, 20 (note 1) | 1930 MHz | – | 1995 MHz | 5, 10, 15, 20 | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 5, 10, 15, 20 (note 1) | 2496 MHz | – | 2690 MHz | 5, 10, 15, 20 | TDD |
| NOTE 1: The WI considers only one uplink component carrier to be used in Band 25 at any time |  |  |  |  |  |  |  |  |  |  |

#### 7.2.X.1		List of specific combination issues

##### 7.2.X.1.1	Channel bandwidths per operating band for CA

Table 7.2.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_25A-41A | 25 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode.

##### 7.2.X.1.2	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 25 and Band 41 DL carriers can be calculated as shown in Table 7.2.X.1.2-1 below:

Table 7.2.X.1.2-1: Band 25 and Band 41 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1995 | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 3860 | 3990 | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 5790 | 5985 | 7488 | 8070 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 501 | 760 | 4426 | 4685 |
| 3rd order IMD products | (2*f1-low – f2-high) | (2*f1-high – f2-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1170 | 1494 | 2997 | 3450 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6356 | 6680 | 6922 | 7375 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 1736 | 2189 | 2431 | 2755 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1910 | 2015 | 2476 | 2710 |

It can be seen from Table 7.2.X.1.2-1 that the 2nd IMD products caused by BS supporting carrier aggregation of Band 25 and Band 41 may fall into the BS receive band of Bands 12, 17, 28 and 44, and the 3rd IMD products may fall into the BS receive band of Bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 22, 23, 25, 33, 34, 35, 36, 37, 38, 39, 41 and 42. Note that the calculation in Table 7.2.X.1.2-1 (except the last row) assumes the BS is transmitting with the whole 65 MHz frequency of Band 25 and the whole 194 MHz frequency of Band 41. If the BS is only transmitting an up to 20 MHz DL in Band 25 and Band 41 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 2, 3, 4, 9, 10 or 35 as shown in the last row in Table 7.2.X.1.2-1. Moreover, the 3rd IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 25, as the transmit-receiver frequency separation is 80 MHz.

Note that Bands 1, 7, 11, 21, 28, 33, 34, 38, 39 and 44 are not intended for use in the same geographical area as Bands 25 and 41. Moreover, co-location of Band 25 transmitter and Band 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Furthermore, the Band 41 BS deployed in the same geographical area are assumed to be synchronized so that the Band 41 BS receivers will not be interfered by the Band 41 BS transmission. Therefore, the focus here is on the IMD falling into Bands 12, 17, 22, 23, 25 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12, 17, 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 25 and 41 BS transmitters do not share the same antenna with Band 12, 17, 22 or 42 BS receiver. But there is not a large frequency gap between Band 25 DL and Bands 23 and 25 UL, and hence Bands 23 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Bands 25 and 41 BS transmitters should not share the same antenna with Band 23 or 25 BS receiver to prevent BS receiver desensitization, or Band 12, 17, 22 or 42 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 17, 22 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### 7.2.X.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_25A-41A the ΔTIB,c is defined for applicable bands in table 7.2.X.1.3-1.

Table 7.2.X.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-41A | 25 | 0.5 |
|  | 41 | 0.5 |

For the UE which supports CA_25A-41A the ΔRIB is defined for applicable bands in table 7.2.X.1.3-2.

Table 7.2.X.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_25A-41A | 25 | 0 |
|  | 41 | 0 |

<End of change>

# References

[1]	R4-148124, “Introduction of additional band combinations for 3DL inter-band CA”, Ericsson, CATT, CMCC.

[2]	R4-148121, “Introduction of 3DL CA combinations”, Nokia Networks.

[3]	R4-148122, “Introduction of 3DL CA combinations”, Nokia Networks.

[4]	R4-146170, “TP for Rel-13 2DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 41)”, Alcatel-Lucent, Sprint.

[5]	R4-146840, “TP for Rel-13 2DL TR 36.8xx: TIB and RIB values of LTE-A Inter-band Carrier Aggregation of Band 25A and Band 41A (1UL)”, Alcatel-Lucent, Sprint.

[6]	R4-73AH-00xx, “Rel-13 2DL TR 36.8xx: LTE-A Inter-band Carrier Aggregation v0.3.0”, Ericsson.

[7]	R4-73AH-00xx, “TR 36.851 V0.15.0: Rel-12 Inter-band Carrier Aggregation (2DL/1UL)”, Ericsson.