3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800024

San Diego, California, USA, 22 – 26 January 2018

Agenda Item:	4.4.3.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation Results for FRC for BS receiver reference sensitivity requirements

Document for:	Discussion

# 1.	Introduction

Fixed Reference Channels (FRC) for BS receiver requirements and simulation assumptions were discussed in RAN4#85, and the way forward was agreed in [1]. Further discussions have been held via RAN4 reflector, and some simulation parameters had been revised accordingly [2].

This contribution provides the simulation results for FRC for BS receiver reference sensitivity requirements using QPSK, according to the agreed simulation parameters in [1] as well as those revised by email discussions [2].

# 2.	Discussion

The simulation results for FRC without FDM (i.e. the 6REs not used for DMRS in each DMRS symbol are not used for PUSCH transmission) for BS receiver reference sensitivity requirements using QPSK are summarized in Table 1 below. Note that practical channel estimation is used but no other imperfection is applied in the simulation.

Table 1: Simulation results for reference sensitivity requirements without FDM using QPSK

|  | Channel Bandwidth (MHz) | SCS (kHz) | TBS (bits) | LDPC Code Rate | Number of PRBs | 95% Throughput SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR1-A1-1 | 5 | 15 | 2408 | 1/3 | 25 | 0 |
| G-FR1-A1-2 | 5 | 30 | 1064 | 1/3 | 11 | 0.1 |
| G-FR1-A1-3 | 10 | 60 | 1064 | 1/3 | 11 | 0.1 |
| G-FR1-A1-4 | 20 | 15 | 10248 | 1/3 | 106 | -0.1 |
| G-FR1-A1-5 | 20 | 30 | 4864 | 1/3 | 51 | -0.2 |
| G-FR1-A1-6 | 20 | 60 | 2408 | 1/3 | 24 | 0.2 |
| G-FR2-A1-1 | 50 | 60 | 6272 | 1/3 | 66 | -0.1 |
| G-FR2-A1-2 | 50 | 120 | 3104 | 1/3 | 32 | 0.2 |
| G-FR2-A1-3 | 100 | 120 | 6272 | 1/3 | 66 | -0.1 |

# 3.	Conclusion

This contribution has provided the simulation results for FRC for BS receiver reference sensitivity requirements using QPSK.

# References

[1]	R4-1714137, “WF on NR BS RF FRC”, Ericsson, Huawei, ZTE, CATT, Nokia.

[2]	R4-1800334, “WF on NR BS RF RFCs – update”, Ericsson, Nokia, Nokia Shanghai Bell.