3GPP TSG-RAN WG5 Meeting #44 	 R5-095179

- Shenzhen, China, 24 – 28 August 2009

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | CRNum |  rev | - |  Current version: | 8.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | GCF Priority 3 –Addition of new test case 6.2.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | R5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | LTE-UEConTest_SIG |  |  |  |  |  |  | Date:  |  |  |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Inter-RAT Cell Selection / from E-UTRAN RRC_IDLE to -> 1xRTT idle, cell re-selection when the serving cell becomes non-suitable (SServingCell<0) |  |  |  |  |  |  |  |  |
| Summary of change:  |  | New text |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | 36.523-1 will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: 36.523-2 |  | x |  | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

#### 6.2.2.4	Inter-RAT Cell Selection / from E-UTRAN RRC_IDLE to -> 1xRTT idle, cell re-selection when the serving cell becomes non-suitable (SServingCell<0)

        - Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (SServingCell<0)and

there is a suitable neighbour 1xRTT cell (S>0) }

then { UE selects the suitable neighbour 1xRTT cell }

}

6.2.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.2.1, and 5.2.3.2.

[TS 36.300, clause 10.1.1.1]

...

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on  that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

...

[TS 36.304, clause 4.1]

...

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN.

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

...

[TS 36.304, clause 5.2.1]

...

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | [FFS] |

6.2.2.4.3	Test description

6.2.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is high quality cell on E-UTRA

-	Cell 19 is high quality cell on 1xRTT

UE:

None

Preamble:

-	UE is in state Registered, Idle Mode (state 2) according to [18].

6.2.2.4.3.2	Test procedure sequence

Table 6.2.2.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.2.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 19 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15 KHz | -75 | - | The power levels are such that camping on Cell 1 is guaranteed. |
| T1 | Îor/Ioc | dB | - | -15 | Cell 19 is on |
|  | Pilot Ec/ Ior |  |  | -7 |  |
|  | Ioc | dBm | - | -75 |  |
|  | CPICH_Ec/Io (Note 1) | dB | - | -22 |  |
| T2 | Îor/Ioc | dB | - | 0 | Increase pilot power of 1xRTT cell such that SnonServingCell, of Cell 19 > Thresh1xRTT, high |
|  | Pilot Ec/ Ior | dB | - | -7 |  |
|  | Ioc | dBm/1.23 MHz | - | -75 |  |
|  | CPICH_Ec/Io (Note 1) | dB | - | -10 |  |

Note 1:	This parameter is not directly settable, but is derived by calculation from the

other parameters set by the SS

Table 6.2.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 according to row "T2" in Table 6.2.2.3.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE transmit an Access Probe on Cell 19? | --> | Access Probe | 1 | P |

6.2.2.4.3.3	Specific message contents

Table 6.2.2.4.3.3-1 SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.2.4.3.2-1)

| Derivation path: 36.508 table clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| schedulingInformation ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | Combination 6 in TS 36.508 section 4.4.3.1.1 |  |  |
| } |  |  |  |

Table 6.2.2.4.3.3-2: SystemInformationBlockType8 to Cell 1 (Preamble and all steps, table 6.2.2.3.3.2-1)

| Derivation path: 36.508 table 4.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| cdma2000-SystemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| cdma-SynchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| oneXRTT-Parameters SEQUENCE {} | Not present |  | HRPD |
| oneXRTT-Parameters SEQUENCE { |  |  | 1XRTT |
| oneXRTT-CSFB-RegistrationInfo SEQUENCE { |  |  |  |
| oneXRTT-CSFB-RegistrationAllowed | FFS | BOOLEAN |  |
| oneXRTT-RegistrationParameters SEQUENCE | FFS |  |  |
| {} |  |  |  |
| } |  |  |  |
| oneXRTT-LongCodeState | FFS | BIT STRING (SIZE (42)) OPTIONAL |  |
| oneXRTT-CellReselectionParameters |  |  |  |
| SEQUENCE { |  |  |  |
| oneXRTT-BandClassList SEQUENCE (SIZE |  |  |  |
| (1..maxCDMA -BandClass)) OF SEQUENCE { | The same number of entries as the configured 1xRTT carriers |  |  |
| oneXRTT | BandClass FFS | ENUMERATED { bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| oneXRTT-CellReselectionPriority [Set according to specific | test case] | [3 is applicable |  |
| threshX-High | FFS | INTEGER (0..63) |  |
| threshX-Low | FFS | INTEGER (0..63) |  |
| oneXRTT-NeighborCellList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | The same number of entries as the configured 1xRTT neighbor cells |  |  |
| bandClass | FFS | ENUMERATED { bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| frequencyList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | FFS | INTEGER (0..2047) |  |
| frequency | FFS |  |  |
| cellIdList SEQUENCE (SIZE (1..16)) OF {INTEGER (0..maxPNOffset) } | FFS | INTEGER (0..maxPNOffset) |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA-OneXRTT | FFS | INTEGER (0..7), |  |
| } |  |  |  |