3GPP TSG RAN WG5 Meeting #72	 R5-166096

# Göteborg, Sweden, 22-26 August 2016

| Title: | Discussion for test configuration table format of RX for 4DL with UL CA |
| --- | --- |
| Source: | SGS Wireless |
| Agenda Item: | 5.3.11.2 |
| Document for: | Discussion |

1	Introduction

At RAN#70, Rel-13 CA WI was approved by [1] and test configuration table format for 4DL with UL CA needs to be discussed.

2	Discussion

According to the discussion paper by [2] in RAN5#70 meeting, the following 8 types of 4DL/1UL CA shown in Table 2-1 can be put in together.

Table 2-1: CA types list for 4DL/1UL CA in TS 36.101 v13.2.0

| CA types | Classes |
| --- | --- |
| All intra C | CA_XE |
| inter + intra C(3CCs) | CA_XA-YD/CA_XD-YA |
| inter + intra C(2CCs) | CA_XA-YA-ZC/CA_XA-YC-ZA/CA_XC-YA-ZA CA_XA-YA-ZB |
| All inter | CA_WA-XA-YA-ZA |
| intra C(2CCs) + intra C(2CCs) | CA_XC-YC |
| intra NC + intra NC | CA_XA-XA-YA-YA |
| intra NC + Intra C(2CCs) | CA_XA-XA-YB |
| intra NC+inter | CA_XA-XA-YA-ZA/CA_XA-YA-YA-ZA |

From discussion paper [2], a type for all intra-band contiguous (highlighted by orange in Table 2-1) needs to be defined only with this case like Table 2-2, but the types for some inter-band CAs (highlighted by green in Table 2-1) can be merged like Table 2-3.

For the types for intra-band non-contiguous CAs, as defined in Table 7.3A.5.4.1-4 and Table 7.3A.5.4.1-5 in TS 36.521-1, there is one different point for UL PCC allocation and this point might make merging tables difficult.

So, in this meeting, we are going to create a new configuration table format to consider the types for intra-band non-contiguous, intra-band contiguous or inter-band CAs with UL PCC allocation.

In here we have introduced two formats as below, Table 2-3 is an older format with suggestion from discussion paper [2], and we can think about how to add more UL CC in this wider table format; the Table 2-4 is an new format and the table format has changed from vertical to horizontal, and it is ok to add more UL CC, but we need to think about if any other challenge to this new table format.

NOTE: Test points in the tables below are just example and we are not proposing default test points for each CA type.

Table 2-2: Test Configuration Table (Intra-band contiguous)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB | SCCs NRB |  |  | CC MOD | PCC & SCCs RB allocation |  |  |  | CC MOD | PCC LCRB @ RBstart |
|  | Band | Range | PCC | SCC1 | SCC2 | SCC3 |  | SCC1 | SCC2 | SCC3 |  | PCC | SCC1 | SCC2 | SCC3 |  |  |
|  |  |  | CC | CC | CC | CC |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XE Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low | CC1 | CC2 | CC3 | CC4 | Highest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Low | CC1 | CC2 | CC3 | CC4 | Lowest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | X | High | CC1 | CC2 | CC3 | CC4 | Highest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | X | High | CC1 | CC2 | CC3 | CC4 | Lowest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-3: Test Configuration Table (Inter-band) (older format)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  |  |  | PCC NRB | SCCs NRB |  |  | CC MOD | PCC & SCCs RB allocation |  |  |  | CC MOD | PCC LCRB @ RBstart |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  | SCC1 | SCC2 | SCC3 |  | PCC | SCC1 | SCC2 | SCC3 |  |  |
|  | Band | Range | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_WA-XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | W | Mid | X | Mid | Y | Mid | Z | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Mid | W | Mid | Y | Mid | Z | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | Mid | W | Mid | X | Mid | Z | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Z | Mid | W | Mid | X | Mid | Y | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for a CA_XA-YD Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid /CC1 | Y | Mid /CC2 | Y | Mid /CC3 | Highest NRB | Highest NRB_agg |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | Y | Low /CC1 | Y | Low /CC2 | Y | Low /CC3 | X | Mid | Highest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | High /CC1 | Y | High /CC2 | Y | High /CC3 | X | Mid | Lowest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Y | Low /CC1 | Y | Low /CC2 | Y | Low /CC3 | X | Mid | Highest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 5 | Y | High /CC1 | Y | High /CC2 | Y | High /CC3 | X | Mid | Lowest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for a CA_XA-YA-ZC and CA_XA-YA-ZB Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid | Z | Mid /CC1 | Z | Mid /CC2 | Highest NRB | Highest NRB | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | Y | Mid | X | Mid | Z | Mid /CC1 | Z | Mid /CC2 | Highest NRB | Highest NRB | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Z | Low /CC1 | Z | Low /CC2 | X | Mid | Y | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Z | Low /CC1 | Z | Low /CC2 | Y | Mid | X | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 5 | Z | High /CC1 | Z | High /CC2 | X | Mid | Y | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 6 | Z | High /CC1 | Z | High /CC2 | Y | Mid | X | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for a CA_XC-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low /CC1 | X | Low /CC2 | Y | Low /CC1 | Y | Low /CC2 | Highest NRB_agg |  | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Low /CC1 | X | Low /CC2 | Y | Low /CC1 | Y | Low /CC2 | Lowest NRB_agg |  | Lowest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | High /CC1 | Y | High /CC2 | X | High /CC1 | X | High /CC2 | Highest NRB_agg |  | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Y | High /CC1 | Y | High /CC2 | X | High /CC1 | X | High /CC2 | Lowest NRB_agg |  | Lowest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Example:

Table 7.3A.9.4.1-2: Test Configuration Table (Inter-band)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  |  |  | PCC NRB | SCCs NRB |  |  | CC MOD | PCC & SCCs RB allocation |  |  |  | CC MOD | PCC LCRB @ RBstart |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  | SCC1 | SCC2 | SCC3 |  | PCC | SCC1 | SCC2 | SCC3 |  |  |
|  | Band | Range /CC | Band | Range /CC | Band | Range /CC | Band | Range /CC |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA-ZA-RA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid | Z | Mid | R | Mid | Highest NRB | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | Y | Mid | X | Mid | Z | Mid | R | Mid | Highest NRB | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Z | Mid | X | Mid | Y | Mid | R | Mid | Highest NRB | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | R | Mid | X | Mid | Y | Mid | Z | Mid | Highest NRB | Highest NRB | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for CA_XB-YA-ZA and CA_XC-YA-ZA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low /CC1 | X | Low /CC2 | Y | Mid | Z | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Low /CC1 | X | Low /CC2 | Y | Mid | Z | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | X | High /CC1 | X | High /CC2 | Y | Mid | Z | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | X | High /CC1 | X | High /CC2 | Y | Mid | Z | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 5 | Y | Mid | X | Mid /CC1 | X | Mid /CC2 | Z | Mid | Highest NRB | Highest NRB_agg |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 6 | Z | Mid | X | Mid /CC1 | X | Mid /CC2 | Y | Mid | Highest NRB | Highest NRB_agg |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings CA_XA-ZA-YB and for CA_XA-ZA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | Low /CC1 | Y | Low /CC2 | X | Mid | Z | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | Y | Low /CC1 | Y | Low /CC2 | X | Mid | Z | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | High /CC1 | Y | High /CC2 | X | Mid | Z | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Y | High /CC1 | Y | High /CC2 | X | Mid | Z | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 5 | X | Mid | Y | Mid /CC1 | Y | Mid /CC2 | Z | Mid | Highest NRB | Highest NRB_agg |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 6 | Z | Mid | Y | Mid /CC1 | Y | Mid /CC2 | Y | Mid | Highest NRB | Lowest NRB_agg |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: X,Y,Z,R correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-7A-8A,X=1,Y=3,Z=7,R=8 (need to consider X, Y, Z for 3C-7A-28A) Note 5: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 6: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-4: Test Configuration Table (Inter-band) (new format)

In this new format, the test environment, test frequencies, test CC combination and Network signalling value is in the same area in the top of table.

In the 2nd those main items for test parameters for CA configurations is changed from horizontal to vertical, and move to the left side, but PCC, SCC still separate in different block.

In the 3rd the ID is change from vertical to horizontal.

So according to above, we can recongnize that if it is in inter-band, which is meant they are not connected, so the NRB shouldn’t be the same.

Some of redundant information or columns may be can remove, let me know if any suggestion to this?

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |
| TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Default Test Settings for a CA_XA-YA-ZA-RA Configuration (Inter-band) |  |  |  | Default Test Settings for CA_XB-YA-ZA and CA_XC-YA-ZA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  | Default Test Settings CA_XA-ZA-YB and for CA_XA-ZA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |
| DL Allocation |  | ID |  |  |  | ID |  |  |  |  |  | ID |  |  |  |  |  |
|  |  | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 5 | 6 | 1 | 2 | 3 | 4 | 5 | 6 |
| PCC | Band | X | Y | Z | R | X | X | X | X | Y | Z | Y | Y | Y | Y | X | Z |
|  | Range/CC | Mid | Mid | Mid | Mid | Low/CC1 | Low/CC1 | High/CC1 | High/CC1 | Mid | Mid | Low/CC1 | Low/CC1 | High/CC1 | High/CC1 | Mid | Mid |
|  | PCC NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB_agg | Lowest NRB_agg | Highest NRB_agg | Lowest NRB_agg | Highest NRB | Highest NRB | Highest NRB_agg | Lowest NRB_agg | Highest NRB_agg | Lowest NRB_agg | Highest NRB | Highest NRB |
|  | SCCs NRB | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) |
|  | CC Mod | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
|  | RB allocation | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs |
| SCC1 | Band | Y | X | Z | R | X | X | X | X | X | X | Y | Y | Y | Y | Y | Y |
|  | Range/CC | Mid | Mid | Mid | Mid | Low/CC2 | Low/CC2 | High/CC2 | High/CC2 | Mid/CC1 | Mid/CC1 | Low/CC2 | Low/CC2 | High/CC2 | High/CC2 | Mid/CC1 | Mid/CC1 |
|  | PCC NRB | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) |
|  | SCCs NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB_agg | Lowest NRB_agg | Highest NRB_agg | Lowest NRB_agg | Highest NRB_agg | Highest NRB_agg | Highest NRB_agg | Lowest NRB_agg | Highest NRB_agg | Lowest NRB_agg | Highest NRB_agg | Lowest NRB_agg |
|  | CC Mod | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
|  | RB allocation | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs |
| SCC2 | Band | Z | X | Y | R | Y | Y | Y | Y | X | X | X | X | X | X | Y | Y |
|  | Range/CC | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid/CC2 | Mid/CC2 | Mid | Mid | Mid | Mid | Mid/CC2 | Mid/CC2 |
|  | PCC NRB | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) |
|  | SCCs NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB_agg | Highest NRB_agg | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB_agg | Lowest NRB_agg |
|  | CC Mod | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
|  | RB allocation | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs |
| SCC3 | Band | R | X | Y | Z | Z | Z | Z | Z | Z | Y | Z | Z | Z | Z | Z | X |
|  | Range/CC | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid | Mid |
|  | PCC NRB | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) | N/A (-) |
|  | SCCs NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB | Highest NRB |
|  | CC Mod | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
|  | RB allocation | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs | All RBs |
| UL Allocation |  | ID |  |  |  | ID |  |  |  |  |  | ID |  |  |  |  |  |
|  |  | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 5 | 6 | 1 | 2 | 3 | 4 | 5 | 6 |
| PCC | CC Mod | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
|  | LCRB @ Rbstart | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS | REFSENS |
| SCC1 | CC Mod |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LCRB @ Rbstart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SCC2 | CC Mod |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LCRB @ Rbstart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

3	Proposal

In this document it is proposed that for 4DL/1UL CA receiver test cases

Proposal #1: Table 2-2 & 2-3 is older format from [2], and these two doesn’t consider UL CA

Proposal #2: Table 2-4 for inter-band CA (all types for green highlighted CA types in Table 2-1) and it is a new table format to consider the UL CA.

4	References

[1]	RP-151764	New Work Item proposal: UE Conformance Test Aspects - Rel-13 LTE CA configurations

[2] R5-160353	Discussion for test configuration table format of receiver test cases for 4DL/1UL CA