3GPP TSG-RAN5 Meeting #75	R5-172454

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4026 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC eMTC test case 8.2.4.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, MCC TF 160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2017-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - It is unsuitable to do capability checking in handover test case. TP1 should be moved into UE capability test case. - This test case tests the UE supports ce-ModeA-r13, intraFreqA3-CE-ModeA-r13 and intraFreqHO-CE-ModeA-r13 capabilities. This is also fixed in the TCSE. The TC will only be executed if both PICS items are supported. Therefore the IF statements before step 3 and step 7 are redundant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete TP1. The specific message in Table 8.2.4.27.3.3-1(UECapabilityEnquiry) and Table 8.2.4.27.3.3-2 (UECapabilityInformation) and steps 1 and 2 are removed since the UECapabilityEnquiry procedure is not needed in the test procedure. - Delete the IF statements before step 3 and step 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test UE capabilities in handover test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-1 CR 3971 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

8.2.4.27	RRC connection reconfiguration / Handover / Success / Intra-frequency in Enhanced Coverage

8.2.4.27.1	Test Purpose (TP)

(1)

Void

(2)

with { UE supporting CE Mode A in E-UTRA RRC_CONNECTED state supports eventA3 for intra-frequency neighbouring cells and measurement configured for event A3 with event based periodical reporting }

ensure that {

when { Neighbour becomes offset better than serving }

then { UE sends MeasurementReport with correct measId for event A3 }

}

(3)

with { UE supporting CE Mode A in E-UTRA RRC_CONNECTED state supports intra-frequency handover   and UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {

when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

8.2.4.27.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.306, clauses 4.3.29.1, 4.3.29.3, 4.3.29.5.

[TS 36.306, clause 4.3.29.1]

ce-ModeA-r13

This field defines whether the UE supports operation in coverage enhancement mode A, as specified in TS 36.211 [17], TS 36.213 [22] and TS 36.331 [5], and PRACH CE levels 0 and 1 at Random Access, as specified in TS 36.321 [4]. It is mandatory for UEs of DL category M1 and UL category M1.

[TS 36.306, clause 4.3.29.3]

intraFreqA3-CE-ModeA-r13

This field defines whether the UE when operating in CE Mode A supports eventA3 for intra-frequency neighbouring cells in normal coverage and CE Mode A, as specified in TS 36.331 [5] and TS 36.133 [16]. It is mandatory for UEs of this release if ce-ModeA-r13 is supported.

[TS 36.306, clause 4.3.29.5]

intraFreqHO-CE-ModeA-r13

This field defines whether the UE when operating in CE Mode A supports intra-frequency handover to target cell in normal coverage and CE Mode A, as specified in TS 36.331 [5] and TS 36.133 [16]. It is mandatory for UEs of this release if ce-ModeA-r13 is supported.

8.2.4.27.3	Test description

8.2.4.27.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1 and Cell 2.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3-CE) on Cell 1 according to [18].

8.2.4.27.3.2	Test procedure sequence

Table 8.2.4.27.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.27.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.27.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void |  |  | - | - |
| 2 | Void |  |  | - | - |
|  | Void |  |  |  |  |
| 3 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup intra frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.2.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport | 2 | P |
|  | Void |  |  |  |  |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 using common preamble to confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | 3 | P |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | - | - |

8.2.4.27.3.3	Specific message contents

Table 8.2.4.27.3.3-1: Void

Table 8.2.4.27.3.3-2: Void

Table 8.2.4.27.3.3-3: RRCConnectionReconfiguration (step 3, Table 8.2.4.27.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.27.3.3-4: MeasConfig (Table 8.2.4.27.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.27.3.3-5 MeasurementReport (step 6, Table 8.2.4.27.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.27.3.3-6: RRCConnectionReconfiguration (step 7, Table 8.2.4.27.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.27.3.3-7: MobilityControlInfo (Table 8.2.4.27.3.3-6)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

CHANGE END