3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL RB number for NS_27 in A-MPR test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 6.2.4.4.1-24 Configuration ID 20,24 : RB allocation FDD is incorrect. According to 5.3.3 of TS 36.211, UL RB allocation needs to be where is a set of non-negative integers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 6.2.4.4.1-24 Configuration ID 20,24 : RB allocation FDD is changed from 21 to 24 ( min number of allowed RBs with A-MPR = 0 ) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR for NS_27 are not tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of changes]

### 6.2.4	Additional Maximum Power Reduction (A-MPR)

Editor’s note: The following aspects are either missing or not yet determined:

- TT and test points are TBD.

#### 6.2.4.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

#### 6.2.4.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA Power Class 3 UE release 8 and forward.

The requirements of this test apply in test case 6.6.3.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_28, NS_29, NS_30, NS 31, NS_36, NS_38. NS_39 to all types of E-UTRA Power Class 3 UE release 8 and forward.

NOTE:	As a result TC 6.2.4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4.3	Minimum conformance requirements

For UE Power Class 3 the specific requirements and identified sub-clauses are specified in Table 6.2.4.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.3-1 to 6.2.4.3-25 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2.5 apply.

Table 6.2.4.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.2-1 | 1.4,3,5,10,15,20 | Table 5.4.2-1 | N/A |  |
| NS_03 | 6.6.2.2.3.1 | 2,4,10, 23, 25,35,36, 66, 70 | 3 | >5 | ≤ 1 |  |
|  |  |  | 5 | >6 | ≤ 1 |  |
|  |  |  | 10 | >6 | ≤ 1 |  |
|  |  |  | 15 | >8 | ≤ 1 |  |
|  |  |  | 20 | >10 | ≤ 1 |  |
| NS_04 | 6.6.2.2.3.2 | 41 | 5 | >6 | ≤1 |  |
|  |  |  | 10, 15, 20 | Table 6.2.4.3-2 |  |  |
| NS_04 | 6.6.2.2.3.2, 6.6.3.3.3.1 | 41 | 5,10, 15, 20 | Table 6.2.4.3-3, Table 6.2.4.3-4 |  |  |
| NS_05 | 6.6.3.3.3.2 | 1 | 10,15,20 | ≥ 50 1 | ≤ 1 1 |  |
|  |  |  | 15, 20 | Table 6.2.4.3-5 2 |  |  |
|  |  | 65 3 | 10,15,20 | ≥ 50 | ≤ 1 1 |  |
|  |  |  | 15,20 | Table 6.2.4.3-5 2 |  |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.4.2-1 | N/A |  |
| NS_07 | 6.6.2.2.3.3 6.6.3.3.3.3 | 13 | 10 | Table 6.2.4.3-6 | Table 6.2.4.3-6 |  |
| NS_08 | 6.6.3.3.3.4 | 19 | 10, 15 | > 44 | ≤ 3 |  |
| NS_09 | 6.6.3.3.3.5 | 21 | 10, 15 | > 40 | ≤ 1 |  |
|  |  |  |  | > 55 | ≤ 2 |  |
|  |  | 74 6 | 10, 15 | > 40 | ≤ 1 |  |
|  |  |  |  | > 55 | ≤ 2 |  |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4.3-7 | Table 6.2.4.3-7 |  |
| NS_11 | 6.6.2.2.1 6.6.3.3.3.6 | 231 | 1.4, 3, 5, 10,15,20 | Table 6.2.4.3-8 | Table 6.2.4.3-8 |  |
| NS_12 | 6.6.3.3.3.7 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-9, Table 6.2.4.3-10 | Table 6.2.4.3-9, Table 6.2.4.3-10 |  |
| NS_13 | 6.6.3.3.3.8 | 26 | 5 | Table 6.2.4.3-11 | Table 6.2.4.3-11 |  |
| NS_14 | 6.6.3.3.3.9 | 26 | 10, 15 | Table 6.2.4.3-12 | Table 6.2.4.3-12 |  |
| NS_15 | 6.6.3.3.3.10 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-13 Table 6.2.4.3-14 | Table 6.2.4.3-13, Table 6.2.4.3-14 |  |
| NS_16 | 6.6.3.3.3.11 | 27 | 3, 5, 10 | Table 6.2.4.3-15, Table 6.2.4.3-16, Table 6.2.4.3-17 |  |  |
| NS_17 | 6.6.3.3.3.12 | 28 | 5, 10 | Table 5.4.2-1 | N/A |  |
| NS_18 | 6.6.3.3.3.13 | 28 | 5 | ≥ 2 | ≤ 1 |  |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |  |
| NS_19 | 6.6.3.3.3.14 | 44 | 10, 15, 20 | Table 6.2.4.3-18 | Table 6.2.4.3-18 |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.3.15 | 23 | 5, 10, 15, 20 | Table 6.2.4.3-19 | Table 6.2.4.3-19 |  |
| NS_21 | 6.6.2.2.3.1 6.6.3.3.3.16 | 30 | 5, 10 | Table 6.2.4.3-20 | Table 6.2.4.3-20 |  |
| NS_22 | 6.6.3.3.3.17 | 42, 43 | 5, 10, 15, 20 | Table 6.2.4.3-21 |  |  |
| NS_23 | 6.6.3.3.3.18 | 42, 43 | 5, 10, 15, 20 | N/A |  |  |
| NS_24 | 6.6.3.3.3.19 | 65 4 | 5, 10, 15, 20 | Table 6.2.4.3-22 |  |  |
| NS_25 | 6.6.3.3.3.20 | 65 4 | 5, 10, 15, 20 | Table 6.2.4.3-23 |  |  |
| … |  |  |  |  |  |  |
| NS_27 | 6.6.2.2.3.4, 6.6.3.3.3.21 | 48 | 5, 10, 15, 20 | Table 6.2.4.3-24 |  |  |
| NS_28 | 6.2.2A, 6.6.3.3.22 | 46 (NOTE 5) | 20 | Table 6.2.4-25 | NS_28 |  |
| NS_29 | 6.2.2A, 6.6.2.3.1a, 6.6.3.3.23 | 46 (NOTE 5) | 20 | Table 6.2.4-26 | NS_29 |  |
| NS_30 | 6.2.2A, 6.6.3.3.24 | 46 (NOTE 5) | 20 | Table 6.2.4-27 | NS_30 |  |
| NS_31 | 6.2.2A, 6.6.3.3.25 | 46 (NOTE 5) | 20 | Table 6.2.4-28 | NS_31 |  |
| NS_32 | - | - | - | - | - |  |
| NS_35 | 6.6.2.2.3.5 | 71 | 5, 10, 15, 20 | N/A |  |  |
| NS_36 |  | 68 | 5, 10, 15 | Table 6.2.4.3-29 |  |  |
| … |  |  |  |  |  |  |
| NS_38 | 6.6.3.3.3.27 | 74 | 3, 5, 10, 15, 20 | Table 6.2.4.3-30 |  |  |
| NS_39 | 6.6.3.3.3.28 | 74 | 10, 15, 20 | Table 6.2.4.3-31 |  |  |
| Note 1 Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in Note2 as the emissions requirement in 6.6.3.3.3.2 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. Note 2 Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. Note 3: Applicable when the E-UTRA carrier is within 1920-1980 MHz. Note 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. Note 5: Applicable only for an LAA Scell configured in Band 46. |  |  |  |  |  |  |

Table 6.2.4.3-2: A-MPR for NS_04 for bandwidths > 5MHz (Rel-11 and earlier)

| Channel bandwidth [MHz] | Parameters | Region A | Region B |  | Region C |
| --- | --- | --- | --- | --- | --- |
| 10 | RBstart 1 | 0 – 12 | 13 – 36 |  | 37 – 49 |
|  | RBstart 1 + LCRB2 [RBs] | N/A (Note 3) | 14 - 37 | >37 | N/A (Note 3) |
|  | A-MPR [dB] | ≤3dB | 0 | ≤2dB | ≤3dB |
| 15 | RBstart 1 | 0 – 18 | 19 – 55 |  | 56 – 74 |
|  | RBstart 1 + LCRB2 [RBs] | N/A (Note 3) | 20 - 56 | >56 | N/A (Note 3) |
|  | A-MPR [dB] | ≤3dB | 0 | ≤2dB | ≤3dB |
| 20 | RBstart 1 | 0 – 24 | 25 – 74 |  | 75 – 99 |
|  | RBstart 1 + LCRB2 [RBs] | N/A (Note 3) | 26 - 75 | >75 | N/A (Note 3) |
|  | A-MPR [dB] | ≤3dB | 0 | ≤2dB | ≤3dB |
| Note 1: RBstart indicates the lowest RB index of transmitted resource blocks. Note 2: LCRB is the length of a contiguous resource block allocation. Note 3: Any RB allocation that starts in Region A or C is allowed the specified A-MPR. Note 4: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. Note 5: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. |  |  |  |  |  |

Table 6.2.4-3: A-MPR requirements for "NS_04" with bandwidth >5MHz (Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0)

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2499.5 |  |  |  |  |  | > 2499.5 |
|  | RBstart | 0 - 8 |  |  |  | 9 – 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 2 |  |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  |  | > 2504 |
|  | RBstart | 0 - 8 |  |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 |  | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A |  | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 | ≤ 1 |  | ≤ 2 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  |  | > 2510.8 |
|  | RBstart | 0 - 13 |  |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 or ≥ 36 |  | > 18 and < 36 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 or ≥ 40 |  | > 18 and < 40 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |  |  |  |

Table 6.2.4.3-4: A-MPR requirements for "NS_04" for Power Class 3 UE (Rel-12 and later and bit 2 of modifiedMPRbehavior = 1)

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  |  | 9 – 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 2 |  |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  |  | > 2504 |
|  | RBstart | 0 - 8 |  |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 |  | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A |  | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 | ≤ 1 |  | ≤ 2 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  |  | > 2510.8 |
|  | RBstart | 0 - 13 |  |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 or ≥ 36 |  | > 18 and < 36 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 or ≥ 40 |  | > 18 and < 40 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |  |  |  |

Table 6.2.4.3-5: A-MPR for "NS_05"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Fc [MHz] | 1932.5 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-7 | 8 – 66 |  |  |  |  |  | 67-74 |  |  |  |
|  | LCRB [RBs] | ≥1 | ≤30 |  | 31 – 54 |  | >54 |  | ≤6 |  | >6 |  |
|  | A-MPR [dB] | ≤11 | 0 |  | ≤3 |  | ≤5 |  | ≤5 |  | ≤1 |  |
| 20 | Fc [MHz] | 1930 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-23 | 24-75 |  |  |  |  |  |  | 76-99 |  |  |
|  | LCRB [RBs] | ≥1 | ≤24 | 25 – 40 |  | 41 – 50 |  | > 50 |  | ≤6 |  | >6 |
|  | A-MPR [dB] | ≤11 | 0 | ≤3 |  | ≤5 |  | ≤10 |  | ≤5 |  | ≤1 |

Table 6.2.4.3-6: A-MPR for "NS_07"

| Parameters | Region A |  | Region B |  |  |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBstart 1 | 0 – 12 |  | 13 –18 |  | 19 – 42 |  | 43 – 49 |  |
| LCRB2 [RBs] | 6 – 8 | 1 to 5 and 9-50 | <8 | ≥8 | <18 | ≥18 | ≤2 | >2 |
| A-MPR [dB] | ≤8 | ≤12 | 0 | ≤12 | 0 | ≤6 | ≤3 | 0 |
| Note 1: RBstart indicates the lowest RB index of transmitted resource blocks Note 2: LCRB is the length of a contiguous resource block allocation Note 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. Note 4: For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. |  |  |  |  |  |  |  |  |

Table 6.2.4.3-7: A-MPR for "NS_10"

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart 1 | 0 – 10 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 2 |
| 20 | RBstart 1 | 0 – 15 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 5 |
| Note 1: RBstart indicates the lowest RB index of transmitted resource blocks. Note 2: LCRB is the length of a contiguous resource block allocation. Note 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. Note 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. |  |  |

Table 6.2.4.3-8: A-MPR for NS_11

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Fc [MHz] | <2004 |  |  | ≥2004 |  |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-15 |  |  | >5 |  |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤5 |  |  | ≤ 1 |  |  |  |  |  |  |  |  |  |  |
| 5 | Fc [MHz] | <2004 |  |  | 2004 ≤ Fc <2007 |  |  |  |  |  |  | ≥2007 |  |  |  |
|  | LCRB [RBs] | 1-25 |  |  | 1-6 & 15-25 |  |  |  | 8-12 |  |  | >6 |  |  |  |
|  | A-MPR [dB] | ≤7 |  |  | ≤ 4 |  |  |  | 0 |  |  | ≤ 1 |  |  |  |
| 10 | Fc [MHz] | 2005 ≤ Fc <2015 |  |  |  |  |  |  | 2015 |  |  |  |  |  |  |
|  | RBstart | 0-49 |  |  |  |  |  |  | 0-49 |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-50 |  |  |  |  |  |  | 1-50 |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 12 |  |  |  |  |  |  | 0 |  |  |  |  |  |  |
| 15 | Fc [MHz] | <2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-4 | 5-21 |  |  |  |  |  |  |  | 22-56 |  |  |  | 57-74 |
|  | LCRB [RBs] | ≥1 | 7-50 |  |  | 0-6 & ≥50 |  |  |  |  | ≤25 |  | >25 |  | >0 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  | ≤10 |  |  |  |  | 0 |  | ≤6 |  | ≤15 |
|  | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 |  | 13-39 |  |  |  |  |  | 40-65 |  |  |  | 66-74 |  |
|  | LCRB [RBs] | ≥1 |  | ≥30 |  |  |  | <30 |  | ≥ (69 – RBstart) |  |  |  | ≥1 |  |
|  | A-MPR [dB] | ≤10 |  | ≤6 |  |  |  | 0 |  | ≤2 |  |  |  | ≤6.5 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 | 13-29 |  |  |  |  |  |  | 30-68 |  |  |  |  | 69-99 |
|  | LCRB [RBs] | ≥1 | 10-60 |  |  |  | 1-9 & >60 |  |  | 1-24 |  |  | ≥25 |  | ≥1 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  |  | ≤10 |  |  | 0 |  |  | ≤7 |  | ≤15 |

Table 6.2.4.3-9: A-MPR for "NS_12" (Rel-11 and earlier)

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 1.4 | RBstart | 0 |  | 1-2 |
|  | LCRB [RBs] | ≤3 | ≥4 | ≥4 |
|  | A-MPR [dB] | ≤3 | ≤6 | ≤3 |
| 3 | RBstart | 0-3 |  | 4-5 |
|  | LCRB [RBs] | 4-9 | 1-3 and 10-15 | ≥9 |
|  | A-MPR [dB] | ≤4 | ≤3 | ≤3 |
| 5 | RBstart | 0-6 |  | 7-9 |
|  | LCRB [RBs] | ≤8 | ≥9 | ≥15 |
|  | A-MPR [dB] | ≤5 | ≤3 | ≤3 |

Table 6.2.4.3-10: A-MPR for "NS_12" (Rel-12 and later)

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 1.4 | RBstart | 0 |  | 1-2 |
|  | LCRB [RBs] | ≤3 | ≥4 | ≥4 |
|  | A-MPR [dB] | ≤3 | ≤6 | ≤3 |
| 3 | RBstart | 0-3 |  | 4-5 |
|  | LCRB [RBs] | 1-15 |  | ≥9 |
|  | A-MPR [dB] | ≤4 |  | ≤3 |
| 5 | RBstart | 0-6 |  | 0-9 |
|  | LCRB [RBs] | ≤8 |  | ≥9 |
|  | A-MPR [dB] | ≤5 |  | ≤3 |
| 10 | RBstart | 0-15 |  | 0-22 |
|  | LCRB [RBs] | ≤18 |  | ≥20 |
|  | A-MPR [dB] | ≤4 |  | ≤2 |
| 15 | RBstart | 0-30 |  | 0-30 |
|  | LCRB [RBs] | ≤30 |  | ≥32 |
|  | A-MPR [dB] | ≤4 |  | ≤3 |

Table 6.2.4.3-11: A-MPR for "NS_13"

| Channel bandwidth [MHz] | Parameters | Region A |  |
| --- | --- | --- | --- |
| 5 | RBstart | 0-2 |  |
|  | LCRB [RBs] | ≤5 | ≥18 |
|  | A-MPR [dB] | ≤3 | ≤2 |

Table 6.2.4.3-12: A-MPR for "NS_14"

| Channel bandwidth [MHz] | Parameters | Region A |  |
| --- | --- | --- | --- |
| 10 | RBstart | 0 |  |
|  | LCRB [RBs] | ≤5 | ≥50 |
|  | A-MPR [dB] | ≤3 | ≤1 |
| 15 | RBstart | ≤8 |  |
|  | LCRB [RBs] | ≤16 | ≥50 |
|  | A-MPR [dB] | ≤3 | ≤1 |

Table 6.2.4.3-13: A-MPR for "NS_15" for E-UTRA highest channel edge ≤ 845 MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 5 | RBend [RB] |  |  | 19-24 |
|  | LCRB [RB] |  |  | ≥18 |
|  | A-MPR [dB] |  |  | ≤2 |
| 10 | RBend [RB] | 0-4 | 29-44 | 45-49 |
|  | LCRB [RB] | ≤2 | ≥24 | >0 |
|  | A-MPR [dB] | ≤4 | ≤4 | ≤9 |
| 15 | RBend [RB] | 0-12 | 44-61 | 62-74 |
|  | LCRB [RB] | ≤2 | ≥20 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |

Table 6.2.4.3-14: A-MPR for "NS_15" for E-UTRA highest channel edge > 845 MHz and ≤ 849 MHz

| Channel bandwidth [MHz] |  |  | Parameters |  |  | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 |  |  | RBend [RB] |  |  |  |  |  |  |  |  | 4-5 |  |  |
|  |  |  | A-MPR [dB] |  |  |  |  |  |  |  |  | ≤3 |  |  |
| 3 |  |  | RBend [RB] |  |  | 0-1 |  |  | 8-12 |  |  | 13-14 |  |  |
|  |  |  | LCRB [RB] |  |  | ≤2 |  |  | ≥8 |  |  | >0 |  |  |
|  |  |  | A-MPR [dB] |  |  | ≤4 |  |  | ≤4 |  |  | ≤9 |  |  |
| 5 |  |  | RBend [RB] |  |  | 0-4 |  |  | 12-19 |  |  | 20-24 |  |  |
|  |  |  | LCRB [RB] |  |  | ≤2 |  |  | ≥8 |  |  | >0 |  |  |
|  |  |  | A-MPR [dB] |  |  | ≤4 |  |  | ≤5 |  |  | ≤9 |  |  |
| 10 |  |  | RBend [RB] |  |  | 0-12 |  |  | 23-36 |  |  | 37-49 |  |  |
|  |  |  | LCRB [RB] |  |  | ≤2 |  |  | ≥15 |  |  | >0 |  |  |
|  |  |  | A-MPR [dB] |  |  | ≤4 |  |  | ≤6 |  |  | ≤9 |  |  |
| 15 |  |  | RBend [RB] |  |  | 0-20 |  |  | 26-53 |  |  | 54-74 |  |  |
|  |  |  | LCRB [RB] |  |  | ≤2 |  |  | ≥20 |  |  | >0 |  |  |
|  |  |  | A-MPR [dB] |  |  | ≤4 |  |  | ≤5 |  |  | ≤9 |  |  |

Table 6.2.4.3-15: A-MPR for “NS_16” with channel lower edge at ≥807 MHz and <808.5 MHz

| Channel band-width [MHz] | Parameter | Region A | Region B | Region C | Region D | Region E |
| --- | --- | --- | --- | --- | --- | --- |
| 3 MHz | RBstart | 0 | 1-2 |  |  |  |
|  | LCRB [RBs] | ≥12 | 12 |  |  |  |
|  | A-MPR [dB] | ≤2 | ≤1 |  |  |  |
| 5 MHz | RBstart | 0-1 | 2 | 2-9 | 2-5 |  |
|  | LCRB [RBs] | 1 - 25 | 12 | 15-18 | 20 |  |
|  | A-MPR [dB] | ≤5 | ≤1 | ≤2 | ≤3 |  |
| 10 MHz | RBstart | 0 - 8 | 0-14 |  | 15-20 | 15-24 |
|  | LCRB [RBs] | 1 - 12 | 15-20 | ≥24 | ≥30 | 24-27 |
|  | A-MPR [dB] | ≤5 | ≤3 | ≤7 | ≤3 | ≤1 |

Table 6.2.4.3-16: A-MPR for “NS_16” with channel lower edge at ≥808.5 MHz and <812 MHz

| Channel band-width [MHz] | Parameter | Region A | Region B | Region C | Region D | Region E |
| --- | --- | --- | --- | --- | --- | --- |
| 5 MHz | RBstart | 0 | 0-1 | 1-5 |  |  |
|  | LCRB [RBs] | 16-20 | ≥24 | 16-20 |  |  |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤1 |  |  |
| 10 MHz | RBstart | 0-6 |  | 0-10 | 0-14 | 11-20 |
|  | LCRB [RBs] | 1-12 | 15-20 | 24-32 | ≥36 | 24-32 |
|  | A-MPR [dB] | ≤5 | ≤2 | ≤4 | ≤5 | ≤1 |

Table 6.2.4.3-17: A-MPR for “NS_16” with channel lower edge at ≥812 MHz

| Channel band-width [MHz] | Parameter | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 10 MHz | RBstart | 0 - 9 | 0 | 1-14 | 0-5 |
|  | LCRB [RBs] | 27-32 | 36-40 | 36-40 | ≥45 |
|  | A-MPR [dB] | ≤1 | ≤2 | ≤1 | ≤3 |

Table 6.2.4.3-18: A-MPR for “NS_19”

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 10 | RBstart |  |  | 0-6 |
|  | LCRB [RBs] |  |  | ≥40 |
|  | A-MPR [dB] |  |  | ≤1 |
| 15 | RBstart | 0-6 |  | 7-20 |
|  | LCRB [RBs] | ≤18 | ≥36 | ≥42 |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤2 |
| 20 | RBstart | 0-14 |  | 15-30 |
|  | LCRB [RBs] | ≤40 | ≥45 | ≥50 |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤2 |

Table 6.2.4.3-19: A-MPR for “NS_20”

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc (MHz) | < 2007.5 |  |  | 2007.5 ≤ Fc < 2012.5 |  |  |  |  |  |  |  |  |  |  |  | 2012.5 ≤ Fc ≤ 2017.5 |  |  |  |
|  | RBstart | ≤24 |  |  | 0-3 |  |  |  |  |  |  |  | 4-6 |  |  |  | ≤24 |  |  |  |
|  | LCRB (RBs) | >0 |  |  | 15-19 |  |  |  | ≥20 |  |  |  | ≥18 |  |  |  | 1-25 |  |  |  |
|  | A-MPR [dB] | ≤17 |  |  | ≤1 |  |  |  | ≤4 |  |  |  | ≤2 |  |  |  | ≤ 0 |  |  |  |
| 10 | Fc (MHz) | 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-25 |  |  |  |  | 26-34 |  |  |  |  |  |  |  |  | 35-49 |  |  |  |  |
|  | LCRB (RBs) | >0 |  |  |  |  | 8-15 |  |  |  |  | >15 |  |  |  | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤16 |  |  |  |  | ≤2 |  |  |  |  | ≤5 |  |  |  | ≤ 6 |  |  |  |  |
|  | Fc (MHz) | 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-5 |  |  |  |  |  |  |  |  | 6-10 |  |  |  |  |  |  |  |  |  |
|  | LCRB (RBs) | ≥32 |  |  |  |  |  |  |  |  | ≥40 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |  |  |  |  |  | ≤2 |  |  |  |  |  |  |  |  |  |
| 15 | Fc (MHz) | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-14 |  |  |  |  |  | 15-24 |  |  |  |  |  |  |  |  | 25-39 |  | 61-74 |  |
|  | LCRB (RBs) | 1-9 & 40-75 |  | 10-39 |  |  |  | 24-29 |  |  |  |  |  | ≥30 |  |  | ≥36 |  | ≤6 |  |
|  | A-MPR [dB] | ≤11 |  | ≤6 |  |  |  | ≤1 |  |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤6 |  |
| 20 | Fc (MHz) | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-21 | 22-31 |  |  |  |  |  |  | 32-38 |  |  |  |  | 39-49 |  |  | 50-69 |  | 70-99 |
|  | LCRB (RBs) | >0 | 1-9 & 31-75 |  |  | 10-30 |  |  |  | ≥15 |  |  |  |  | ≥24 |  |  | ≥25 |  | >0 |
|  | A-MPR [dB] | ≤17 | ≤12 |  |  | ≤6 |  |  |  | ≤9 |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤16 |
| NOTE 1: When NS_20 is signalled the minimum requirements for the 10 MHz bandwidth are specified for E-UTRA UL carrier centre frequencies of 2005 MHz or 2015 MHz. NOTE 2: When NS_20 is signalled the minimum requirements for the 15 MHz channel bandwidth are specified for E-UTRA UL carrier centre frequency of 2012.5 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4.3-20: A-MPR for "NS_21"

| Channel Bandwidth [MHz] | Parameters | Region A |  | Region B |  |
| --- | --- | --- | --- | --- | --- |
| 10 | RBstart | 0 – 6 | 0 – 6 | N/A | N/A |
|  | RBend | N/A | N/A | 43 – 49 | 43 – 49 |
|  | LCRB [RBs] | 1 – 2 | 3 – 12, 32 - 50 | 1 – 2 | 3 – 12, 32 - 50 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 |

Table 6.2.4.3-21: A-MPR for "NS_22"

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 5 | No A-MPR is needed for 5 MHz channel bandwidth |  |  |  |  |
| 10 | RBstart | 0-13 | 0-17 | ≤ 6 | ≥12 |
|  | LCRB [RBs] | > 36 | 33-36 | ≤ 32 | ≤ 32 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥44 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 3 |
| 15 | RBstart | 0-24 | 0-38 | ≤ 14 | ≥ 23 |
|  | LCRB [RBs] | > 50 | 37-50 | ≤ 36 | ≤ 36 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥59 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| 20 | RBstart | 0-35 | 0-51 | ≤ 21 | ≥ 31 |
|  | LCRB [RBs] | > 64 | 49-64 | ≤ 48 | ≤ 48 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥79 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. NOTE 4: For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. |  |  |  |  |  |

Table 6.2.4.3-22: A-MPR for "NS_24"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | Fc > [1987.5] |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0 – 24 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 0 – 24 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 10 |  |  |  |  |  |  |  |  |  |
| 10 | Fc [MHz] | 1975 < Fc ≤ 1985 |  |  |  |  |  |  | 1985<Fc≤1995 |  | Fc>1995 |
|  | RBstart | 0 - 1 | 2 - 14 |  | 15 - 26 | 36 - 49 |  |  | 0 - 49 |  | 0 - 49 |
|  | LCRB [RBs] | > 10 | ≥ 35 |  | N/A | ≤ 2 |  | > 11 | 0 - 49 |  | 0 - 49 |
|  | RBend | N/A | N/A |  | > 48 | N/A |  | N/A | N/A |  | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 2 |  | 1 | ≤ 3 |  | ≤ 1 | ≤ 9 |  | ≤ 17 |
| 15 | Fc [MHz] | 1972.5 < Fc ≤ 1987.5 |  |  |  |  |  |  |  | Fc > 1987.5 |  |
|  | RBstart | 0 - 11 |  |  |  |  | 12 - 74 |  |  | 0 - 74 |  |
|  | LCRB [RBs] | ≤ 45 |  | > 45 |  |  | > 3 |  |  | 0 - 74 |  |
|  | RBend | N/A |  | N/A |  |  | ≥ 45 |  |  | N/A |  |
|  | A-MPR [dB] | ≤ 2 |  | ≤ 8 |  |  | ≤ 7 |  |  | ≤ 17 |  |
| 20 | Fc [MHz] | Fc > 1970 |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0 – 99 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 0 – 99 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 17 |  |  |  |  |  |  |  |  |  |

Table 6.2.4.3-23: A-MPR for "NS_25"

| Channel Bandwidth [MHz] |  | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] |  | Fc > [1997.5] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart |  | 0 - 9 |  |  |  |  |  |  |  | 10 – 24 |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  | > 12 |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
|  | RBend |  | N/A |  |  |  |  |  |  |  | ≥ 22 |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  | ≤ 5 |  |  |  |  |  |  |  | ≤ 2 |  |  |  |  |  |  |  |
| 10 | Fc [MHz] |  | 1975 < Fc ≤ 1985 |  |  | 1985 < Fc ≤ 1995 |  |  |  |  |  |  |  | Fc > 1995 |  |  |  |  |
|  | RBstart |  | 0-1 | 2-49 |  | 0 |  |  |  | 1 - 18 |  | 19-49 |  | 0-6 |  | 7-15 |  | 16-49 |
|  | LCRB [RBs] |  | > 10 | N/A |  | ≤ 25 |  | > 25 |  | > 25 |  | N/A |  | N/A |  | > 20 |  | N/A |
|  | RBend |  | N/A | > 48 |  | N/A |  | N/A |  | N/A |  | > 42 |  | N/A |  | N/A |  | > 35 |
|  | A-MPR [dB] |  | ≤ 1 | ≤ 1 |  | ≤ 1 |  | ≤ 5 |  | ≤ 5 |  | ≤ 1 |  | ≤ 10 |  | ≤ 7 |  | ≤ 11 |
| 15 | Fc [MHz] |  | 1972.5 < Fc ≤ 1987.5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Fc > 1987.5 |  |
|  | RBstart |  | 0 - 4 |  | 5 - 30 |  |  |  | 31 - 62 |  |  |  | 63 - 74 |  |  |  | 0 - 74 |  |
|  | LCRB [RBs] |  | ≥ 15 |  | ≥ 45 |  |  |  | N/A |  |  |  | N/A |  |  |  | 0 - 74 |  |
|  | RBend |  | N/A |  | N/A |  |  |  | > 71 |  |  |  | N/A |  |  |  | N/A |  |
|  | A-MPR [dB] |  | ≤ 4 |  | ≤ 3 |  |  |  | ≤ 1 |  |  |  | ≤ 1 |  |  |  | ≤ 13 |  |
| 20 | Fc [MHz] |  | 1970 < Fc ≤ 1990 |  |  |  |  |  |  |  |  |  |  |  | Fc > 1990 |  |  |  |
|  | RBstart |  | 0 - 13 |  |  |  | 14 - 40 |  |  |  | 41 - 99 |  |  |  | 0 - 99 |  |  |  |
|  | LCRB [RBs] |  | N/A |  |  |  | ≥ 32 |  |  |  | N/A |  |  |  | 0 - 99 |  |  |  |
|  | RBend |  | N/A |  |  |  | N/A |  |  |  | > 72 |  |  |  | N/A |  |  |  |
|  | A-MPR [dB] |  | ≤ 11 |  |  |  | ≤ 11 |  |  |  | ≤13 |  |  |  | ≤ 13 |  |  |  |

Table 6.2.4.3-24: A-MPR for "NS_27“

| Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | RBstart | RBend | LCRB | A-MPR |
| 15 | 0 – 6 |  | ≤ 15 | ≤ 4 dB |
|  |  | 68 – 74 |  |  |
|  | ≥ 0 |  | ≥ 60 | ≤ 2 dB |
| 20 | 0 – 12 |  | ≤ 20 | ≤ 4 dB |
|  |  | 87 – 99 |  |  |
|  | 13 – 15 |  | ≤ 3 | ≤ 1 dB |
|  |  | 84 – 86 |  |  |
|  | ≥ 0 |  | ≥ 60 | ≤ 3 dB |

Table 6.2.4-25: A-MPR for "NS_28"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 51601, 53402, 54801 | L = 10 (RIV = 19) | 9 |
|  |  | Any other RIV | 8 |
|  | 57003 | L = 10 (RIV = 19) | [9] |
|  |  | Any other RIV | [8] |
|  | 51803, 52003, 52203, 52403, 52603, 52803, 53003, 53203, 55003, 55203, 55403, 55603, 55803, 56003, 56203, 56403, 56603, 56803 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 2 |
| NOTE 1: Applies also for carrier centre frequencies at a frequency offset of +100 kHz or +200 kHz from the carrier centre frequency stated. NOTE 2: Applies also for carrier centre frequencies at a frequency offset of-200 kHz or -100 kHz from the carrier centre frequency stated. NOTE 3: Applies also for carrier centre frequencies at a frequency offset of -200 kHz, -100 kHz, 100 kHz or +200 kHz from the carrier centre frequency stated. NOTE 4: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [10]. |  |  |  |

Table 6.2.4-26: A-MPR for "NS_29"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 51801, 52001, 52201, 52401, 52601, 52801, 53001, 53201, 55001, 55201, 55401, 55601, 55801, 56001, 56201, 56401, 56601, 56801, 57001 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 1 |
| NOTE 1: Applies also for carrier centre frequencies at a frequency offset of -200 kHz, -100 kHz, 100 kHz or +200 kHz from the carrier centre frequency stated NOTE 2: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [10]. |  |  |  |

Table 6.2.4-27: A-MPR for "NS_30"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 51601, 52402 | L = 10 (RIV = 19) | 15 |
|  |  | Any other RIV | 16 |
|  | 51803, 53203 | L = 10 (RIV = 19) | 3 |
|  |  | Any other RIV | 3 |
|  | 54801 | L = 10 (RIV = 19) | 11 |
|  |  | Any other RIV | 11 |
|  | 57003 | L = 10 (RIV = 19) | [6] |
|  |  | Any other RIV | [6] |
|  | 51803, 52003, 52203, 52403, 52603, 52803, 53003, 55003, 55203, 55403, 55603, 55803, 56003, 56203, 56403, 56603, 56803 | L = 10 (RIV = 19) | 1 |
|  |  | Any other RIV | N/A |
| NOTE 1: Applies also for carrier centre frequencies at a frequency offset of +100 kHz or +200 kHz from the carrier centre frequency stated. NOTE 2: Applies also for carrier centre frequencies at a frequency offset of-200 kHz or -100 kHz from the carrier centre frequency stated. NOTE 3: Applies also for carrier centre frequencies at a frequency offset of -200 kHz, -100 kHz, 100 kHz or +200 kHz from the carrier centre frequency stated. NOTE 4: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [10]. The uplink resource allocation is defined in Clause 8.1.4 of [10]. |  |  |  |

Table 6.2.4-28: A-MPR for "NS_31"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 51601, 51803, 52003, 52203, 52402 | L = 10 (RIV = 19) | 7 |
|  |  | Any other RIV | 7 |
|  | 52601, 53402, 54801 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 57003, 58253 | L = 10 (RIV = 19) | [6] |
|  |  | Any other RIV | [6] |
|  | 57453 | L = 10 (RIV = 19) | [6] |
|  |  | Any other RIV | [6] |
|  | 52803, 53003, 53203, 55003, 55203, 55403, 55603, 55803, 56003, 56203, 56403, 56603, 56803, 57653, 57853, 58053, 58253 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 1 |
| NOTE 1: Applies also for carrier centre frequencies at a frequency offset of +100 kHz or +200 kHz from the carrier centre frequency stated. NOTE 2: Applies also for carrier centre frequencies at a frequency offset of-200 kHz or -100 kHz from the carrier centre frequency stated. NOTE 3: Applies also for carrier centre frequencies at a frequency offset of -200 kHz, -100 kHz, 100 kHz or +200 kHz from the carrier centre frequency stated. NOTE 4: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [10]. |  |  |  |

Table 6.2.4.3-29: A-MPR for “NS_36”

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 MHz | Fc [MHz] | 700.5 ≤ Fc < 705.5 |  |  |  |
|  | RBstart | 0 |  | 1-4 |  |
|  | LCRB [RBs] | 1 | ≥ 15 | ≥ 15 | 24 |
|  | A-MPR [dB] | ≤ 2 | ≤ 4 | ≤ 1 | ≤ 3 |
| 10 MHz | Fc [MHz] | 703 ≤ Fc < 708 |  |  |  |
|  | RBstart | 0 - 8 | 9 - 12 | 13 - 39 | 40 - 43 |
|  | LCRB [RBs] | > 0 | > 12 | > 16 | > 0 |
|  | A-MPR [dB] | ≤ 10 | ≤ 8 | ≤ 6 | ≤ 6 |
| 15 MHz | Fc [MHz] | Fc = 705.5 |  |  |  |
|  | RBstart | 0 - 16 | 17 - 23 | 24 - 56 | 57 - 60 |
|  | LCRB [RBs] | > 0 | > 12 | > 20 | < 6 |
|  | A-MPR [dB] | ≤ 10 | ≤ 9 | ≤ 7 | ≤ 6 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4.3-30: A-MPR for "NS_38"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  | Region E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 1428.5 ≤ FC < 1431.5 | RBstart | 0 |  | 1– 2 |  | 3 – 6 |  |  |  |  |  |
|  |  | LCRB [RBs] | 1 | ≥2 | 2 | ≥ 3 | 5-7 | ≥8 |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 11 | ≤ 16 | ≤ 9 | ≤ 15 | ≤ 12 | ≤ 14 |  |  |  |  |
| 5 | 1429.5 ≤ FC < 1434.5 | RBstart | 0 - 1 |  | 2 – 4 |  | 5 – 7 |  | 8 – 10 |  |  |  |
|  |  | LCRB [RBs] | 2 | ≥3 | 4-6 | ≥ 7 | 8-10 | ≥11 | 12-14 | ≥15 |  |  |
|  |  | A-MPR [dB] | ≤ 12 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 15 | ≤ 10 | ≤ 12 |  |  |
| 10 | 1432 ≤ FC < 1442 | RBstart | 0 |  | 1 – 4 |  | 5 – 9 |  | 10 – 14 |  | 15 – 21 |  |
|  |  | LCRB [RBs] | 3-4 | ≥5 | 5-8 | ≥ 9 | 9-16 | ≥17 | 15-23 | ≥24 | 24-29 | ≥30 |
|  |  | A-MPR [dB] | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 11 | ≤ 14 | ≤ 11 | ≤ 13 |
| 15 | 1434.5 ≤ FC < 1447.5 | RBstart | 0 -1 |  | 2 – 7 |  | 8 – 17 |  | 18 – 23 |  | 24 – 30 |  |
|  |  | LCRB [RBs] | 5-10 | ≥11 | 8-12 | ≥ 13 | 15-25 | ≥26 | 26-40 | ≥41 | ≥ 36 |  |
|  |  | A-MPR [dB] | ≤ 14 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 14 | ≤ 12 |  |
| 20 | 1437 ≤ FC < 1450 | RBstart | 0 -4 |  | 5 – 9 |  | 10 – 19 |  | 20 – 29 |  | 30 – 40 |  |
|  |  | LCRB [RBs] | 6-10 | ≥11 | 11-20 | ≥ 21 | 17-32 | ≥33 | 28-45 | ≥46 | 41-54 | ≥55 |
|  |  | A-MPR [dB] | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 14 | ≤ 11 | ≤ 13 |

Table 6.2.4.3-31: A-MPR for "NS_39"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 1462 < FC ≤1465 | RBstart | 43-50 |  |  |  |  |  |  |
|  |  | LCRB [RBs] | ≤ 3 | > 3 and ≤ 25 | ≥ 32 |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 |  |  |  |  |
| 15 | 1456.3 < FC ≤ 1462.5 | RBstart | 56-75 |  |  | 64-75 |  | 60-75 |  |
|  |  | LCRB [RBs] | ≥ 36 |  |  | ≥ 13 and ≤ 35 |  | ≥ 5 and ≤ 12 | < 5 |
|  |  | A-MPR [dB] | ≤ 3 |  |  | ≤ 2 |  | ≤ 3 | ≤ 4 |
| 20 | 1450.8 < FC ≤ 1460 | RBstart | 71-100 |  |  | >76 |  |  |  |
|  |  | LCRB [RBs] | ≥ 36 |  |  | ≥ 7 and ≤ 35 | < 7 |  |  |
|  |  | A-MPR [dB] | ≤ 3 |  |  | ≤ 3 | ≤ 4 |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each subframe, the A-MPR is evaluated per slot and given by the maximum value taken over the transmission(s) within the slot; the maximum A-MPR over the two slots is then applied for the entire subframe.

The normative reference for this requirement is TS 36.101 clause 6.2.4.

#### 6.2.4.4	Test description

##### 6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4.4.1-1 through table 6.2.4.4.1-26. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for NS_03 A-MPR |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |  | FDD | TDD |
| 1 | 1.4MHz | N/A for A-MPR testing. |  |  | QPSK | 6 | 6 |
| 2 | 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 3 | 1.4MHz |  |  |  | 16QAM (Note 6) | 5 | 5 |
| 4 | 3MHz |  |  |  | QPSK | 15 | 15 |
| 5 | 3MHz |  |  |  | QPSK | 4 | 4 |
| 6 | 3MHz |  |  |  | 16QAM (Note 6) | 15 | 15 |
| 7 | 3MHz |  |  |  | 16QAM (Note 6) | 4 | 4 |
| 8 | 5MHz |  |  |  | QPSK | 25 | 25 |
| 9 | 5MHz |  |  |  | QPSK | 8 | 8 |
| 10 | 5MHz |  |  |  | QPSK | 6 | 6 |
| 11 | 5MHz |  |  |  | 16QAM (Note 6) | 25 | 25 |
| 12 | 5MHz |  |  |  | 16QAM (Note 6) | 8 | 8 |
| 13 | 10MHz |  |  |  | QPSK | 50 | 50 |
| 14 | 10MHz |  |  |  | QPSK | 12 | 12 |
| 15 | 10MHz |  |  |  | QPSK | 6 | 6 |
| 16 | 10MHz |  |  |  | 16QAM (Note 6) | 50 (Note 4) | 50 (Note 4) |
| 17 | 10MHz |  |  |  | 16QAM (Note 6) | 12 | 12 |
| 18 | 15MHz |  |  |  | QPSK | 75 | 75 |
| 19 | 15MHz |  |  |  | QPSK | 16 | 16 |
| 20 | 15MHz |  |  |  | QPSK | 8 | 8 |
| 21 | 15MHz |  |  |  | 16QAM (Note 6) | 75 (Note 4) | 75 (Note 4) |
| 22 | 15MHz |  |  |  | 16QAM (Note 6) | 16 | 16 |
| 23 | 20MHz |  |  |  | QPSK | 100 | 100 |
| 24 | 20MHz |  |  |  | QPSK | 18 | 18 |
| 25 | 20MHz |  |  |  | QPSK | 10 | 10 |
| 26 | 20MHz |  |  |  | 16QAM (Note 6) | 100 (Note 4) | 100 (Note 4) |
| 27 | 20MHz |  |  |  | 16QAM (Note 6) | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: For band 23, above table only applies to mid and high range test frequencies. Low range test frequencies will be covered by NS_11 test configuration table. Note 6: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.2.4.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.2.4.4.1-3: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10 MHz Channel Bandwidth: 1934.7 MHz (NUL = 18147) - For 15 MHz Channel Bandwidth: 1932.5 MHz (NUL = 18125) - For 20 MHz Channel Bandwidth: 1930 MHz (NUL = 18100) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_05 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | 10MHz |  |  | QPSK | 1 | 0 |
| 5 | 10MHz |  |  | QPSK | 1 | 49 |
| 6 | 10MHz |  |  | QPSK | 12 | 0 |
| 7 | 10MHz |  |  | QPSK | 12 | 38 |
| 8 | 10MHz |  |  | QPSK | 48 | 0 |
| 9 | 10MHz |  |  | QPSK | 48 | 2 |
| 10 (Note 8) | 10MHz |  |  | QPSK | 50 | 0 |
| 11 (Note 8) | 10MHz |  |  | 16QAM (Note 7) | 50 (Note 4) | 0 |
| 12 (Note 9) | 15MHz |  |  | QPSK | 1 | 8 |
| 13 (Note 9) | 15MHz |  |  | QPSK | 1 | 66 |
| 14 (Note 9) | 15MHz |  |  | QPSK | 16 | 8 |
| 15 (Note 9) | 15MHz |  |  | QPSK | 16 | 51 |
| 16 (Note 9) | 15MHz |  |  | QPSK | 30 | 8 |
| 17 (Note 9) | 15MHz |  |  | QPSK | 30 | 37 |
| 18 (Note 9) | 20MHz |  |  | QPSK | 1 | 24 |
| 19 (Note 9) | 20MHz |  |  | QPSK | 1 | 75 |
| 20 (Note 9) | 20MHz |  |  | QPSK | 18 | 24 |
| 21 (Note 9) | 20MHz |  |  | QPSK | 18 | 58 |
| 22 (Note 9) | 20MHz |  |  | QPSK | 24 | 24 |
| 23 (Note 9) | 20MHz |  |  | QPSK | 24 | 52 |
| 24 (Note 10) | 15MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 10) | 15MHz |  |  | QPSK | 1 | 74 |
| 26 (Note 10) | 15MHz |  |  | QPSK | 16 | 0 |
| 27 (Note 10) | 15MHz |  |  | QPSK | 16 | 59 |
| 28 (Note 10) | 15MHz |  |  | QPSK | 48 | 0 |
| 29 (Note 10) | 15MHz |  |  | QPSK | 48 | 27 |
| 30 (Note 10) | 15MHz |  |  | QPSK | 75 | 0 |
| 31 (Note 10) | 15MHz |  |  | 16QAM (Note 7) | 75 (Note 4) | 0 |
| 32 (Note 10) | 20MHz |  |  | QPSK | 1 | 0 |
| 33 (Note 10) | 20MHz |  |  | QPSK | 1 | 99 |
| 34 (Note 10) | 20MHz |  |  | QPSK | 18 | 0 |
| 35 (Note 10) | 20MHz |  |  | QPSK | 18 | 82 |
| 36 (Note 10) | 20MHz |  |  | QPSK | 48 | 0 |
| 37 (Note 10) | 20MHz |  |  | QPSK | 48 | 52 |
| 38 (Note 10) | 20MHz |  |  | QPSK | 100 | 0 |
| 39 (Note 10) | 20MHz |  |  | 16QAM (Note 7) | 100 (Note 4) | 0 |
| 40 (Note 11) | 10MHz |  |  | QPSK | 50 | 0 |
| 41 (Note 11) | 10MHz |  |  | 16QAM (Note 7) | 50 (Note 4) | 0 |
| 42 (Note 11) | 15MHz |  |  | QPSK | 1 | 0 |
| 43 (Note 11) | 15MHz |  |  | QPSK | 75 | 0 |
| 44 (Note 11) | 15MHz |  |  | QPSK | 16 | 8 |
| 45 (Note 11) | 15MHz |  |  | QPSK | 30 | 8 |
| 46 (Note 11) | 15MHz |  |  | QPSK | 54 | 8 |
| 47 (Note 11) | 15MHz |  |  | QPSK | 64 | 8 |
| 48 (Note 11) | 15MHz |  |  | QPSK | 1 | 74 |
| 49 (Note 11) | 15MHz |  |  | QPSK | 6 | 69 |
| 50 (Note 11) | 15MHz |  |  | QPSK | 8 | 67 |
| 51 (Note 11) | 20MHz |  |  | QPSK | 1 | 0 |
| 52 (Note 11) | 20MHz |  |  | QPSK | 100 | 0 |
| 53 (Note 11) | 20MHz |  |  | QPSK | 18 | 24 |
| 54 (Note 11) | 20MHz |  |  | QPSK | 24 | 24 |
| 55 (Note 11) | 20MHz |  |  | QPSK | 40 | 24 |
| 56 (Note 11) | 20MHz |  |  | QPSK | 50 | 24 |
| 57 (Note 11) | 20MHz |  |  | QPSK | 72 | 24 |
| 58 (Note 11) | 20MHz |  |  | QPSK | 1 | 99 |
| 59 (Note 11) | 20MHz |  |  | QPSK | 6 | 94 |
| 60 (Note 11) | 20MHz |  |  | QPSK | 24 | 76 |
| 61 (Note 12) | 10MHz |  |  | QPSK | 50 | 0 |
| 62 (Note 12) | 10MHz |  |  | 16QAM (Note 7) | 50 (Note 4) | 0 |
| 63 (Note 12) | 15MHz |  |  | QPSK | 1 | 0 |
| 64 (Note 12) | 15MHz |  |  | QPSK | 1 | 74 |
| 65 (Note 12) | 15MHz |  |  | QPSK | 16 | 0 |
| 66 (Note 12) | 15MHz |  |  | QPSK | 16 | 59 |
| 67 (Note 12) | 15MHz |  |  | QPSK | 48 | 0 |
| 68 (Note 12) | 15MHz |  |  | QPSK | 48 | 27 |
| 69 (Note 12) | 15MHz |  |  | QPSK | 75 | 0 |
| 70 (Note 12) | 15MHz |  |  | 16QAM (Note 7) | 75 (Note 4) | 0 |
| 71 (Note 12) | 20MHz |  |  | QPSK | 1 | 0 |
| 72 (Note 12) | 20MHz |  |  | QPSK | 1 | 99 |
| 73 (Note 12) | 20MHz |  |  | QPSK | 18 | 0 |
| 74 (Note 12) | 20MHz |  |  | QPSK | 18 | 82 |
| 75 (Note 12) | 20MHz |  |  | QPSK | 48 | 0 |
| 76 (Note 12) | 20MHz |  |  | QPSK | 48 | 52 |
| 77 (Note 12) | 20MHz |  |  | QPSK | 100 | 0 |
| 78 (Note 12) | 20MHz |  |  | 16QAM (Note 7) | 100 (Note 4) | 0 |
| Note 1: Void Note 2: Void Note 3: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 4: Applies only for UE-Categories ≥2. Note 5: Void. Note 6: Void. Note 7: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 8: Applies when bit number 1 in modifiedMPRbehavior = 0 (Rel-11 minimum requirements apply) for Band1 and not available for Band65. Note 9: Applies for low range and when bit number 1 in modifiedMPRbehavior = 0 (Rel-11 minimum requirements apply) for Band1 and not available for Band65. Note 10: Applies for mid range and when bit number 1 in modifiedMPRbehavior = 0 (Rel-11 minimum requirements apply) for Band1 and not available for Band65. Note 11: Applies only for low range and when bit number 1 in modifiedMPRbehavior = 1 (Rel-12 minimum requirements apply) for Band1 and low range for Band65. Note 12: Applies only for mid range and when bit number 1 in modifiedMPRbehavior = 1 (Rel-12 minimum requirements apply) for Band1 and mid range for Band65. Note 13: ModifiedMPRbehavior is defined in annex J.1. |  |  |  |  |  |  |

Table 6.2.4.4.1-4: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | QPSK | 6 |
| 2 | 1.4MHz |  |  | QPSK | 5 |
| 3 | 1.4MHz |  |  | 16QAM (Note 4) | 5 |
| 4 | 5MHz |  |  | QPSK | 25 |
| 5 | 5MHz |  |  | QPSK | 8 |
| 6 | 5MHz |  |  | 16QAM (Note 4) | 8 |
| 7 | 10MHz |  |  | QPSK | 50 |
| 8 | 10MHz |  |  | QPSK | 12 |
| 9 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-5: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for NS_07 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 10MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 10MHz |  |  | QPSK | 8 | 0 |
| 3 | 10MHz |  |  | QPSK | 6 | 13 |
| 4 | 10MHz |  |  | QPSK | 20 | 13 |
| 5 | 10MHz |  |  | QPSK | 12 | 13 |
| 6 | 10MHz |  |  | 16QAM (Note 3) | 36 (Note 2) | 13 |
| 7 | 10MHz |  |  | QPSK | 16 | 19 |
| 8 | 10MHz |  |  | QPSK | 12 | 19 |
| 9 | 10MHz |  |  | 16QAM (Note 3) | 16 | 19 |
| 10 | 10MHz |  |  | QPSK | 30 | 19 |
| 11 | 10MHz |  |  | 16QAM (Note 3) | 30 (Note 2) | 19 |
| 12 | 10MHz |  |  | QPSK | 6 | 43 |
| 13 | 10MHz |  |  | QPSK | 2 | 48 |
| 14 | 10MHz |  |  | QPSK | 50 | 0 |
| 15 | 10MHz |  |  | QPSK | 12 | 0 |
| 16 | 10MHz |  |  | 16QAM (Note 3) | 50 (Note 2) | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.2.4.4.1-6: Test Configuration Table (network signalled value "NS_08")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz |  |
| Test Parameters for NS_08 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 10MHz |  |  | QPSK | 1 |
| 5 | 10MHz |  |  | QPSK | 12 |
| 6 | 10MHz |  |  | QPSK | 40 |
| 7 | 10MHz |  |  | QPSK | 50 |
| 8 | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) |
| 9 | 15MHz |  |  | QPSK | 1 |
| 10 | 15MHz |  |  | QPSK | 16 |
| 11 | 15MHz |  |  | QPSK | 40 |
| 12 | 15MHz |  |  | QPSK | 75 |
| 13 | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max + 1 - RB allocation) of the channel bandwidth Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-7: Test Configuration Table (network signalled value "NS_09")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 10MHz |  |  | QPSK | 1 |
| 5 | 10MHz |  |  | QPSK | 12 |
| 6 | 10MHz |  |  | QPSK | 40 |
| 7 | 10MHz |  |  | QPSK | 50 |
| 8 | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) |
| 9 | 15MHz |  |  | QPSK | 1 |
| 10 | 15MHz |  |  | QPSK | 16 |
| 11 | 15MHz |  |  | QPSK | 40 |
| 12 | 15MHz |  |  | QPSK | 54 |
| 13 | 15MHz |  |  | QPSK | 75 |
| 14 | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max + 1 - RB allocation) of the channel bandwidth Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-8: Test Configuration Table (network signalled value "NS_10")

Void, not tested

Table 6.2.4.4.1-9: Test Configuration Table (network signalled value "NS_11")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | For 3 MHz Channel Bandwidth: a. 2001.5 MHz (NUL = 25515) b. 2004.5 MHz (NUL = 25545) For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2004.5 MHz (NUL = 25545) c. 2007.5 MHz (NUL = 25575) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2005.5 MHz (NUL = 25555) c. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2007.5 MHz (NUL = 25575) b. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_11 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A for A-MPR testing |  | QPSK | 6 |
| 2 | 3MHz |  |  | QPSK | 15 |
| 3 | 3MHz |  |  | 16QAM (Note 4) | 6 |
| 4 | 3MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | QPSK | 1 |
| 6 | 5MHz |  |  | QPSK | 8 |
| 7 | 5MHz |  |  | QPSK | 25 |
| 8 | 5MHz |  |  | 16QAM (Note 4) | 8 |
| 9 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 10 | 10MHz |  |  | QPSK | 1 |
| 11 | 10MHz |  |  | QPSK | 12 |
| 12 | 10MHz |  |  | QPSK | 50 |
| 13 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 14 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 15 | 15MHz |  |  | QPSK | 1 |
| 16 | 15MHz |  |  | QPSK | 8 |
| 17 | 15MHz |  |  | QPSK | 25 |
| 18 | 15MHz |  |  | QPSK | 30 |
| 19 | 15MHz |  |  | QPSK | 75 |
| 20 | 15MHz |  |  | 16QAM (Note 4) | 8 |
| 21 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 22 | 15MHz |  |  | 16QAM (Note 4) | 30 |
| 23 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 24 | 20MHz |  |  | QPSK | 1 |
| 25 | 20MHz |  |  | QPSK | 10 |
| 26 | 20MHz |  |  | QPSK | 25 |
| 27 | 20MHz |  |  | QPSK | 100 |
| 28 | 20MHz |  |  | 16QAM (Note 4) | 10 |
| 29 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 30 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-10: Test Configuration Table (network signalled value "NS_12")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment |  |  |  | NC |  |  |
| (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  |
| Test Frequencies |  |  |  | For 1.4 MHz Channel Bandwidth: UL 814.9 MHz (NUL = 26699) For 3 MHz Channel Bandwidth: UL 815.7 MHz (NUL = 26707) For 5 MHz Channel Bandwidth: UL 816.7 MHz (NUL = 26717) For 10 MHz Channel Bandwidth: UL 819.2 MHz (NUL = 26742) For 15 MHz Channel Bandwidth: UL 821.7 MHz (NUL = 26767) |  |  |
| (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  |
| Test Channel Bandwidths |  |  |  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz and 15 MHz |  |  |
| (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 1.4 MHz | N/A for A-MPR testing. |  | QPSK | 1 | 0 |
| 2 | 1.4 MHz |  |  | QPSK | 6 | 0 |
| 3 | 1.4 MHz |  |  | QPSK | 1 | 1 |
| 4 | 1.4 MHz |  |  | QPSK | 5 | 1 |
| 5 | 1.4 MHz |  |  | 16QAM (Note 3) | 6 | 0 |
| 6 (Note 1) | 3 MHz |  |  | QPSK | 4 | 0 |
| 7 (Note 1) | 3 MHz |  |  | QPSK | 10 | 0 |
| 8 | 3 MHz |  |  | QPSK | 4 | 4 |
| 9 | 3 MHz |  |  | QPSK | 10 | 4 |
| 10 (Note 1) | 3 MHz |  |  | 16QAM (Note 3) | 15 | 0 |
| 11 | 5 MHz |  |  | QPSK | 8 | 0 |
| 12 | 5 MHz |  |  | QPSK | 15 | 0 |
| 13 | 5 MHz |  |  | QPSK | 8 | 7 |
| 14 (Note 1) | 5 MHz |  |  | QPSK | 15 | 7 |
| 15 | 5 MHz |  |  | 16QAM (Note 3) | 25 | 0 |
| 16 (Note 2) | 10 MHz |  |  | QPSK | 18 | 0 |
| 17 (Note 2) | 10 MHz |  |  | QPSK | 18 | 16 |
| 18 (Note 2) | 10 MHz |  |  | 16QAM (Note 4, 3) | 50 | 0 |
| 19 (Note 2) | 15 MHz |  |  | QPSK | 30 | 0 |
| 20 (Note 2) | 15 MHz |  |  | QPSK | 30 | 31 |
| 21 (Note 2) | 15 MHz |  |  | 16QAM (Note 4, 3) | 75 | 0 |
| Note 1: Only for UEs of Rel-11 and earlier Note 2: Only for UEs of Rel-12 and later Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 4 : Applies only for UE-Categories ≥2. |  |  |  |  |  |  |

Table 6.2.4.4.1-11: Test Configuration Table (network signalled value "NS_13")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment |  |  |  | NC |  |  |
| (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  |
| Test Frequencies |  |  |  | For 1.4 MHz Channel Bandwidth: UL 819.7 MHz (NUL = 26747) For 3 MHz Channel Bandwidth: UL 820.5 MHz (NUL = 26755) For 5 MHz Channel Bandwidth: UL 821.5 MHz (NUL = 26765) |  |  |
| (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  |
| Test Channel Bandwidths |  |  |  | 1.4 MHz, 3MHz and 5 MHz |  |  |
| (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 1.4 MHz | N/A for A-MPR testing. |  | QPSK | 6 | 0 |
| 2 (Note 1) | 3 MHz |  |  | QPSK | 15 | 0 |
| 3 | 5 MHz |  |  | QPSK | 1 | 0 |
| 4 | 5 MHz |  |  | QPSK | 25 | 0 |
| 5 | 5 MHz |  |  | QPSK | 15 | 0 |
| 6 | 5 MHz |  |  | QPSK | 15 | 7 |
| 7 | 5 MHz |  |  | 16QAM (Note 2) | 25 | 0 |
| Note 1: Only for UEs of Rel-12 and later Note 2: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.2.4.4.1-12: Test Configuration Table (network signalled value "NS_14")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment |  |  |  | NC |  |  |
| (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  |
| Test Frequencies |  |  |  | For 10 MHz Channel Bandwidth: UL 829 MHz (NUL = 26840) For 15 MHz Channel Bandwidth: Mid range |  |  |
| (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  |
| Test Channel Bandwidths |  |  |  | 10 MHz, 15 MHz |  |  |
| (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 10 MHz | N/A for A-MPR testing. |  | QPSK | 1 | 0 |
| 2 | 10 MHz |  |  | QPSK | 25 | 0 |
| 3 | 10 MHz |  |  | QPSK | 50 | 0 |
| 4 | 10 MHz |  |  | QPSK | 25 | 1 |
| 5 (Note 1) | 10 MHz |  |  | 16QAM (Note 2) | 50 | 0 |
| 6 | 15 MHz |  |  | QPSK | 8 | 0 |
| 7 | 15 MHz |  |  | QPSK | 25 | 0 |
| 8 | 15 MHz |  |  | QPSK | 75 | 0 |
| 9 | 15 MHz |  |  | QPSK | 50 | 15 |
| 10 (Note 1) | 15 MHz |  |  | 16QAM (Note 2) | 75 | 0 |
| Note 1: Applies only for UE-Categories ≥2. Note 2: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.2.4.4.1-13: Test Configuration Table (network signalled value "NS_15")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | For 1.4 MHz Channel Bandwidth: High range For 3 MHz Channel Bandwidth: UL 843.5 MHz (NUL = 26985) or High range For 5 MHz Channel Bandwidth: UL 842.5 MHz (NUL = 26975) or High range For 10 MHz Channel Bandwidth: UL 840 MHz (NUL = 26950) or High range For 15 MHz Channel Bandwidth: UL 837.5 MHz (NUL = 26925) or High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 (note 3) |  | 1.4 MHz |  | N/A for A-MPR testing. |  |  | QPSK |  | 4 |  | 0 |  |
| 2 (note 3) |  | 1.4 MHz |  |  |  |  | 16QAM (Note 4) |  | 6 |  | 0 |  |
| 3 (note 3) |  | 3 MHz |  |  |  |  | QPSK |  | 6 |  | 7 |  |
| 4 (note 3) |  | 3 MHz |  |  |  |  | QPSK |  | 12 |  | 1 |  |
| 5 (note 3) |  | 3 MHz |  |  |  |  | 16QAM (Note 4) |  | 15 |  | 0 |  |
| 6 (note 2) |  | 3 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 7 (note 3) |  | 5 MHz |  |  |  |  | QPSK |  | 6 |  | 14 |  |
| 8 (note 3) |  | 5 MHz |  |  |  |  | QPSK |  | 20 |  | 0 |  |
| 9 (note 3) |  | 5 MHz |  |  |  |  | 16QAM (Note 4) |  | 25 |  | 0 |  |
| 10 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 16 |  | 9 |  |
| 11 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 12 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 39 |  |
| 13 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 10 |  |
| 14 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 3 |  | 0 |  |
| 15 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 20 |  | 3 |  |
| 16 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 36 |  | 1 |  |
| 17 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 18 (note 1, 3) |  | 10 MHz |  |  |  |  | 16QAM (Note 4) |  | 50 |  | 0 |  |
| 19 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 20 |  | 25 |  |
| 20 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 45 |  | 0 |  |
| 21 (note 3) |  | 15 MHz |  |  |  |  | QPSK |  | 18 |  | 36 |  |
| 22 (note 3) |  | 15 MHz |  |  |  |  | QPSK |  | 25 |  | 1 |  |
| 23 (note 3) |  | 15 MHz |  |  |  |  | QPSK |  | 54 |  | 0 |  |
| 24 (note 1, 3) |  | 15 MHz |  |  |  |  | 16QAM (Note 4) |  | 75 |  | 0 |  |
| 25 (note 2) |  | 15 MHz |  |  |  |  | QPSK |  | 18 |  | 44 |  |
| 26 (note 2) |  | 15 MHz |  |  |  |  | QPSK |  | 60 |  | 2 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applicable only test frequency < high range. Note 3: Applicable only to high range frequency testing. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4.4.1-14: Test Configuration Table (network signalled value "NS_16")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | For 1.4 MHz Channel Bandwidth: Low range For 3 MHz Channel Bandwidth: Low range, 810 MHz (NUL= 27070) For 5 MHz Channel Bandwidth: Low range, 811 MHz (NUL= 27080), 814.5 MHz (NUL= 27115) For 10 MHz Channel Bandwidth: Low range, 813.5 MHz (NUL= 27105), 817 MHz (NUL= 27140) |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n | RB allocation |  | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 | 1.4 MHz |  | N/A for A-MPR testing. |  |  | QPSK |  | 1 |  | 0 |  |
| 2 | 1.4 MHz |  |  |  |  | QPSK |  | 6 |  | 0 |  |
| 3 | 1.4 MHz |  |  |  |  | 16QAM (Note 4) |  | 6 |  | 0 |  |
| 4 | 3 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 5 |  | 3 MHz |  |  |  |  | QPSK |  | 12 |  | 1 |  |
| 6 |  | 3 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 7 |  | 3 MHz |  |  |  |  | 16QAM (Note 4) |  | 15 |  | 0 |  |
| 8 |  | 5 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 9 |  | 5 MHz |  |  |  |  | QPSK |  | 12 |  | 2 |  |
| 10 |  | 5 MHz |  |  |  |  | QPSK |  | 18 |  | 2 |  |
| 11 |  | 5 MHz |  |  |  |  | QPSK |  | 20 |  | 0 |  |
| 12 |  | 5 MHz |  |  |  |  | QPSK |  | 20 |  | 2 |  |
| 13 |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 14 |  | 5 MHz |  |  |  |  | 16QAM (Note 4) |  | 25 |  | 0 |  |
| 15 |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 16 (Note 2) | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 10 |  |
| 17 (Note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 20 |  | 0 |  |
| 18 (Note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 27 |  | 15 |  |
| 19 (Note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 32 |  | 15 |  |
| 20 |  | 10 MHz |  |  |  |  | QPSK |  | 32 |  | 0 |  |
| 21 |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 22 (Note 1) |  | 10 MHz |  |  |  |  | 16QAM (Note 4) |  | 50 |  | 0 |  |
| 23 (Note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 40 |  | 0 |  |
| 24 (Note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 40 |  | 1 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applies only for 10 MHz channel for Low Range, and 813.5 MHz Note 3: Applies only for 10 MHz channel for 817 MHz range Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4.4.1-15: Test Configuration Table (network signalled value "NS_17")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 5MHz |  |  | 16QAM (Note 4) | 25 (Note 3) |
| 5 | 10MHz |  |  | QPSK | 1 |
| 6 | 10MHz |  |  | QPSK | 12 |
| 7 | 10MHz |  |  | QPSK | 50 |
| 8 | 10MHz |  |  | 16QAM (Note 4) | 50 (Note 3) |
| Note 1: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-16: Test Configuration Table (network signalled value "NS_18")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 5MHz |  |  | 16QAM (Note 4) | 25 (Note 3) |
| 5 | 10MHz |  |  | QPSK | 1 |
| 6 | 10MHz |  |  | QPSK | 12 |
| 7 | 10MHz |  |  | QPSK | 50 |
| 8 | 10MHz |  |  | 16QAM (Note 4) | 50 (Note 3) |
| 9 | 15MHz |  |  | QPSK | 1 |
| 10 | 15MHz |  |  | QPSK | 16 |
| 11 | 15MHz |  |  | QPSK | 75 |
| 12 | 15MHz |  |  | 16QAM (Note 4) | 75 (Note 3) |
| 13 | 20MHz |  |  | QPSK | 1 |
| 14 | 20MHz |  |  | QPSK | 18 |
| 15 | 20MHz |  |  | QPSK | 100 |
| 16 | 20MHz |  |  | 16QAM (Note 4) | 100 (Note 3) |
| Note 1: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-17: Test Configuration Table (network signalled value "NS_19")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A |  | QPSK | 15 |
| 2 | 3MHz |  |  | 16QAM (Note 4) | 15 (Note 3) |
| 3 | 10MHz |  |  | QPSK | 40 |
| 4 | 10MHz |  |  | 16QAM (Note 4) | 40 (Note 3) |
| 5 | 15MHz |  |  | QPSK | 1 |
| 6 | 15MHz |  |  | QPSK | 18 |
| 7 | 15MHz |  |  | QPSK | 36 |
| 8 | 15MHz |  |  | QPSK | 45 |
| 9 | 15MHz |  |  | 16QAM (Note 4) | 45 (Note 3) |
| 10 | 20MHz |  |  | QPSK | 1 |
| 11 | 20MHz |  |  | QPSK | 40 |
| 12 | 20MHz |  |  | QPSK | 45 |
| 13 | 20MHz |  |  | QPSK | 50 |
| 14 | 20MHz |  |  | 16QAM (Note 4) | 50 (Note 3) |
| Note 1: The 1 RB allocation shall be tested at both RB #0 and RB #max. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-18: Test Configuration Table (network signalled value "NS_20")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2007.5 MHz (NUL = 25575) c. 2012.5 MHz (NUL = 25625) d. 2017.5 MHz (NUL = 25675) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_20 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 8 |
| 2 | 5MHz |  |  | QPSK | 15 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 5MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 6 | 10MHz |  |  | QPSK | 8 |
| 7 | 10MHz |  |  | QPSK | 12 |
| 8 | 10MHz |  |  | QPSK | 50 |
| 9 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 10 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 11 | 15MHz |  |  | QPSK | 6 |
| 12 | 15MHz |  |  | QPSK | 25 |
| 13 | 15MHz |  |  | QPSK | 36 |
| 14 | 15MHz |  |  | QPSK | 75 |
| 15 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 16 | 15MHz |  |  | 16QAM (Note 4) | 36 |
| 17 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 18 | 20MHz |  |  | QPSK | 8 |
| 19 | 20MHz |  |  | QPSK | 18 |
| 20 | 20MHz |  |  | QPSK | 25 |
| 21 | 20MHz |  |  | QPSK | 75 |
| 22 | 20MHz |  |  | QPSK | 100 |
| 23 | 20MHz |  |  | 16QAM (Note 4) | 18 |
| 24 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 25 | 20MHz |  |  | 16QAM (Note 4) | 75 |
| 26 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-19: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | Low range or High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 5 MHz, 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 2 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 3 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 4 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 49 |  |
| 5 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 6 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 3 |  | 47 |  |
| 7 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 8 (note 1, 3) |  | 10 MHz |  |  |  |  | 16QAM (Note 4) |  | 50 |  | 0 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applicable only to low range frequency testing. Note 3: Applicable only to high range frequency testing. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4.4.1-20: Test Configuration Table (network signalled value "NS_22")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_22 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 | 5MHz |  |  | QPSK | 1 | 24 |
| 4 | 5MHz |  |  | 16QAM | 1 | 0 |
| 5 | 5MHz |  |  | 16QAM | 1 | 24 |
| 6 | 10MHz |  |  | QPSK | 50 | 0 |
| 7 | 10MHz |  |  | QPSK | 36 | 0 |
| 8 | 10MHz |  |  | QPSK | 1 | 0 |
| 9 | 10MHz |  |  | QPSK | 1 | 49 |
| 10 | 10MHz |  |  | 16QAM | 1 | 0 |
| 11 | 10MHz |  |  | 16QAM | 1 | 49 |
| 12 | 15MHz |  |  | QPSK | 75 | 0 |
| 13 | 15MHz |  |  | QPSK | 50 | 0 |
| 14 | 15MHz |  |  | QPSK | 1 | 0 |
| 15 | 15MHz |  |  | QPSK | 1 | 74 |
| 16 | 15MHz |  |  | 16QAM | 1 | 0 |
| 17 | 15MHz |  |  | 16QAM | 1 | 74 |
| 18 | 20MHz |  |  | QPSK | 100 | 0 |
| 19 | 20MHz |  |  | QPSK | 64 | 0 |
| 20 | 20MHz |  |  | QPSK | 1 | 0 |
| 21 | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | 20MHz |  |  | 16QAM | 1 | 0 |
| 23 | 20MHz |  |  | 16QAM | 1 | 99 |
| Note 1: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.2.4.4.1-21: Test Configuration Table (network signalled value "NS_23")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_23 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | 5MHz |  |  | 16QAM | 1 | 0 |
| 5 | 5MHz |  |  | 16QAM | 1 | 24 |
| 6 | 10MHz |  |  | QPSK | 50 | 0 |
| 7 | 15MHz |  |  | QPSK | 75 | 0 |
| 8 | 20MHz |  |  | QPSK | 100 | 0 |
| Note 1: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.2.4.4.1-22: Test Configuration Table (network signalled value "NS_24")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 1985 MHz (NUL = 131722) - 2007.5 MHz (NUL = high range) 10 MHz CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2005 MHz (NUL = high range) 15 MHZ CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2002.5 MHz (NUL = high range) 20 MHz CH BW: - 2000 MHz (NUL = high range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_24 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 2007.5 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 |  | 2007.5 | 5MHz |  |  | QPSK | 8 | 17 |
| 3 |  | 2007.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 4 (Notes 2,3) |  | 2007.5 | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 |  | 1985 | 5MHz |  |  | QPSK | 8 | 17 |
| 6 |  | 1980 | 10MHz |  |  | QPSK | 12 | 1 |
| 7 (Notes 2,3) |  | 1980 | 10MHz |  |  | 16QAM | 50 | 0 |
| 8 |  | 1980 | 10MHz |  |  | QPSK | 48 | 2 |
| 9 |  | 1980 | 10MHz |  |  | QPSK | 25 | 25 |
| 10 |  | 1980 | 10MHz |  |  | QPSK | 1 | 49 |
| 11 |  | 1980 | 10MHz |  |  | QPSK | 12 | 38 |
| 12 (Notes 2,3) |  | 1980 | 10MHz |  |  | 16QAM | 12 | 38 |
| 13 |  | 1980 | 10MHz |  |  | QPSK | 3 | 47 |
| 14 |  | 1980 | 10MHz |  |  | QPSK | 8 | 1 |
| 15 |  | 1990 | 10MHz |  |  | QPSK | 1 | 0 |
| 16 |  | 1990 | 10MHz |  |  | QPSK | 12 | 38 |
| 17 (Notes 2,3) |  | 1990 | 10MHz |  |  | 16QAM | 50 | 0 |
| 18 |  | 2005 | 10MHz |  |  | QPSK | 12 | 38 |
| 19 |  | 2005 | 10MHz |  |  | QPSK | 50 | 0 |
| 20 (Notes 2,3) |  | 2005 | 10MHz |  |  | 16QAM | 50 | 0 |
| 21 |  | 1980 | 15MHz |  |  | QPSK | 1 | 11 |
| 22 |  | 1980 | 15MHz |  |  | QPSK | 16 | 11 |
| 23 |  | 1980 | 15MHz |  |  | QPSK | 45 | 11 |
| 24 (Notes 2,3) |  | 1980 | 15MHz |  |  | 16QAM | 50 | 0 |
| 25 |  | 1980 | 15MHz |  |  | QPSK | 16 | 28 |
| 26 |  | 1980 | 15MHz |  |  | QPSK | 16 | 59 |
| 27 |  | 1980 | 15MHz |  |  | QPSK | 60 | 15 |
| 28 |  | 1980 | 15MHz |  |  | QPSK | 30 | 12 |
| 29 |  | 2002.5 | 15MHz |  |  | QPSK | 1 | 0 |
| 30 |  | 2002.5 | 15MHz |  |  | QPSK | 16 | 59 |
| 31 (Notes 2,3) |  | 2002.5 | 15MHz |  |  | 16QAM | 16 | 59 |
| 32 (Notes 2,3) |  | 2002.5 | 15MHz |  |  | 16QAM | 75 | 0 |
| 33 |  | 2000 | 20MHz |  |  | QPSK | 1 | 0 |
| 34 |  | 2000 | 20MHz |  |  | QPSK | 18 | 82 |
| 35 (Notes 2,3) |  | 2000 | 20MHz |  |  | 16QAM | 18 | 82 |
| 36 (Notes 2,3) |  | 2000 | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |

Table 6.2.4.4.1-23: Test Configuration Table (network signalled value "NS_25")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 2007.5 MHz (NUL = high range) 10 MHz CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2005 MHz (NUL = high range) 15 MHZ CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2002.5 MHz (NUL = high range) 20 MHz CH BW: - 2000 MHz (NUL = high range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_25 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 2007.5 | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Notes 2,3) |  | 2007.5 | 5MHz |  |  | 16QAM | 25 | 0 |
| 3 |  | 2007.5 | 5MHz |  |  | QPSK | 8 | 9 |
| 4 |  | 2007.5 | 5MHz |  |  | QPSK | 1 | 24 |
| 5 |  | 2007.5 | 5MHz |  |  | QPSK | 8 | 17 |
| 6 (Notes 2,3) |  | 2007.5 | 5MHz |  |  | 16QAM | 8 | 17 |
| 7 |  | 1980 | 10MHz |  |  | QPSK | 12 | 1 |
| 8 (Notes 2,3) |  | 1980 | 10MHz |  |  | 16QAM | 50 | 0 |
| 9 (Notes 2,3) |  | 1980 | 10MHz |  |  | 16QAM | 48 | 2 |
| 10 |  | 1980 | 10MHz |  |  | QPSK | 25 | 25 |
| 11 |  | 1980 | 10MHz |  |  | QPSK | 1 | 49 |
| 12 |  | 1980 | 10MHz |  |  | QPSK | 12 | 38 |
| 13 |  | 1990 | 10MHz |  |  | QPSK | 12 | 0 |
| 14 |  | 1990 | 10MHz |  |  | QPSK | 25 | 0 |
| 15 |  | 1990 | 10MHz |  |  | QPSK | 50 | 0 |
| 16 (Notes 2,3) |  | 1990 | 10MHz |  |  | 16QAM | 25 | 0 |
| 17 |  | 1990 | 10MHz |  |  | QPSK | 48 | 2 |
| 18 |  | 1990 | 10MHz |  |  | QPSK | 30 | 19 |
| 19 (Notes 2,3) |  | 1990 | 10MHz |  |  | 16QAM | 30 | 19 |
| 20 |  | 1990 | 10MHz |  |  | QPSK | 12 | 31 |
| 21 |  | 2005 | 10MHz |  |  | QPSK | 1 | 0 |
| 22 |  | 2005 | 10MHz |  |  | QPSK | 12 | 6 |
| 23 |  | 2005 | 10MHz |  |  | QPSK | 50 | 0 |
| 24 (Notes 2,3) |  | 2005 | 10MHz |  |  | 16QAM | 50 | 0 |
| 25 |  | 2005 | 10MHz |  |  | QPSK | 40 | 10 |
| 26 (Notes 2,3) |  | 2005 | 10MHz |  |  | 16QAM | 40 | 10 |
| 27 |  | 2005 | 10MHz |  |  | QPSK | 32 | 18 |
| 28 (Notes 2,3) |  | 2005 | 10MHz |  |  | 16QAM | 32 | 18 |
| 29 |  | 2005 | 10MHz |  |  | QPSK | 12 | 23 |
| 30 |  | 1980 | 15MHz |  |  | QPSK | 16 | 4 |
| 31 |  | 1980 | 15MHz |  |  | QPSK | 75 | 0 |
| 32 |  | 1980 | 15MHz |  |  | QPSK | 60 | 15 |
| 33 (Notes 2,3) |  | 1980 | 15MHz |  |  | 16QAM | 40 | 35 |
| 34 |  | 1980 | 15MHz |  |  | QPSK | 1 | 74 |
| 35 |  | 1980 | 15MHz |  |  | QPSK | 12 | 63 |
| 36 |  | 1980 | 15MHz |  |  | QPSK | 16 | 55 |
| 37 |  | 2002.5 | 15MHz |  |  | QPSK | 1 | 0 |
| 38 |  | 2002.5 | 15MHz |  |  | QPSK | 16 | 0 |
| 39 (Notes 2,3) |  | 2002.5 | 15MHz |  |  | 16QAM | 16 | 0 |
| 40 (Notes 2,3) |  | 2002.5 | 15MHz |  |  | 16QAM | 1 | 0 |
| 41 |  | 1980 | 20MHz |  |  | QPSK | 1 | 0 |
| 42 |  | 1980 | 20MHz |  |  | QPSK | 18 | 0 |
| 43 (Notes 2,3) |  | 1980 | 20MHz |  |  | 16QAM | 100 | 0 |
| 44 |  | 1980 | 20MHz |  |  | QPSK | 64 | 36 |
| 45 |  | 1980 | 20MHz |  |  | QPSK | 18 | 72 |
| 46 (Notes 2,3) |  | 1980 | 20MHz |  |  | 16QAM | 54 | 46 |
| 47 |  | 1980 | 20MHz |  |  | QPSK | 18 | 55 |
| 48 |  | 2000 | 20MHz |  |  | QPSK | 1 | 0 |
| 49 |  | 2000 | 20MHz |  |  | QPSK | 18 | 72 |
| 50 (Notes 2,3) |  | 2000 | 20MHz |  |  | 16QAM | 18 | 72 |
| 51 (Notes 2,3) |  | 2000 | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |

Table 6.2.4.4.1-24: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | Low/Mid/High | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | Low/Mid/High | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Low/Mid | 10MHz |  |  | QPSK | 1 | 0 |
| 6 | High | 10MHz |  |  | QPSK | 1 | 49 |
| 7 | Low/Mid/High | 10MHz |  |  | QPSK | 50 | 0 |
| 8 (Note 2) | Low/Mid/High | 10MHz |  |  | 16QAM | 50 | 0 |
| 9 | Low/Mid | 15MHz |  |  | QPSK | 1 | 0 |
| 10 | Low | 15MHz |  |  | QPSK | 1 | 7 |
| 11 | Low/Mid | 15MHz |  |  | QPSK | 16 | 0 |
| 12 | High | 15MHz |  |  | QPSK | 1 | 74 |
| 13 | High | 15MHz |  |  | QPSK | 1 | 67 |
| 14 | High | 15MHz |  |  | QPSK | 16 | 0 |
| 15 | Low/Mid/High | 15MHz |  |  | QPSK | 75 | 0 |
| 16 (Note 2) | Low/Mid/High | 15MHz |  |  | 16QAM | 75 | 0 |
| 17 | Low/Mid | 20MHz |  |  | QPSK | 1 | 0 |
| 18 | Low | 20MHz |  |  | QPSK | 1 | 13 |
| 19 | Low | 20MHz |  |  | QPSK | 1 | 16 |
| 20 | Low/Mid | 20MHz |  |  | QPSK | 24 | 0 |
| 21 | High | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | High | 20MHz |  |  | QPSK | 1 | 86 |
| 23 | High | 20MHz |  |  | QPSK | 1 | 83 |
| 24 | High | 20MHz |  |  | QPSK | 24 | 0 |
| 25 | Low/Mid/High | 20MHz |  |  | QPSK | 100 | 0 |
| 26 (Note 2) | Low/Mid/High | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.2.4.4.1-25: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_35 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 25 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | 16QAM (Note 4) | 8 |
| 4 | 10MHz |  |  | QPSK | 50 |
| 5 | 10MHz |  |  | QPSK | 12 |
| 6 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 7 | 15MHz |  |  | QPSK | 75 |
| 8 | 15MHz |  |  | QPSK | 16 |
| 9 | 15MHz |  |  | 16QAM (Note 4) | 16 |
| 10 | 20MHz |  |  | QPSK | 100 |
| 11 | 20MHz |  |  | QPSK | 18 |
| 12 | 20MHz |  |  | 16QAM (Note 4) | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.2.4.4.1-26: Test Configuration Table (network signalled value "NS_36")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 700.5 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 |  | 700.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 |  | 700.5 | 5MHz |  |  | QPSK | 20 | 1 |
| 4 |  | 700.5 | 5MHz |  |  | QPSK | 24 | 1 |
| 5 |  | 700.5 | 5MHz |  |  | QPSK | 12 | 0 |
| 6 (Notes 2,3) |  | 700.5 | 5MHz |  |  | 16QAM | 12 | 0 |
| 7 |  | 705.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 8 (Notes 2,3) |  | 705.5 | 5MHz |  |  | 16QAM | 25 | 0 |
| 9 |  | 703 | 10MHz |  |  | QPSK | 50 | 0 |
| 10 |  | 703 | 10MHz |  |  | QPSK | 1 | 0 |
| 11 |  | 703 | 10MHz |  |  | QPSK | 40 | 9 |
| 12 |  | 703 | 10MHz |  |  | QPSK | 36 | 13 |
| 13 |  | 703 | 10MHz |  |  | QPSK | 10 | 40 |
| 14 |  | 703 | 10MHz |  |  | QPSK | 6 | 44 |
| 15 |  | 708 | 10MHz |  |  | QPSK | 50 | 0 |
| 16 (Notes 2,3) |  | 708 | 10MHz |  |  | 16QAM | 50 | 0 |
| 17 |  | 705.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 18 |  | 705.5 | 15MHz |  |  | QPSK | 50 | 17 |
| 19 |  | 705.5 | 15MHz |  |  | QPSK | 50 | 24 |
| 20 |  | 705.5 | 15MHz |  |  | QPSK | 1 | 57 |
| 21 |  | 705.5 | 15MHz |  |  | QPSK | 10 | 17 |
| 22 |  | 705.5 | 15MHz |  |  | QPSK | 18 | 24 |
| 23 (Notes 2,3) |  | 705.5 | 15MHz |  |  | 16QAM | 18 | 24 |
| 24 |  | 710.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 25 |  | 710.5 | 15MHz |  |  | 16QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of section 6.2.4.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.4.4.1-1 to Table 6.2.4.4.1-26.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4.4.3.

##### 6.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.4.4.1-1 to Table 6.2.4.4.1-26. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

##### 6.2.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, with the following exceptions for each network signalled value.

6.2.4.4.3.1	Message contents exceptions (network signalled value "NS_03")

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.2.4.4.3.2	Message contents exceptions (network signalled value "NS_04")

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_04) |  |  |

6.2.4.4.3.3	Message contents exceptions (network signalled value "NS_05")

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.3-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_05) |  |  |

6.2.4.4.3.4	Message contents exceptions (network signalled value "NS_06")

1.	Information element additionalSpectrumEmission is set to NS_06. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_06"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 6 (NS_06) |  |  |

6.2.4.4.3.5	Message contents exceptions (network signalled value "NS_07")

1.	Information element additionalSpectrumEmission is set to NS_07. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_07"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 7 (NS_07) |  |  |

6.2.4.4.3.6	Message contents exceptions (network signalled value "NS_08")

1.	Information element additionalSpectrumEmission is set to NS_08. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_08"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 8 (NS_08) |  |  |

6.2.4.4.3.7	Message contents exceptions (network signalled value "NS_09")

1.	Information element additionalSpectrumEmission is set to NS_09. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_09"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 9 (NS_09) |  |  |

6.2.4.4.3.8	Message contents exceptions (network signalled value "NS_10")

Void

6.2.4.4.3.9	Message contents exceptions (network signalled value "NS_11")

1.	Information element additionalSpectrumEmission is set to NS_11. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.9-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_11"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 11 (NS_11) |  |  |

6.2.4.4.3.10	Message contents exceptions (network signalled value "NS_12")

1.	Information element additionalSpectrumEmission is set to NS_12. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.10-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_12"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 12 (NS_12) |  |  |

6.2.4.4.3.11	Message contents exceptions (network signalled value "NS_13")

1.	Information element additionalSpectrumEmission is set to NS_13. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.11-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_13"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 13 (NS_13) |  |  |

6.2.4.4.3.12	Message contents exceptions (network signalled value "NS_14")

1.	Information element additionalSpectrumEmission is set to NS_14. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.12-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_14"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 14 (NS_14) |  |  |

6.2.4.4.3.13	Message contents exceptions (network signalled value "NS_15")

1. Information element additionalSpectrumEmission is set to NS_15. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.13-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_15"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 15 (NS_15) |  |  |

6.2.4.4.3.14	Message contents exceptions (network signalled value "NS_16")

1. Information element additionalSpectrumEmission is set to NS_16. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.14-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_16"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 16 (NS_16) |  |  |

6.2.4.4.3.15	Message contents exceptions (network signalled value "NS_17")

1. Information element additionalSpectrumEmission is set to NS_17. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.15-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_17"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 17 (NS_17) |  |  |

6.2.4.4.3.16	Message contents exceptions (network signalled value "NS_18")

1. Information element additionalSpectrumEmission is set to NS_18. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.16-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_18"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 18 (NS_18) |  |  |

6.2.4.4.3.17	Message contents exceptions (network signalled value "NS_19")

1. Information element additionalSpectrumEmission is set to NS_19. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.17-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_19"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 19 (NS_19) |  |  |

6.2.4.4.3.18	Message contents exceptions (network signalled value "NS_20")

1. Information element additionalSpectrumEmission is set to NS_20. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.18-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_20"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 20 (NS_20) |  |  |

6.2.4.4.3.19	Message contents exceptions (network signalled value "NS_21")

1. Information element additionalSpectrumEmission is set to NS_21. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.19-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

6.2.4.4.3.20	Message contents exceptions (network signalled value "NS_22")

1. Information element additionalSpectrumEmission is set to NS_21. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.20-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_22"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 22 (NS_22) |  |  |

6.2.4.4.3.21	Message contents exceptions (network signalled value "NS_23")

1. Information element additionalSpectrumEmission is set to NS_21. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.21-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_23"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 23 (NS_23) |  |  |

6.2.4.4.3.22	Message contents exceptions (network signalled value "NS_24")

1. Information element additionalSpectrumEmission is set to NS_24. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.22-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 24 (NS_24) |  |  |

6.2.4.4.3.23	Message contents exceptions (network signalled value "NS_25")

1. Information element additionalSpectrumEmission is set to NS_25. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.23-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_25"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 25 (NS_25) |  |  |

6.2.4.4.3.24 Reserved

6.2.4.4.3.25	Message contents exceptions (network signalled value "NS_27")

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 27 (NS_27) |  |  |

6.2.4.4.3.26	Message contents exceptions (network signalled value "NS_35")

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.26-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_35"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 32 |  |  |
| additionalSpectrumEmission-v10l0 | 35 (NS_35) |  |  |

6.2.4.4.3.27	Message contents exceptions (network signalled value "NS_36")

1.	Information element additionalSpectrumEmission is set to NS_36. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.27-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_36"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 32 |  |  |
| additionalSpectrumEmission-v10l0 | 36 (NS_36) |  |  |

6.2.4.4.3.28	Message contents exceptions (network signalled value "NS_38")

1.	Information element additionalSpectrumEmission is set to NS_38. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.28-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_38"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 32 |  |  |
| additionalSpectrumEmission-v10l0 | 38 (NS_38) |  |  |

6.2.4.4.3.29	Message contents exceptions (network signalled value "NS_39")

1.	Information element additionalSpectrumEmission is set to NS_39. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.29-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_39"

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 32 |  |  |
| additionalSpectrumEmission-v10l0 | 39 (NS_39) |  |  |

#### 6.2.4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4.5-1 to Table 6.2.4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

Table 6.2.4.5-1: UE Power Class test requirements (network signalled value "NS_03")
(for Bands 4, 10, 23, 35, 36, 66, and 70)

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 2 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 3 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 4 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 5 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 6 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 7 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 8 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 9 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 10 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 11 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 12 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 13 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 14 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 15 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 16 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 17 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 18 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 19 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 20 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 21 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 22 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 23 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 24 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 25 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 26 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 27 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-2: UE Power Class test requirements (network signalled value "NS_03")
(for Bands 2 and 25)

| Configuration ID | EUTRA band | Test Freq. | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -3.7 |
| 1 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -5.7 |
| 2 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -2.7 |
| 2 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -4.2 |
| 3 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -3.7 |
| 3 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -5.7 |
| 4 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -4.7 |
| 4 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -7.7 |
| 5 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -2.7 |
| 5 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -4.2 |
| 6 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -6.2 |
| 6 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -9.2 |
| 7 | 2, 25 | Mid |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 2, 25 | Low, High |  |  |  |  | 23 | +2.7 / -5.7 |
| 8 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -4.7 |
| 9 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -3.7 |
| 9 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -5.7 |
| 10 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -2.7 |
| 10 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -4.2 |
| 11 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -6.2 |
| 12 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -4.7 |
| 12 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -7.7 |
| 13 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -4.7 |
| 14 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -3.7 |
| 14 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -5.7 |
| 15 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -2.7 |
| 15 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -4.2 |
| 16 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -6.2 |
| 17 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -4.7 |
| 17 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -7.7 |
| 18 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -4.7 |
| 19 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -3.7 |
| 19 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -5.7 |
| 20 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -2.7 |
| 20 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -4.2 |
| 21 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -6.2 |
| 22 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -4.7 |
| 22 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -7.7 |
| 23 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -4.7 |
| 24 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -3.7 |
| 25 | 2, 25 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -2.7 |
| 25 | 2, 25 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -4.2 |
| 26 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -6.2 |
| 27 | 2, 25 | All |  |  |  |  | 23 | +2.7 / -4.7 |

Table 6.2.4.5-3: UE Power Class test requirements (network signalled value "NS_04")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 2 (RBstart=0; Low Range) | 0 | 1 | 1.5 | 20.5 | 2.5 | 25.7 | 17.3 |
| 2 (RBstart=0; Mid, High Range) | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 2 (RBstart=17; Low, Mid Range) | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 2 (RBstart=17; High Range) | 0 | 1 | 1.5 | 20.5 | 2.5 | 25.7 | 17.3 |
| 3 (RBstart=0; Low Range) | 0 | 0 | 1.5 | 21.5 | 2.0 | 25.7 | 18.8 |
| 3 (RBstart=0; Mid, High Range) | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 3 (RBstart=19; Low, Mid Range) | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 3 (RBstart=19; High Range) | 0 | 0 | 1.5 | 21.5 | 2.0 | 25.7 | 18.8 |
| 4 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 5 (RBstart=0; Low Range) | 1 | 1 | 1.5 | 19.5 | 3.5 | 25.7 | 15.3 |
| 5 (RBstart=0; Mid, High Range) | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 5 (RBstart=17; Low, Mid Range) | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 5 (RBstart=17; High Range) | 1 | 1 | 1.5 | 19.5 | 3.5 | 25.7 | 15.3 |
| 6 (Low Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 6 (Mid, High Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 7 (Low Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 7 (Mid, High Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 8 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 9 | 2 | 3 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 10 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 11 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 12 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 13 (Low, Mid Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 13 (High Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 14 (Low, Mid Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 14 (High Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 15 (Low Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 15 (Mid, High Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 16 (Low, Mid Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 16 (High Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 17 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 18 | 2 | 3 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 19 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 20 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 21 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 22 | 1 | 3 | 0 | 19 | 2.0 | 25.7 | 16.3 |
| 23 (Low, Mid Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 23 (High Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 24 (Low Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 24 (Mid, High Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 25 (Low Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 25 (Mid, High Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 26 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 27 | 2 | 3 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 28 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 29 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 30 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 31 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 32 (Low, Mid Range) | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 32 (High Range) | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 33 | 0 | 2 | 1.5 | 19.5 | 3.5 | 25.7 | 15.3 |
| 34 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 35 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 36 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 37 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 38 | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 39 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 40 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 41 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 42 (Low Range) | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 42 (Mid, High Range, 2504.1 MHz) | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 43 (Low Range) | 2 | 2 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 43 (Mid, High Range, 2504.1 MHz ) | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 44 | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 45 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 46 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 47 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 48 (Low Range) | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 48 (Mid, High Range, 2510.9 MHz) | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 49 (Low Range) | 2 | 3 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 49 (Mid, High Range, 2510.9 MHz) | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 50 | 0 | 3 | 1.5 | 18.5 | 4.0 | 25.7 | 13.8 |
| 51 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 52 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 53 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 54 (Low Range) | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 54 (Mid, High Range, 2517.6 MHz) | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 55 (Low Range) | 2 | 3 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 55 (Mid, High Range, 2517.6 MHz) | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-4: UE Power Class test requirements (network signalled value "NS_05")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 2 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 3 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 4 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 5 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 6 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 7 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 8 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 9 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 10 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 11 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 12 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 13 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 14 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 15 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 16 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 17 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 18 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 19 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 20 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 21 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 22 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 23 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 24 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 25 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 26 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 27 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 28 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 29 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 30 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 31 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 32 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 33 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 34 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 35 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 36 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 37 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 38 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 39 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 40 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 41 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 42 | 0 | 11 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 43 | 1 | 11 | 0 | 11 | 6.0 | 25.7 | 4.3 |
| 44 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 45 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 46 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 47 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 48 | 0 | 5 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 49 | 0 | 5 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 50 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 51 | 0 | 11 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 52 | 1 | 11 | 0 | 11 | 6.0 | 25.7 | 4.3 |
| 53 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 54 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 55 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 56 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 57 | 1 | 10 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 58 | 0 | 5 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 59 | 0 | 5 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 60 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 61 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 62 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 63 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 64 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 65 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 66 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 67 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 68 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 69 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 70 | 2 | 0 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 71 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 72 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 73 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 74 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 75 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 76 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 77 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 78 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |

Table 6.2.4.5-5: UE Power Class test requirements (network signalled value "NS_06")
(for Bands 13, 14, and 17)

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 13,14,17 |  |  |  |  | 23 | +2.7 / -3.7 |
| 2 | 13,14,17 |  |  |  |  | 23 | +2.7 / -2.7 |
| 3 | 13,14,17 |  |  |  |  | 23 | +2.7 / -2.7 |
| 4 | 13,14,17 |  |  |  |  | 23 | +2.7 / -3.7 |
| 5 | 13,14,17 |  |  |  |  | 23 | +2.7 / -2.7 |
| 6 | 13,14,17 |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 13,14,17 |  |  |  |  | 23 | +2.7 / -3.7 |
| 8 | 13,14,17 |  |  |  |  | 23 | +2.7 / -2.7 |
| 9 | 13,14,17 |  |  |  |  | 23 | +2.7 / -3.7 |

Table 6.2.4.5-6: UE Power Class test requirements (network signalled value "NS_06")
(for Band 12)

| Configuration ID | EUTRA band | Test Freq. | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12 | Mid |  |  |  |  | 23 | +2.7 / -3.7 |
| 1 | 12 | Low, High |  |  |  |  | 23 | +2.7 / -5.7 |
| 2 | 12 | Mid |  |  |  |  | 23 | +2.7 / -2.7 |
| 2 | 12 | Low, High |  |  |  |  | 23 | +2.7 / -4.2 |
| 3 | 12 | Mid |  |  |  |  | 23 | +2.7 / -2.7 |
| 3 | 12 | Low, High |  |  |  |  | 23 | +2.7 / -4.2 |
| 4 | 12 | All |  |  |  |  | 23 | +2.7 / -3.7 |
| 5 | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -2.7 |
| 5 | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -4.2 |
| 6 | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -3.7 |
| 6 | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -5.7 |
| 7 | 12 | All |  |  |  |  | 23 | +2.7 / -3.7 |
| 8 | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -2.7 |
| 8 | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -4.2 |
| 9 | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  |  | 23 | +2.7 / -3.7 |
| 9 | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  |  | 23 | +2.7 / -5.7 |

Table 6.2.4.5-7: UE Power Class test requirements (network signalled value "NS_07")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 13 |  |  |  |  | 23 | +2.7 / -18.7 |
| 2 | 13 |  |  |  |  | 23 | +2.7 / -13.7 |
| 3 | 13 |  |  |  |  | 23 | +2.7 / -2.7 |
| 4 | 13 |  |  |  |  | 23 | +2.7 / -19.7 |
| 5 | 13 |  |  |  |  | 23 | +2.7 / -18.7 |
| 6 | 13 |  |  |  |  | 23 | +2.7 / -20.7 |
| 7 | 13 |  |  |  |  | 23 | +2.7 / -3.7 |
| 8 | 13 |  |  |  |  | 23 | +2.7 / -2.7 |
| 9 | 13 |  |  |  |  | 23 | +2.7 / -4.7 |
| 10 | 13 |  |  |  |  | 23 | +2.7 / -12.7 |
| 11 | 13 |  |  |  |  | 23 | +2.7 / -13.7 |
| 12 | 13 |  |  |  |  | 23 | +2.7 / -2.7 |
| 13 | 13 |  |  |  |  | 23 | +2.7 / -6.2 |
| 14 | 13 |  |  |  |  | 23 | +2.7 / -19.7 |
| 15 | 13 |  |  |  |  | 23 | +2.7 / -18.7 |
| 16 | 13 |  |  |  |  | 23 | +2.7 / -20.7 |

Table 6.2.4.5-8: UE Power Class test requirements (network signalled value "NS_08")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 19 |  |  |  |  | 23 | +2.7 / -2.7 |
| 2 | 19 |  |  |  |  | 23 | +2.7 / -2.7 |
| 3 | 19 |  |  |  |  | 23 | +2.7 / -3.7 |
| 4 | 19 |  |  |  |  | 23 | +2.7 / -2.7 |
| 5 | 19 |  |  |  |  | 23 | +2.7 / -2.7 |
| 6 | 19 |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 19 |  |  |  |  | 23 | +2.7 / -8.2 |
| 8 | 19 |  |  |  |  | 23 | +2.7 / -9.7 |
| 9 | 19 |  |  |  |  | 23 | +2.7 / -2.7 |
| 10 | 19 |  |  |  |  | 23 | +2.7 / -2.7 |
| 11 | 19 |  |  |  |  | 23 | +2.7 / -3.7 |
| 12 | 19 |  |  |  |  | 23 | +2.7 / -8.2 |
| 13 | 19 |  |  |  |  | 23 | +2.7 / -9.7 |

Table 6.2.4.5-9: UE Power Class test requirements (network signalled value "NS_09")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 21 |  |  |  |  | 23 | +2.7 / -2.7 |
| 2 | 21 |  |  |  |  | 23 | +2.7 / -2.7 |
| 3 | 21 |  |  |  |  | 23 | +2.7 / -3.7 |
| 4 | 21 |  |  |  |  | 23 | +2.7 / -2.7 |
| 5 | 21 |  |  |  |  | 23 | +2.7 / -2.7 |
| 6 | 21 |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 21 |  |  |  |  | 23 | +2.7 / -4.7 |
| 8 | 21 |  |  |  |  | 23 | +2.7 / -6.2 |
| 9 | 21 |  |  |  |  | 23 | +2.7 / -2.7 |
| 19 | 21 |  |  |  |  | 23 | +2.7 / -2.7 |
| 11 | 21 |  |  |  |  | 23 | +2.7 / -3.7 |
| 12 | 21 |  |  |  |  | 23 | +2.7 / -4.7 |
| 13 | 21 |  |  |  |  | 23 | +2.7 / -6.2 |
| 14 | 21 |  |  |  |  | 23 | +2.7 / -8.2 |

Table 6.2.4.5-10: UE Power Class test requirements (network signalled value "NS_10")

Void, not tested

Table 6.2.4.5-11: UE Power Class test requirements (network signalled value "NS_11 for Band 23")

| Configuration ID | EUTRA Band | Centre Frequency | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | 23 | 2001.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 1b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -4.7 |
| 2a | 23 | 2001.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 2b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -4.7 |
| 3a | 23 | 2001.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 3b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -6.2 |
| 4a | 23 | 2001.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 4b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -6.2 |
| 5a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 5b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -8.2 |
| 5c | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 6a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 6b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 6c | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 7a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 7b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -9.7 |
| 7c | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -4.7 |
| 8a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 8b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 8c | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -4.7 |
| 9a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -14.7 |
| 9b | 23 | 2004.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 9c | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -6.2 |
| 10a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| 10b | 23 | 2005.5 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| 10c | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 11a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| 11b | 23 | 2005.5 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| 11c | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 12a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -18.7 |
| 12b | 23 | 2005.5 MHz |  |  |  |  | 23 | + 2.7 / -18.7 |
| 12c | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 13a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -18.7 |
| 13b | 23 | 2005.5 MHz |  |  |  |  | 23 | + 2.7 / -18.7 |
| 13c | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 14a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -20.7 |
| 14b | 23 | 2005.5 MHz |  |  |  |  | 23 | + 2.7 / -20.7 |
| 14c | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -4.7 |
| 15a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -21.7 |
| 15b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -16.7 |
| 16a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 16b (Note 1) | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -13.2 |
| 16b (Note 2) | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -12.2 |
| 17a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 17b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 18a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 18b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 19a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -16.7 |
| 19b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -8.7 |
| 20a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 20b (Note 1) | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -13.2 |
| 20b (Note 2) | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -13.2 |
| 21a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -4.7 |
| 21b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 22a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 22b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 23a | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| 23b | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -9.7 |
| 24 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -20.7 |
| 25 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 26 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -13.7 |
| 27 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -16.7 |
| 28 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 29 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -14.7 |
| 30 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| Note 1: RBstart = RB# 0 Note 2: RBstart = RB# (max +1 - RB allocation) |  |  |  |  |  |  |  |  |

Table 6.2.4.5-12: UE Power Class 3 test requirements (network signalled value "NS_12")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3 | 1.5 | 18.5 | 4 | 25.7 | 13.8 |
| 2 | 1 | 4 | 1.5 | 16.5 | 5 | 25.7 | 10.8 |
| 3 | 0 | 0 | 1.5 | 21.5 | 2 | 25.7 | 18.8 |
| 4 | 0 | 3 | 1.5 | 18.5 | 4 | 25.7 | 13.8 |
| 5 | 2 | 4 | 1.5 | 15.5 | 5 | 25.7 | 9.8 |
| 6 | 0 | 4 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 7 | 1 | 3 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 8 | 0 | 0 | 1.5 | 21.5 | 2 | 25.7 | 18.8 |
| 9 | 1 | 3 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 10 | 2 | 4 | 1.5 | 15.5 | 5 | 25.7 | 9.8 |
| 11 | 0 | 4 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 12 | 1 | 3 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 13 | 0 | 0 | 1.5 | 21.5 | 2 | 25.7 | 18.8 |
| 14 | 1 | 3 | 0 | 19 | 3,5 | 25.7 | 14.8 |
| 15 | 2 | 3 | 0 | 18 | 4 | 25.7 | 13.3 |
| 16 | 1 | 4 | 1.5 | 16.5 | 5 | 25.7 | 10.8 |
| 17 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 18 | 2 | 2 | 0 | 19 | 2 | 25.7 | 16.3 |
| 19 | 1 | 4 | 0 | 18 | 4 | 25.7 | 13.3 |
| 20 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 21 | 2 | 3 | 0 | 18 | 4 | 25.7 | 13.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-13: UE Power Class 3 test requirements (network signalled value "NS_13")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 2 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 3 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 4 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 5 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 6 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 7 | 2 | 2 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-14: UE Power Class 3 test requirements (network signalled value "NS_14")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 2 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 3 | 1 | 1 | 0 | 21 | 2 | 25.7 | 18.3 |
| 4 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 5 | 2 | 1 | 0 | 20 | 2,5 | 25.7 | 16.8 |
| 6 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 7 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 8 | 1 | 1 | 0 | 21 | 2 | 25.7 | 18.3 |
| 9 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 10 | 2 | 1 | 0 | 20 | 2,5 | 25.7 | 16.8 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-15: UE Power Class test requirements (network signalled value "NS_15")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 |  |  |  |  | 23 | +2.7 / -4.2 |
| 2 | 26 |  |  |  |  | 23 | +2.7 / -12.2 |
| 3 | 26 |  |  |  |  | 23 | +2.7 / -5.7 |
| 4 | 26 |  |  |  |  | 23 | +2.7 / -12.2 |
| 5 | 26 |  |  |  |  | 23 | +2.7 / -19.2 |
| 6 | 26 |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 26 |  |  |  |  | 23 | +2.7 / -4.2 |
| 8 | 26 |  |  |  |  | 23 | +2.7 / -11.7 |
| 9 | 26 |  |  |  |  | 23 | +2.7 / -17.7 |
| 10 | 26 |  |  |  |  | 23 | +2.7 / -3.7 |
| 11 | 26 |  |  |  |  | 23 | +2.7 / -6.2 |
| 12 | 26 |  |  |  |  | 23 | +2.7 / -17.2 |
| 13 | 26 |  |  |  |  | 23 | +2.7 / -8.2 |
| 14 | 26 |  |  |  |  | 23 | +2.7 / -2.7 |
| 15 | 26 |  |  |  |  | 23 | +2.7 / -3.7 |
| 16 | 26 |  |  |  |  | 23 | +2.7 / -12.7 |
| 17 | 26 |  |  |  |  | 23 | +2.7 / -15.7 |
| 18 | 26 |  |  |  |  | 23 | +2.7 / -17.7 |
| 19 | 26 |  |  |  |  | 23 | +2.7 / 3.7- |
| 20 | 26 |  |  |  |  | 23 | +2.7 / -9.7 |
| 21 | 26 |  |  |  |  | 23 | +2.7 / -3.7 |
| 22 | 26 |  |  |  |  | 23 | +2.7 / -3.7 |
| 23 | 26 |  |  |  |  | 23 | +2.7 / -11.7 |
| 24 | 26 |  |  |  |  | 23 | +2.7 / -17.7 |
| 25 | 26 |  |  |  |  | 23 | +2.7 / -3.7 |
| 26 | 26 |  |  |  |  | 23 | +2.7 / -11.7 |

Table 6.2.4.5-16A: UE Power Class test requirements (network signalled value "NS_16") when lower channel edge is at 807 MHz

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 2 | 27 |  |  |  |  | 23 | +2.7/-3.7 |
| 3 | 27 |  |  |  |  | 23 | +2.7-4.7 |
| 4 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 5 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 6 | 27 |  |  |  |  | 23 | +2.7 / -6.2 |
| 7 | 27 |  |  |  |  | 23 | +2.7 / -8.2 |
| 8 | 27 |  |  |  |  | 23 | +2.7 / -9.7 |
| 9 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 10 | 27 |  |  |  |  | 23 | +2.7 / -6.2 |
| 11 | 27 |  |  |  |  | 23 | +2.7 / -11.7 |
| 12 | 27 |  |  |  |  | 23 | +2.7 / -8.2 |
| 13 | 27 |  |  |  |  | 23 | +2.7 / -11.7 |
| 14 | 27 |  |  |  |  | 23 | +2.7 / -12.7 |
| 15 | 27 |  |  |  |  | 23 | +2.7 / -9.7 |
| 16 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 17 | 27 |  |  |  |  | 23 | +2.7 / -8.2 |
| 18 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 19 | 27 |  |  |  |  | 23 | +2.7 / -8.2 |
| 20 | 27 |  |  |  |  | 23 | +2.7 / -13.7 |
| 21 | 27 |  |  |  |  | 23 | +2.7 / -13.7 |
| 22 | 27 |  |  |  |  | 23 | +2.7 / -14.7 |
| 23 | 27 |  |  |  |  | 23 | +2.7 / -13.7 |
| 24 | 27 |  |  |  |  | 23 | +2.7 / -13.7 |

Table 6.2.4.5-16B: UE Power Class test requirements (network signalled value "NS_16") when lower channel edge is at 808.5 MHz

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 2 | 27 |  |  |  |  | 23 | +2.7/-3.7 |
| 3 | 27 |  |  |  |  | 23 | +2.7/-5.2 |
| 4 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 5 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 6 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 8 | 27 |  |  |  |  | 23 | +2.7 / -2.7 |
| 9 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 10 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 11 | 27 |  |  |  |  | 23 | +2.7 / -6.2 |
| 12 | 27 |  |  |  |  | 23 | +2.7 / -6.2 |
| 13 | 27 |  |  |  |  | 23 | +2.7 / -8.2 |
| 14 | 27 |  |  |  |  | 23 | +2.7 / -9.7 |
| 15 | 27 |  |  |  |  | 23 | +2.7 / -9.7 |
| 16 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 17 | 27 |  |  |  |  | 23 | +2.7 / -6.2 |
| 18 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 19 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 20 | 27 |  |  |  |  | 23 | +2.7 / -9.7 |
| 21 | 27 |  |  |  |  | 23 | +2.7 / -11.7 |
| 22 | 27 |  |  |  |  | 23 | +2.7 / -12.7 |
| 23 | 27 |  |  |  |  | 23 | +2.7 / -11.7 |
| 24 | 27 |  |  |  |  | 23 | +2.7 / -11.7 |

Table 6.2.4.5-16C: UE Power Class test requirements (network signalled value "NS_16") when lower channel edge is at 812 MHz

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 2 | 27 |  |  |  |  | 23 | +2.7/-3.7 |
| 3 | 27 |  |  |  |  | 23 | +2.7/-4.7 |
| 4 | 27 |  |  |  |  | 23 | +2.7/-2.7 |
| 5 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 6 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 7 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 8 | 27 |  |  |  |  | 23 | +2.7 / -2.7 |
| 9 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 10 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 11 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 12 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 13 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 14 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 15 | 27 |  |  |  |  | 23 | +2.7 / -2.7 |
| 16 |  |  |  |  |  | 23 | +2.7/-2.7 |
| 17 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 18 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 19 | 27 |  |  |  |  | 23 | +2.7 / -3.7 |
| 20 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |
| 21 | 27 |  |  |  |  | 23 | +2.7 / -8.2 |
| 22 | 27 |  |  |  |  | 23 | +2.7 / -9.7 |
| 23 | 27 |  |  |  |  | 23 | +2.7 / -6.2 |
| 24 | 27 |  |  |  |  | 23 | +2.7 / -4.7 |

Table 6.2.4.5-17: UE Power Class test requirements (network signalled value "NS_17")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 28 |  |  |  |  | 23 | +2.7 / -3.2 |
| 2 | 28 |  |  |  |  | 23 | +2.7 / -3.2 |
| 3 | 28 |  |  |  |  | 23 | +2.7 / -4.2 |
| 4 | 28 |  |  |  |  | 23 | +2.7 / -5.2 |
| 5 | 28 |  |  |  |  | 23 | +2.7 / -3.2 |
| 6 | 28 |  |  |  |  | 23 | +2.7 / -3.2 |
| 7 | 28 |  |  |  |  | 23 | +2.7 / -4.2 |
| 8 | 28 |  |  |  |  | 23 | +2.7 / -5.2 |

Table 6.2.4.5-18: UE Power Class test requirements (network signalled value "NS_18")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 28 |  |  |  |  | 23 | +2.7 / -3.2 |
| 2 | 28 |  |  |  |  | 23 | +2.7 / -4.2 |
| 3 | 28 |  |  |  |  | 23 | +2.7 / -5.2 |
| 4 | 28 |  |  |  |  | 23 | +2.7 / -6.2 |
| 5 | 28 |  |  |  |  | 23 | +2.7 / -7.2 |
| 6 | 28 |  |  |  |  | 23 | +2.7 / -7.2 |
| 7 | 28 |  |  |  |  | 23 | + 2.7 / -8.7 |
| 8 | 28 |  |  |  |  | 23 | +2.7 / -9.2 |
| 9 | 28 |  |  |  |  | 23 | +2.7 / -7.2 |
| 10 | 28 |  |  |  |  | 23 | +2.7 / -7.2 |
| 11 | 28 |  |  |  |  | 23 | +2.7 / -8.7 |
| 12 | 28 |  |  |  |  | 23 | +2.7 / -9.2 |
| 13 | 28 |  |  |  |  | 23 | +2.7 / -7.2 |
| 14 | 28 |  |  |  |  | 23 | +2.7 / -7.2 |
| 15 | 28 |  |  |  |  | 23 | +2.7 / -8.7 |
| 16 | 28 |  |  |  |  | 23 | +2.7 / -9.2 |

Table 6.2.4.5-19: UE Power Class test requirements (network signalled value "NS_19")

| Configuration ID | Test Freq | RB location | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Low | Full RB | 1 | 0 | 1.5 | 20.5 | 2.5 | 25.7 | 17.3 |
| 2 | Low | Full RB | 2 | 0 | 1.5 | 19.5 | 3.5 | 25.7 | 15.3 |
| 3 | Low | Low | 1 | 1 | 0 | 21 | 2 | 25.7 | 18.3 |
| 3 | Low | High | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 4 | Low | Low | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 4 | Low | High | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 5 | Low | Low | 0 | 2 | 1.5 | 19.5 | 3.5 | 25.7 | 15.3 |
| 5 | Low | High | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 6 | Low | Low | 1 | 2 | 1.5 | 18.5 | 4 | 25.7 | 13.8 |
| 6 | Low | High | 1 | 0 | 0 | 22 | 2.5 | 25.7 | 18.8 |
| 7 | Low | Low | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 7 | Low | High | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 8 | Low | Low | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 8 | Low | High | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 9 | Low | Low | 2 | 3 | 0 | 18 | 4 | 25.7 | 13.3 |
| 9 | Low | High | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 10 | Low | Low | 0 | 2 | 1.5 | 19.5 | 3.5 | 25.7 | 15.3 |
| 10 | Low | High | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 11 | Low | Low | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 11 | Low | High | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 12 | Low | Low | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 12 | Low | High | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 13 | Low | Low | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 13 | Low | High | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 14 | Low | Low | 2 | 3 | 0 | 18 | 4 | 25.7 | 13.3 |
| 14 | Low | High | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |

Table 6.2.4.5-20: UE Power Class test requirements (network signalled value "NS_20 for Band 23")

| Configuration ID | EUTRA Band | Centre Frequency | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -24.7 (Note 1) |
| 1b | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 1c | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 1d | 23 | 2017.5 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 2a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -23.7 (Note 1) |
| 2b | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -7.7 |
| 2c | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -7.7 |
| 2d | 23 | 2017.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 3a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -23.7 (Note 1) |
| 3b | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 3c | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 3d | 23 | 2017.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 4a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -23.7 (Note 1) |
| 4b | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -7.7 |
| 4c | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -7.7 |
| 4d | 23 | 2017.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 5a | 23 | 2002.5 MHz |  |  |  |  | 23 | + 2.7 / -23.7 (Note 1) |
| 5b | 23 | 2007.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 5c | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 5d | 23 | 2017.5 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 6a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -21.7 (Note 1) |
| 6b | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 7a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -7.7 (Note 1) |
| 7b | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -2.7 |
| 8a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -11.7 (Note 1) |
| 8b | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -8.7 |
| 9a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -8.7 (Note 1) |
| 9b | 23 | 2015 MHz |  |  |  |  | 23 | + 2.7 / -3.7 |
| 10a | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -11.7 (Note 1) |
| 10b | 23 | 2005 MHz |  |  |  |  | 23 | + 2.7 / -8.7 |
| 11 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 12 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -7.7 |
| 13 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -11.7 |
| 14 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -17.7 |
| 15 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -8.7 |
| 16 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -12.7 |
| 17 | 23 | 2012.5 MHz |  |  |  |  | 23 | + 2.7 / -18.7 |
| 18 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -22.7 |
| 19 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -14.7 |
| 20 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -13.7 (Note 1) |
| 21 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -8.7 (Note 1) |
| 22 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -11.7 (Note 1) |
| 23 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -15.7 (Note 1) |
| 24 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -14.7 (Note 1) |
| 25 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -19.7 (Note 1) |
| 26 | 23 | 2010 MHz |  |  |  |  | 23 | + 2.7 / -12.7 (Note 1) |
| Note 1: The output power within 2000-2005 MHz shall not be higher than 7.7 dBm |  |  |  |  |  |  |  |  |

Table 6.2.4.5-21: UE Power Class 3 test requirements (network signalled value "NS_21 for Band 30")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 2 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 3 | 0 | 4 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 4 | 0 | 4 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 5 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 6 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 7 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 8 | 2 | 3 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-22: UE Power Class Test requirements (network signalled value "NS_22")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 2 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 3 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 4 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 5 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 6 | 1 | 4 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 7 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 8 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 9 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 10 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 11 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 12 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 13 | 1 | 4 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 14 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 15 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 16 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 17 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 18 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 19 | 1 | 4 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 20 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 21 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 22 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 23 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-23: UE Power Class Test requirements (network signalled value "NS_23")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 2 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 3 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 4 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 5 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 6 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 7 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 8 | 1 | 0 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-24: UE Power Class 3 test requirements (network signalled value "NS_24")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 10 | 0 | 13 | 5.0 | 25.7 | 7.3 |
| 2 | 0 | 10 | 0 | 13 | 5.0 | 25.7 | 7.3 |
| 3 | 1 | 10 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 4 | 2 | 10 | 0 | 11 | 6.0 | 25.7 | 4.3 |
| 5 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 6 | 0 | 2 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 7 | 2 | 2 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 8 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 9 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 10 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 11 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 12 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 13 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 14 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 15 | 0 | 9 | 0 | 14 | 5.0 | 25.7 | 8.3 |
| 16 | 0 | 9 | 0 | 14 | 5.0 | 25.7 | 8.3 |
| 17 | 1 | 9 | 0 | 13 | 5.0 | 25.7 | 7.3 |
| 18 | 0 | 17 | 0 | 6 | 7.0 | 25.7 | -1.7 |
| 19 | 1 | 17 | 0 | 5 | 7.0 | 25.7 | -2.7 |
| 20 | 2 | 17 | 0 | 4 | 7.0 | 25.7 | -3.7 |
| 21 | 0 | 2 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 22 | 0 | 2 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 23 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 24 | 2 | 8 | 0 | 13 | 5.0 | 25.7 | 7.3 |
| 25 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 26 | 0 | 7 | 0 | 16 | 5.0 | 25.7 | 10.3 |
| 27 | 1 | 7 | 0 | 15 | 5.0 | 25.7 | 9.3 |
| 28 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 29 | 0 | 17 | 0 | 6 | 7.0 | 25.7 | -1.7 |
| 30 | 0 | 17 | 0 | 6 | 7.0 | 25.7 | -1.7 |
| 31 | 1 | 17 | 0 | 5 | 7.0 | 25.7 | -2.7 |
| 32 | 2 | 17 | 0 | 4 | 7.0 | 25.7 | -3.7 |
| 33 | 0 | 17 | 0 | 6 | 7.0 | 25.7 | -1.7 |
| 34 | 0 | 17 | 0 | 6 | 7.0 | 25.7 | -1.7 |
| 35 | 1 | 17 | 0 | 5 | 7.0 | 25.7 | -2.7 |
| 36 | 2 | 17 | 0 | 4 | 7.0 | 25.7 | -3.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-25: UE Power Class 3 test requirements (network signalled value "NS_25")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 2 | 2 | 5 | 0 | 16 | 5.0 | 25.7 | 10.3 |
| 3 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 4 | 0 | 2 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 5 | 0 | 2 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 6 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 7 | 0 | 2 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 8 | 2 | 2 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 9 | 1 | 2 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 10 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 11 | 0 | 3 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 12 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 13 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 14 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 15 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 16 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 17 | 1 | 5 | 0 | 17 | 5.0 | 25.7 | 11.3 |
| 18 | 1 | 1 | 0 | 21 | 2.0 | 25.7 | 18.3 |
| 19 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 20 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 21 | 0 | 10 | 0 | 13 | 5.0 | 25.7 | 7.3 |
| 22 | 0 | 10 | 0 | 13 | 5.0 | 25.7 | 7.3 |
| 23 | 1 | 10 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 24 | 2 | 10 | 0 | 11 | 6.0 | 25.7 | 4.3 |
| 25 | 1 | 7 | 0 | 15 | 5.0 | 25.7 | 9.3 |
| 26 | 2 | 7 | 0 | 14 | 5.0 | 25.7 | 8.3 |
| 27 | 1 | 11 | 0 | 11 | 6.0 | 25.7 | 4.3 |
| 28 | 2 | 11 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 29 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 30 | 0 | 4 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 31 | 1 | 4 | 0 | 18 | 4.0 | 25.7 | 13.3 |
| 32 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 33 | 2 | 1 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 34 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 35 | 0 | 1 | 0 | 22 | 2.0 | 25.7 | 19.3 |
| 36 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 37 | 0 | 13 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 38 | 0 | 13 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 39 | 1 | 13 | 0 | 9 | 6.0 | 25.7 | 2.3 |
| 40 | 2 | 13 | 0 | 8 | 6.0 | 25.7 | 1.3 |
| 41 | 0 | 11 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 42 | 0 | 11 | 0 | 12 | 6.0 | 25.7 | 5.3 |
| 43 | 2 | 11 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 44 | 1 | 11 | 0 | 11 | 6.0 | 25.7 | 4.3 |
| 45 | 0 | 13 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 46 | 2 | 13 | 0 | 8 | 6.0 | 25.7 | 1.3 |
| 47 | 0 | 0 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 48 | 0 | 13 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 49 | 0 | 13 | 0 | 10 | 6.0 | 25.7 | 3.3 |
| 50 | 1 | 13 | 0 | 9 | 6.0 | 25.7 | 2.3 |
| 51 | 2 | 13 | 0 | 8 | 6.0 | 25.7 | 1.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-26: Reserved

Table 6.2.4.5-27: UE Power Class 3 test requirements (network signalled value "NS_27")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | MAX{TL, T(PCMAX_L,c)} (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 2 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 3 | 1 | 0 | 0 | 22 | 3 | 26.0 | 18 |
| 4 | 2 | 0 | 0 | 21 | 3 | 26.0 | 17 |
| 5 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 6 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 7 | 1 | 0 | 0 | 22 | 3 | 26.0 | 18 |
| 8 | 2 | 0 | 0 | 21 | 3 | 26.0 | 17 |
| 9 | 0 | 4 | 0 | 19 | 3.5 | 26.0 | 14.5 |
| 10 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 11 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 12 | 0 | 4 | 0 | 19 | 3.5 | 26.0 | 14.5 |
| 13 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 14 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 15 | 1 | 2 | 0 | 20 | 3 | 26.0 | 16 |
| 16 | 2 | 2 | 0 | 19 | 3.5 | 26.0 | 14.5 |
| 17 | 0 | 4 | 0 | 19 | 3.5 | 26.0 | 14.5 |
| 18 | 0 | 1 | 0 | 22 | 3 | 26.0 | 18 |
| 19 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 20 | 1 | 0 | 0 | 22 | 3 | 26.0 | 18 |
| 21 | 0 | 4 | 0 | 19 | 3.5 | 26.0 | 14.5 |
| 22 | 0 | 1 | 0 | 22 | 3 | 26.0 | 18 |
| 23 | 0 | 0 | 0 | 23 | 3 | 26.0 | 19 |
| 24 | 1 | 0 | 0 | 22 | 3 | 26.0 | 18 |
| 25 | 1 | 3 | 0 | 19 | 3.5 | 26.0 | 14.5 |
| 26 | 2 | 3 | 0 | 18 | 4.0 | 26.0 | 13 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-28: UE Power Class 3 test requirements (network signalled value "NS_35")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 2 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 3 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 4 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 5 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 6 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 7 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 8 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 9 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 10 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 11 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| 12 | 0 | 0 | 0 | 23 | 2 | 25.7 | 18.8 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-29: UE Power Class 3 test requirements (network signalled value "NS_36")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 2 | 0 | 21 | 2 | 25.7 | 18.3 |
| 2 | 1 | 4 | 0 | 18 | 4 | 25.7 | 13.3 |
| 3 | 1 | 1 | 0 | 21 | 2 | 25.7 | 18.3 |
| 4 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 5 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 7 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 8 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 9 | 1 | 10 | 0 | 12 | 6 | 25.7 | 5.3 |
| 10 | 0 | 10 | 0 | 13 | 5 | 25.7 | 7.3 |
| 11 | 1 | 8 | 0 | 14 | 5 | 25.7 | 8.3 |
| 12 | 1 | 6 | 0 | 16 | 5 | 25.7 | 10.3 |
| 13 | 0 | 6 | 0 | 17 | 5 | 25.7 | 11.3 |
| 14 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 15 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 16 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 17 | 1 | 10 | 0 | 12 | 6 | 25.7 | 5.3 |
| 18 | 1 | 9 | 0 | 13 | 5 | 25.7 | 7.3 |
| 19 | 1 | 7 | 0 | 15 | 5 | 25.7 | 9.3 |
| 20 | 0 | 6 | 0 | 17 | 5 | 25.7 | 11.3 |
| 21 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 22 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 23 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 24 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 25 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

[End of changes]