3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core alignment of RMC table for TM9 CSI-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL_CA-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table A.3.3.3.1-1 and Table A.3.4.3.3-1 need to be updated according to the 36.101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table A.3.3.3.1-1 is aligned with core spec : Note 5-7 are added. Table A.3.4.3.3-1 is aligned with core spec : Note 6-8 are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RMC for TM9 CSI-RS remain not aligned wit core-spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.3.1-1, A.3.4.3.3-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

#### A.3.3.3.1	Two antenna port (CSI-RS)

The reference measurement channels in Table A.3.3.3.1-1 apply for verifying demodulation performance for UE-specific reference symbols with two cell-specific antenna ports and two CSI-RS antenna ports.

Table A.3.3.3.1-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.51 FDD | R.51-1 FDD | R.76 FDD | R.51-2 FDD | R.51-3 FDD | R.51-4 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 5 | 15 | 20 |
| Allocated resource blocks |  | 50 (Note 3) | 50 (Note 3) | 50 (Note 3) | 25 (Note 5) | 75 (Note 6) | 100 (Note 7) |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation |  | 16QAM | 16QAM | QPSK | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 0.54 |  | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 11448 | 12960 | 6200 | 5736 | 16992 | 22920 |
| For Sub-Frames 2,3,7,8 | Bits | 11448 | 12960 | 6200 | 5736 | 16992 | 22920 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 9528 | 10680 | 4968 | 3880 | 14112 | 19848 |
| Number of Code Blocks (Note 4) |  |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 2 | 3 | 2 | 1 | 3 | 4 |
| For Sub-Frames 2,3,7,8 | Code blocks | 2 | 3 | 2 | 1 | 3 | 4 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 2 | 2 | 1 | 1 | 3 | 4 |
| Binary Channel Bits |  |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 24000 | 24000 | 12000 | 10800 | 36000 | 48000 |
| For Sub-Frames 2,7 |  | 23600 | 23600 | 11800 | 10600 | 35400 | 47200 |
| For Sub-Frames 3,8 |  | 23200 | 23200 | 12000 | 10400 | 34800 | 46400 |
| For Sub-Frame 5 | Bits | N/A | N/A | n/a | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 19680 | 19680 | 9840 | 6912 | 30240 | 42240 |
| Max. Throughput averaged over 1 frame | Mbps | 10.1112 | 11.436 | 5.4568 | 4.9768 | 15.0048 | 20.3208 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [8]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 25 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 17 resource blocks (RB0–RB9 and RB18–RB24) are allocated in sub-frame 0. Note 6: 75 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 63 resource blocks (RB0–R31 and RB44–RB74) are allocated in sub-frame 0. Note 7: 100 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 88 resource blocks (RB0–RB43 and RB56–RB99) are allocated in sub-frame 0. |  |  |  |  |  |  |  |

The reference measurement channels in Table A3.3.3.1-2 apply for verifying demodulation performance for UE-specific reference symbols with two cell specific antenna ports and two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS in same subframe.

## << Unchanged sections omitted >>

#### A.3.4.3.3	Two antenna ports (CSI-RS)

The reference measurement channels in Table A.3.4.3.3-1 apply for verifying demodulation performance for CDM-multiplexed UE specific reference symbols with two cell-specific antenna ports and two CSI-RS antenna ports.

Table A.3.4.3.3-1: Fixed Reference Channel for CDM-multiplexed DM RS with two CSI-RS antenna ports

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.51 TDD | R.51-1 TDD | R.76 TDD | R.51-2 TDD | R.51-3 TDD | R.51-4 TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 5 | 15 | 20 |
| Allocated resource blocks |  | 50 (Note 5) | 50 (Note 5) | 50 (Note 5) | 25 (Note 6) | 75 (Note 7) | 100 (Note 8) |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Modulation |  | 16QAM | 16QAM | QPSK | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 0.57 |  | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | 11448 | N/A | 6200 | NA | NA | NA |
| For Sub-Frame 4,9 | Bits | 11448 | 12960 | 6200 | 4968 | 16992 | 22920 |
| For Sub-Frames 1,6 | Bits | 7736 | 9144 | 4264 | 3112 | 12216 | 16992 |
| For Sub-Frame 5 | Bits | N/A | N/A | n/a | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 9528 | 10680 | 4968 | 3496 | 14112 | 19848 |
| Number of Code Blocks (Note 4) |  |  |  |  |  |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Code blocks | 2 | N/A | 2 | N/A | N/A | N/A |
| For Sub-Frames 4,9 | Code blocks | 2 | 3 | 2 | 1 | 3 | 4 |
| For Sub-Frames 1,6 | Code blocks | 2 | 2 | 1 | 1 | 3 | 3 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Code blocks | 2 | 2 | 1 | 1 | 3 | 4 |
| Binary Channel Bits |  |  |  |  |  |  |  |
| For Sub-Frames 4, 9 (non CSI-RS subframe) | Bits | 24000 | N/A | 11800 | NA | NA | NA |
| For Sub-Frames 4,9 |  | 22800 | 22800 | 11800 | 10200 | 34200 | 45600 |
| For Sub-Frames 1,6 |  | 15744 | 15744 | 7872 | 6144 | 24192 | 33792 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 19680 | 19680 | 9840 | 6912 | 30240 | 42240 |
| Max. Throughput averaged over 1 frame | Mbps | 4.7896 | 5.4888 | 2.5896 | 1.9656 | 7.2528 | 99.672 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 6: 25 resource blocks are allocated in sub-frames 4,9 and 17 resource blocks (RB0–RB9 and RB18–RB24) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 7: 75 resource blocks are allocated in sub-frames 4,9 and 63 resource blocks (RB0–R31 and RB44–RB74) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. Note 8: 100 resource blocks are allocated in sub-frames 4,9 and 88 resource blocks (RB0–RB43 and RB56–RB99) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. |  |  |  |  |  |  |  |

The reference measurement channels in Table A3.4.3.3-2 apply for verifying demodulation performance for UE-specific reference symbols with two cell specific antenna ports and two CSI-RS antenna ports with ZP CSI-RS and NZP CSI-RS in same subframe.

## << End of changes >>