3GPP TSG-RAN5 #84	R5-195916

# Ljubljana, Slovenia, 26th – 30th Aug 2019

Title:	Uncertainty of the absolute gain of the calibration antenna at spurious region

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Endorsement

# 1.	Introduction

In this contribution we introduce a measurement uncertainty (MU) value of the absolute gain of the calibration antenna at spurious region.

# 2.	Discussion

# 2.1	Estimation of absolute gain of the calibration antenna at spurious region

To research the achievable MU of the calibration antenna at spurious region, we obtained an estimated value from an antenna calibration lab in Japan.

Based on the following reference standard for calibration procedure, we received the uncertainty value of 1.67 dB with normal distribution (k= 2). The test method applied to the calibration procedure is the three antenna method.

CISPR 16-1-6:2014 + AMD 1:2017 [1]

Proposal 1: Set Uncertainty of the absolute gain of the calibration antenna at spurious region (40.8 GHz to 90 GHz) as 1.7 dB with normal distribution (k= 2).

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 22 | Uncertainty of an absolute gain of the calibration antenna | 1.70 | Normal | 2.00 | 0.85 |

# 2.2	Assumption

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | 40.8 GHz to 90 GHz |
| #3 | Size of QZ for IFF | N/A |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | N/A |
| #6 | Temperature variation impact | N/A |
| #7 | UE power class | N/A |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

- Conclusion

In this contribution we introduced a measurement uncertainty value of the absolute gain of the calibration antenna at spurious region.

Proposal 1: Set Uncertainty of the absolute gain of the calibration antenna at spurious region (40.8 GHz to 90 GHz) as 1.7 dB with normal distribution (k= 2).

# 4.	References

[1] CISPR 16-1-6:2014 + AMD 1:2017, “Specification for radio disturbance and immunity measuring apparatus and methods - Part 1-6: Radio disturbance and immunity measuring apparatus - EMC antenna calibration”, IEC, 2017

[2] R5-185954, “On the MU Element “Uncertainty of an absolute gain of the calibration antenna””, Keysight Technologies, RAN5 #3-5G-NR AH, Jaipur, India