3GPP TSG RAN5 Meeting #86-e	R5-200868

Electronic Meeting, 17th – 28th February 2020

Source:		Rohde & Schwarz

Title:	On the SNR for FR2 TRx test cases

Agenda Item:		5.3.5.17

Document for:	Endorsement

# Introduction

At RAN5 #86, estimations for the SNR and relaxations of core requirements have been discussed [1 – 4]. In this document, we present the results of our analysis based on the common assumptions for the not yet decided figures.

# Discussion

A detailed description of the total SNRtotal based on the measurement of total component power for both polarizations can be found in reference [6].

In Table 1, we present our results for SNRtotal under the assumption that the UE is allowed to transmit a maximum peak EIRP of 43 dBm.

For ACLR we provide the same numbers as in the previous meeting but note that the general case can be derived by

SNR = SNRtable + (6.5 dB – MPR) + CHBW + ArrayMetric

The values in the table SNRtable are for assumption 1, i.e., 8x2 antenna array and TRP metric, an MPR of 6.5 dB , and 400 MHz channel bandwidth. MPR is the worst MPR of the chosen test points, and CHBW = 10log10(400MHz/CHBW). ArrayMetric is 0 dB for 8x2 antenna array and TRP metric (ACLR Assumption 1). ArrayMetric is 6 dB for 1x4 antenna array and TRP metric (ACLR Assumption 2). For EIRP metric, ArrayMetric is 16 dB and is independent from the directivity of the antenna array.

The MPR depends on waveform, modulation and RB allocation. For the currently defined test points the highest MPR occurs for 400 MHz channel bandwidth and 64QAM DFT-s-OFDM with a value of 6.5 dB. It is noted that the MU and SNR analysis has to be updated once CP-OFDM 64QAM were applied as test point for ACLR.

Table 1: SNRtotal,core based on core requirement

| Test case | Frequency (GHz) | Estimated SNRtotal(dB) | Requirement (dBm) | Impact of Noise w/o relaxation (dB) | Required Relaxation |
| --- | --- | --- | --- | --- | --- |
| ACLR AC (TRP) Assumption 1 | 23.45 ≤ f ≤ 32.125 | -11.2 | -18.8 (= 20.7 – 16 – 17 – 6.5) | 11.52 | yes |
|  | 32.125 < f ≤ 40.8 | -14.1 | -19.6 (= 18.9 – 16 – 16 – 6.5) | 14.27 | yes |
| ACLR wanted signal (TRP) Assumption 1 | 23.45 ≤ f ≤ 32.125 | 5.8 | -1.8 (= 20.7 – 16 – 6.5) | 1.02 | yes |
|  | 32.125 < f ≤ 40.8 | 1.9 | -3.6 (= 18.9 – 16 – 6.5) | 2.17 | yes |
| Transmitter Spurious Emissions | 6 ≤ f < 12.75 | 10 | -30 | 0.41 | no |
|  | 12.75 ≤ f ≤ 50 | 10 | -13 | 0.41 | no |
|  | 50 ≤ f ≤ 80 | 10 | -13 | 0.41 | no |
| Spurious Emissions for UE co-existence | to other FR2 bands | 10 | -25 (referred to 1 MHz MBW) | 0.41 | no |
|  | 57 ≤ f ≤ 66 | work in progress |  |  |  |
| Receiver Spurious Emissions | 6 ≤ f < 20 | 8 | -47 + 10.2 = -36.8 | 0.64 | yes |
|  | 20 ≤ f < 34 | 8 | -47 + 17.2 = -29.8 | 0.64 | yes |
|  | 34 ≤ f < 40 | 8 | -47 + [17.2] = -29.8 | 0.64 | yes |
|  | 40 ≤ f ≤ 80 | 8 | -47 + 33.1 = -13.9 | 0.64 | yes |

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise for the TT analysis.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements 6 GHz – 80 GHz for spurious emissions measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, SNR estimations for low PSD test cases have been provided and the influence of noise has been analyzed.

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise for the TT analysis.

# References

- R5-198733, On the SNR for FR2 TRx test cases, 3GPP TSG RAN WG5 Meeting #85, November 2019

- R5-199497, Proposal of noise impact for FR2 Transmitter Test Case, Anritsu, Keysight, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #85, November 2019

- R5-199561, On testability issue for prioritized test cases, NTT Docomo, 3GPP TSG RAN WG5 Meeting #85, November 2019

- R5-198451, On the FR2 ACLR test metric, Anritsu, 3GPP TSG RAN WG5 Meeting #85, November 2019