3GPP TSG-RAN5 Meeting #89-e	R5-20xxxx

Electronic Meeting, 9th Nov 2020 - 20th Nov 2020

Agenda Item:	6.3.2.20

Source:	Lenovo, Motorola Mobility

Title:	Duplication of PICS for FDD/TDD, FR1/FR2 different UE implementations

Document for:	Endorsement

# 1	Introduction

RAN5 #88 following Action Item

| . AP#88.02 | SIG | Review and update as needed all existing ICS definition to be compliant with TS 38.306 definition - UE capability field can have a different value for FDD and TDD if 'FDD-TDD DIFF' is set as 'Yes', and for FR1 and FR2 if 'FR1-FR2 DIFF' is set as 'Yes' | Motorola Mobility (TS 38.508-2 Rapporteur) | R5-204853 | RAN5#89 | Pending |
| --- | --- | --- | --- | --- | --- | --- |

There are quite a few PICS in 38.306 with FDD/TDD Diff =Yes and/or FR1/FR2 Diff = Yes. In theory if we strictly follow 38.306, we will have to at-least duplicate or possibly triplicate the PICS respectively for FDD/TDD/FR2.

This in turn will have a cascading impact on other specs. The biggest impact will be on Applicability specs which would require duplication or triplication of Applicability statements. A lesser impact will be on Prose in 38.xxx-1 specs and 38.508-1 where the PICS might have been used.

Example 1: pc_longDRX_Cycle. The PICS for Long DRX cycle has FDD/TDD Diff =Yes. Hence ideally, we would require pc_longDRX_Cycle_FDD and pc_longDRX_Cycle_TDD.

Example 2: pc_interRAT_EUTRA_Handover. This PICS if FDD/TDD and FR1/FR2 diff. Ideally we would require 3 PICS pc_interRAT_EUTRA_Handover_FDD, pc_interRAT_EUTRA_Handover_TDD_FR1, pc_interRAT_EUTRA_Handover_FR2.

From a signalling testing perspective, this may be an overkill and should not go for a ideal approach. The logic being there is single Primary band under which the test will be executed, and the PICS should be set as per the Primary band under test. For example, if an execution sets NR FDD FR1 Band as Primary band, all PICS should be set based on UE support in this band.

There could be need basis exceptions for this above assumption. For example, inter frequency Handover PICS. We already have 3 PICS defined for this, pc_handoverInterF, pc_FR1toFR2_Handover & pc_FDDtoTDD_Handover. Which seems to be sufficient for now.

# 2	Proposal

Meeting is request endorse one of the way forwards

- Duplicate/triplicate the PICS to be strictly complaint with 38.306. OR

- RAN5 assumption that PICS are primarily defined for primary band of operation and add variants of PICS only when there is a justified use case where single PICS approach is deficient.

# 4	References

[1]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.