3GPP TSG-RAN5 Meeting #90-e	R5-210199

# Electronic Meeting, 22nd February – 5th March

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1568 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G Test Models updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The test model needs to be enhanced to be able to implement the NR processing delay test cases 8.1.5.8.1 and 8.1.5.8.2.x. 2. The timing in ASPs for sysinfo modification when SI-OnDemand is applied is left FFS in clause 7.3.3.4. 3. The NR-DC test case 8.2.2.8.2 requires UE to perform an NR-DC intra-Cell PSCell change and an NR-DC intra PCell handover and intra-cell PSCell change. These complex sequences need to be documented. 4. In clause 7.3.5.4, two FollowOnFlag are missing in the RRC connection re-establishment sequence in steps 3 and 13. 5. In the sequence of EN-DC NR intra-cell PSCell change (cl 7.2.6.2) the wording ‘handover’ is unclear, it shall be replaced with ‘PSCell change’. 6. The execution guidelines need to be updated in to take into account test prose updates and the new delivered test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added new clause 7.3.7 for NR processing delay. 2. Deleted Editor’s notes and added a clarification on the timing used in ASPs in clause 7.3.3.4. 3. Added test sequences in clause 7.3.5.5.2 and 7.3.5.5.3. 4. Added two FollowOnFlag in the RRC connection re-establishment sequence in clause 7.3.5.4. 5. Replaced ‘handover’ by ‘PSCell change’ in clause 7.2.6.2. 6. Updated clause 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. NR processing delay test cases will not be implementable. 2. Timing for SI-OnDemand will remain unclear. 3 & 4 & 5 Complex procedural sequences will remain unclear or undocumented. 6. Execution guideline will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.6.2, 7.3.3.4, 7.3.5.4, 7.3.5.5.2(new clause), 7.3.5.5.3(new clause), 7.3.7(new clause), 11.3.1.1, 11.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of modified section>

#### 7.2.6.2	Sequence of EN-DC NR intra-cell PSCell change

For EN-DC NR intra-cell PSCell change dedicated timing information is used: the sequence starts at time T with sending of the RRCConnectionReconfiguration. T is set to 300 ms in advance of the PSCell change.

1.	NR Cell before T:			Get PDCP count for DRBs and SRB3 (if applied).

2.	E-UTRA Cell at T:			Send RRCConnectionReconfiguration.

3.	NR Cell at T:					Release SRB3 (if necessary) and DRBs.

4.	NR Cell at T:					Configure RACH procedure either dedicated or C-RNTI based.

NOTE 1:	The FollowOnFlag is set to true in the ASPs reconfiguring the RadioBearerList (step 3) and C-RNTI (step 4).

5.	NR Cell at T + 5ms:			(Re-)configure SRB3 (if necessary) and DRBs.

6.	NR Cell at T + 5ms:			Restore the PDCP counts

NOTE 2:	For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCConnectionReconfiguration message content.

7.	NR Cell at T + 5ms:			Re-establish security.

8.	E-UTRA Cell (after step 2):	Receive RRCConnectionReconfigurationComplete.

9.	Void

#### <End of modified section>

#### <Start of modified section>

#### 7.3.3.4	Request for on demand system information

In case PRACH preamble (Msg1) is used for indication of requested other SI:

-	TTCN configures SS with the SI-RequestConfig as provided to UE in SIB1 and SS shall monitor these PRACH resources.

-	TTCN configures SS to report PRACH preambles and to transmit Random Access Response (Msg2) as response to Msg1 but not to handle contention resolution.

-	TTCN reconfigures SS to stop broadcasting a specific SI from the start of a modification period and reconfigures SS to restart broadcasting the SI with activation time ‘Now’ (after the reception of PRACH preamble in TTCN). When activation time ‘Now’ is applied, SS shall start broadcasting from the next SI-window.

In case Msg3 is used for indication of requested other SI:

-	TTCN configures the SS to transmit a Msg4 with Contention Resolution Identity upon receipt of RRCSystemInfoRequest (Msg3).

-	TTCN reconfigures SS to stop broadcasting a specific SI from the start of a modification period and reconfigures SS to restart broadcasting the SI with activation time ‘Now’ (after the reception of RRCSystemInfoRequest in TTCN). When activation time ‘Now’ is applied, SS shall start broadcasting from the next SI-window.

#### <End of modified section>

#### <Start of modified section>

#### 7.3.5.4	RRC connection re-establishment

In general, the re-establishment is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCReestablishment is ‘Now'.

The Source Cell and Target cell can be the same cell.

1.	Target Cell:	Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

NOTE 1:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

2.	Target Cell:	Receive RRCReestablismentRequest.

3.	Source Cell:	If the Source Cell is the same as the Target Cell:

Reset SRBs and release DRBs.

Stop periodic TA.

NOTE 2:	The FollowOnFlag is set to true in the ASP configuring the SRBs.

4.	Target Cell:	If the Source Cell is different from the Target Cell:

Configure UL grant configuration ("OnSR").

5.	Target Cell:	Activate Security.

6.	Target Cell:	Send RRCReestablishment.

7.	Target Cell:	Receive RRCReestablishmentComplete.

8.	Target Cell:	Reconfigure DRBs and reconfigure UL DCCH/DTCH DCI on USS.

9.	Target Cell:	Send RRCReconfiguration.

10.	Target Cell:	Receive RRCReconfigurationComplete.

11. Target Cell: 	Reconfigure DL DCCH/DTCH DCI on USS.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

12. Target Cell:	Re-configure RACH procedure as for initial access, enable TA transmissions.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

13.	Source Cell:	If the Source Cell is different from the Target Cell:

Reset SRBs and release DRBs.

Stop periodic UL grant and periodic TA.

NOTE 5:	The FollowOnFlag is set to true in the ASP configuring the SRBs.

<End of modified section>

<Start of modified section>

##### 7.3.5.5.1	Sequence of NR-DC inter-cell PSCell change

In general, the NR inter-cell PSCell change is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCReconfiguration is ‘Now'.

1.	NR Target PSCell:			Configuration of SRB3 (if necessary) and DRBs.

2.	Transfer of the PDCP Count for DRBs and SRB3 (if necessary) from NR source to NR target PSCell:

a)	NR Source PSCell: Get PDCP COUNT.

b)	NR Target PSCell: Set PDCP COUNT.

NOTE 1:	No further sending/receiving of DRB data before the PSCell change is done.

NOTE 2:	For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

3.	NR Target PSCell:			Inform the SS about the PSCell change and about the source cell id.

4.	NR Target PSCell:			Configure RACH procedure either dedicated or C-RNTI based.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

5.	NR Target PSCell:			Activate security.

6.	NR Target PSCell:			Configure UL grant configuration ("OnSR", default grant).

NOTE 4:	Unless explicitly specified UL grant configuration keeps configured as per default at the NR source PSCell.

7.	NR PCell:					Send RRCReconfiguration.

8.	NR PCell:					Receive RRCReconfigurationComplete.

9.	NR Target PSCell:			Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU).

10.	NR Source PSCell:			Release SRB3 (if necessary) and DRBs.

##### 7.3.5.5.2 Sequence of NR-DC intra-cell PSCell change

For NR-DC NR intra-cell PSCell change dedicated timing information is used: the sequence starts at time T with sending of the RRCReconfiguration. T is set to 300 ms in advance of the PSCell change.

1.	NR PSCell before T:			Get PDCP count for DRBs and SRB3 (if applied).

2.	NR PCell at T:					Send RRCReconfiguration.

3.	NR PSCell at T:					Release SRB3 (if necessary) and DRBs.

4.	NR PSCell at T:					Configure RACH procedure either dedicated or C-RNTI based.

NOTE 1:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

5.	NR PSCell at T + 5ms:		(Re-)configure SRB3 (if necessary) and DRBs.

6.	NR PSCell at T + 5ms:		Restore the PDCP counts

NOTE 2:	For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

7.	NR PSCell at T + 5ms:		Re-establish security.

8.	NR PCell:					Receive RRCReconfigurationComplete.

##### 7.3.5.5.3	Sequence of NR-DC intra-PCell handover and intra-cell PSCell change

For NR-DC intra-NR intra PCell handover dedicated timing information is used: the sequence starts at time T with sending of the RRCReconfiguration. T is set to 300 ms in advance of the handover.

1.	PCell and PSCell before T:		Get PDCP count for AM DRBs and SRBs (if applied).

2.	PCell At T :						Send RRCReconfiguration.

3.	PCell and PSCell at T + 5ms:	Release SRBs and DRBs.

4.	PCell and PSCell at T + 5ms:	Configure RACH procedure either dedicated or C-RNTI based.

NOTE 1:	Since the RACH procedure may require a new C-RNTI to be used it cannot be configured before sending out the RRCReconfiguration.

NOTE 2:	The FollowOnFlag is set to true in the ASPs reconfiguring the RadioBearerList (step 3) and C-RNTI (step 4).

5.	PCell and PSCell at T + 10ms:	(Re-) configure SRBs and DRBs.

7.	PCell and PSCell at T + 10ms:	Set PDCP COUNT for AM DRBs and SRBs (if applied).

NOTE 3:	For AM DRBs the PDCP count is maintained while for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

8.	PCell and PSCell at T + 10ms:	Re-establish security, disable TA transmission.

9.	PCell (after step 8)				Receive RRCReconfigurationComplete.

10.	PCell (after step 9)				Re-configure RACH procedure as for initial access, enable TA transmissions.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring RACH.

<End of modified section>

<Start of modified section>

### 7.3.6	Bearers

#### 7.3.6.1	DRB Identity Management

If a UE is configured to establish more than one PDU session at switch on, the order in which they are requested by the UE cannot be guaranteed; and in fact may be different between successive instances of switching on in the same test case.  In order to align NR/5GC with EN-DC and E-UTRA/EPC, TTCN allocates:

-	drb-Identity 1 for the default DRB of the PDU session for IMS.

-	drb-Identity 2 onwards for other DRB(s).

### 7.3.7	Processing delay testing for RRC procedures

#### 7.3.7.1	Introduction

Test cases testing RRC procedure delay (according to TS 38.331 [16] clause 12), when UE is PUCCH synchronized and can respond to UL grant, require special test method.

#### 7.3.7.2		Procedure delays in PUCCH synchronized state

The following SS and UE configurations apply:

By default in TS 38.508-1[5] DRX is not configured.

By default SS is configured to retransmit any DL MAC PDU max 4 times.

HARQ retransmissions in UL cannot be compensated, any HARQ error in UL shall result in an inconclusive verdict for the test case (otherwise a UE may get fail due to a HARQ error).

SS is configured to report  UL HARQ ACK/NACK received from UE to TTCN.

NOTE:	Due to L2 signalling (e.g. RLC STATUS PDUs) it is necessary to limit the reporting of UL HARQ ACK/NACK to the time between sending of the DL RRC message and receiving the HARQ ACK.

To avoid unexpected side effects the timeAlignmentTimer is set to infinity and TTCN configures the SS to not send any Timing Advance Command MAC CE during the test case body (since this may result in additional UL HARQ ACK/NACK).

SS is configured to report HARQ errors and in the case of an UL HARQ error, an inconclusive verdict is assigned.

The following timing applies:

Let Nsf be the maximum allowed RRC procedure delay value (in ms) as specified in the test case prose. Nsf is the maximum time allowed for UE from the end of reception of the DL message on the UE physical layer up to when the UE shall be ready for the reception of UL grant for the transmission of the response message.

NOTE:	A tolerance may have been added to the Nsf value in the test case prose on top of the RRC procedure delay specified in TS 38.331 [16] clause 12.

Editor’s note: FFS discussion with prose author is ongoing.

Let Nslot = (Nsf * Nslotsubframe,μ) + 1

TTCN schedules at time T1 the transmission of the DL message to the UE.

T2 is the time HARQ ACK is received.

TTCN schedules UL grants from T3: T3 = T1 + Nslot + ∆1

  - ∆1 is expressed in slots. T1 and T3 timing are expressed in SFN/subframe/slots.

  - For FDD

    - ∆1 = 0

    - The UL grant is repeated continuously in every slot

  - For FR1 TDD SCS=15kHz

    - ∆1 = 5 - (T1 + Nslot) mod 5. This is the possible slot delay since not all slots can be used for UL grant.

    - The UL grant periodicity is set to 5 slots. It is repeated in subframe#0 and subframe#5, to cope with K2=4 and the UL subframe#4 and subframe#9.

  - For FR1 TDD SCS=30kHz

    - ∆1 = (4 - (T1 + Nslot) mod 10) mod10. This is the possible slot delay since not all slots can be used for UL grant.

    - The UL grant periodicity is set to 10 slots. It is repeated in slot#0 in subframe#2 and subframe#7, to cope with K2=4 and the UL slots: slot#0 in subframe#4 and subframe#9.

  - For FR2 TDD SCS=120kHz

    - ∆1 = (5- (T1 + Nslot) mod 5) mod 5. This is the possible slot delay since not all slots can be used for UL grant.

    - The UL grant periodicity is set to 5 slots. It is repeated for possible transmission in every UL slots.

The UL data is received by SS at T4.

The RRC procedure delay requirements are fulfilled when:  (T4-K2) - (T2 - K1) <= Nslot + ∆2

  - ∆2 is expressed in slots. T4 and T2 timing are expressed in SFN/subframe/slots.

  - For FDD

    - ∆2 =0

  - For FR1 TDD SCS=15kHz

    - ∆2 = 5 - (T2-K1 + Nslot) mod 5

  - For FR1 TDD SCS=30kHz

    - ∆2 = (4 - (T2-K1 + Nslot) mod 10) mod10

  - For FR2 TDD SCS=120kHz

    - ∆2 = (5- (T2-K1 + Nslot) mod 5) mod 5

Figure 7.3.7.2-1 shows the RRC procedure delay check sequence that will be applied when the DL Message is retransmitted once when UE is in PUCCH synchronized state and can respond to UL grants.

Figure 7.3.7.2-1: Delays in PUCCH synchronized state

<End of modified section>

<Start of modified section>

## 11.3	NR/5GC

### 11.3.1	NR/5GC single RAT

#### 11.3.1.1	Single NR carrier

This clause provides the guidelines for the NR/5GC test cases.

A test case using more than one radio frequency, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n51.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.13, 6.1.2.14, 6.1.2.16, 6.1.2.18, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.3.1.1, 6.3.1.2,  6.3.1.3,  6.3.1.4, 6.3.1.5, 6.3.1.8,  6.3.1.9, 6.4.1.1, 6.4.1.2, 6.4.2.2,

8.1.1.2.1, 8.1.1.3.1, 8.1.1.3.3, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9, 8.1.3.1.11, 8.1.3.1.12, 8.1.3.1.14A, 8.1.3.1.15A, 8.1.3.1.20, 8.1.3.21, 8.1.4.1.2,

9.1.4.1, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.4, 9.1.5.1.8, 9.1.5.1.10, 9.1.5.1.12, 9.1.5.1.14,

11.3.6, 11.3.8, 11.3.9,

11.4.7.

A test case using more than two radio frequencies, i.e. using the radio frequencies NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n30

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.20, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.5, 6.3.1.8, 6.4.1.1, 6.4.1.2, 6.4.2.2,

8.1.1.3.3,

9.1.5.1.2, 9.1.5.1.8.

### 11.3.2	NR/5GC Inter-RAT

#### 11.3.2.1	NR/E-UTRA Inter-RAT

This clause contains the guidelines for the NR/5GC and E-UTRA inter-RAT test cases executed on the different bands. According to TS 38.508-1 [5] clause 6.2.3.3, it is assumed that the NR and E-UTRA bands under test are not overlapping.

A test case using more than one radio frequency on E-UTRA, i.e. using the radio frequencies f2, f3 or f4 specified in TS 36.508 [10], shall not be executed on:

Band 13,

Band 18,

Band 31,

Band 72,

Band 73

The list of test cases is given below:

6.4.3.1,

8.1.1.3.4,

11.1.4.

A test case using more than one radio frequency on NR, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n51.

The list of test cases is given below:

6.2.1.1, 6.2.1.3.

A test case using more than two radio frequencies on NR, i.e. using the radio frequencies NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n30.

The list of test cases is given below:

6.2.1.1.

<End of modified section>