3GPP TSG-RAN WG5 Meeting #90-e	R5-211139r1

# Electronic Meeting, 22nd February – 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0926 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of configurations not to be tested in 4CC refsens test case 7.3B.2.3_1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Table 7.3B.2.3_1.2.4.1-0 indicating 4CC EN-DC configurations that shall not be tested is wrong and not aligned with the content of TR38.905. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 7.3B.2.3_1.2.4.1-0 corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on which configurations need to be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3_1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-211139r1: - Removing conflict with R5-210738 - Correction of table references in test requirements |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3_1.2	Reference sensitivity for EN-DC within FR1 (4 CCs)

###### 7.3B.2.3_1.2.1	Test purpose

Same as in clause 7.3B.2.3.1.

###### 7.3B.2.3_1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 CCs EN-DC.

###### 7.3B.2.3_1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

For EN-DC combinations with no exception requirements applicable to NR or LTE, LTE anchor agnostic approach is applied.

For EN-DC combinations with exceptional requirements, LTE anchor agnostic approach is not applied.

###### 7.3B.2.3_1.2.4	Test description

7.3B.2.3_1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.2.1-1. The 4CC EN-DC configurations listed in table 7.3B.2.3_1.2.4.1-0 shall not be tested according to TR 38.905 test point analysis. The other 4CC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-1 for NR band. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3_1.2.4.1-0: 4CC EN-DC configurations that shall not be tested

| EN-DC type |  | E-UTRA CA | NR CA | Notation |
| --- | --- | --- | --- | --- |
| Intra-band non-contiguous EN-DC (1 band) |  | Yes (cont) | No | DC_XD_nXA |
|  |  | Yes (non-cont) | No | DC_XA-XC_nXA |
| Inter-band EN-DC | Yes (all types) | No | DC_XD-nYA, DC_XA-YC_nZA, DC_XA-XA-YA-nZA, DC_XA-YA-ZA_nRA |
|  | Yes (all types) | Yes (cont) | DC_XC_nYC, DC_XA-XA_nYC, DC_XA-YA_nYC |
|  |  | Yes (cont) | DC_XC_nY(2A), DC_XC_nY(2A), DC_XA-YA_nY(2A) |
|  |  | Yes (inter) | DC_XC-nYA-nZA, DC_XA-XA_nYA-nZA, DC_XA-YA_nZA-nRA |
| Intra-band contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA_(n)ZAA, DC_XC_(n)YAA |
|  | Yes | Yes | DC_(n)XCA-nYA |
| Intra-band non contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA-ZA_nZA |
|  | Yes | Yes | DC_XA-YA_nYA-nZA |
| NOTE 1: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 2: The table apply to all band numbers, i.e. all values of X, Y and Z defined in TS38.101-3 NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. |  |  |  |

Table 7.3B.2.3_1.2.4.1-1: Test Configuration Table for EN-DC configurations not affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | DC_(n)XDA: Low range, High Range DC_XA-nYA-nZC: Mid range for X Low range, High Range for nY and nZ DC_XA-nY(2A)-nZA: Mid range for X TBD for nY Mid range for nZ |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 (Note 6) | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | X |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 (Note 7) | PCC(M) | X |  | Highest NRB | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | X |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nY(2A)-nZA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nYA-nZC Configuration– Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. Note 4: LTE anchor agnostic configuration Note 5: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 6: Test point for UE supporting dual UL Note 7: Test point for UE supporting single UL |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.2.4.1-2: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | DC_XA-nYA-nZC: Mid range for X Mid Range for nY and nZ DC_XA-nY(2A)-nZA: Mid range for X TBD for nY Mid range for nZ |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N/A |  |  |  |  |  |
| Default Test Settings for a DC_XA-nY(2A)-nZA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nYA-nZC Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: No refsens exceptions with this EN-DC configuration exist in TS38.101-3 NOTE 5: This test setting is valid for EN-DC configurations affected by UL harmonics, harmonic mixing or cross band isolation exceptions when the exception is avoided by selecting specific test frequency. Test only one of the test points. UL to be active on the cell that form the aggressor of the exception NOTE 6: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. NOTE 7: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.2.4.1-3: Void

7.3B.2.3_1.2.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2

7.3B.2.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

###### 7.3B.2.3_1.2.5	Test Requirement

Reference sensitivity test requirements for EN-DC configurations affected by 2UL intermodulation interference in NR FR1, are specified in Table 7.3B.2.3_1.2.5-1 and Table 7.3B.2.3_1.2.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-2.

Reference sensitivity test requirements for EN-DC configurations not affected by 2UL intermodulation interference in NR FR1, are specified in Table 7.3.5-1 in TS 36.521-1 [10] for the LTE CC, and Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.2.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

FFS

Table 7.3B.2.3_1.2.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

FFS

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

##### 7.3B.2.3_1.3	Reference sensitivity for EN-DC within FR1 (5 CCs)

7.3B.2.3_1.3.1	Test purpose

Same as in clause 7.3B.2.3.1.

7.3B.2.3_1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5 CCs inter-band EN-DC.

7.3B.2.3_1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.3.4	Test description 7.3B.2.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.3.4.1-1 and Table 7.3B.2.3_1.3.4.1-2

Table 7.3B.2.3_1.3.4.1-1: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (four bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA_ZC_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.3.4.1-2: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (five bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA-ZA-SA_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | S | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different E-UTRA bands i.e. X != Y != Z, R corresponds to NR band. NOTE 3: For inter-band EN-DC 4CCs configuration with four bands, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1, testing for 4CC with four band can be skipped. |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.3.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

7.3B.2.3_1.3.5	Test requirement

For inter-band EN-DC configurations, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Tables Table 7.3.2.5-1 in TS 38.521-1 [8] and parameters specified Tables 7.3.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3 in TS 38.521-1 [8] for NR band.

Each EN-DC combination defined in Table 5.5B.4.3-1 shall be tested in ancher-agnoutic anchor-agnostic mode as described in this clause. If a test point is overlapped with uplink EN-DC combo MSD test points, the requirement with MSD shall apply.

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3_1.3.5-1 for four bands and Table 7.3B.2.3_1.3.5-2 for five bands with uplink configuration specified in Table 7.3B.2.3_1.3.4.1-1 for four bands and 7.3B.2.3_1.3.4.1-2 for five bands, respectively.

For a given inter-band EN-DC 5CCs configuration defined in Table 5.5B.4.3-1 and Table 5.5B.4.4-1, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1 and Table 5.5B.4.4-1,  the test requirement for default fallback uplink EN-DC configurations defined in Table 5.5B.4.3-1 applies.

Table 7.3B.2.3_1.3.5-1 Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (four bands)