3GPP TSG-RAN WG5 Meeting #91-e	R5-212293

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0133 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Generic Test Procedure 5.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Generic Test Procedure for MCPTT CT group call establishment, manual commencement does not cover pre-established session case |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Exception to cover pre-established session case added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong procedures result in incorrect MCPTT test case implementation and test case verification not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.5	Generic Test Procedure for MCPTT CT group call establishment, manual commencement

5.3.5.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.5.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.5.3	Procedure

Table 5.3.5.3-1: MCPTT CT group call establishment, manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1b1 describe behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.4 'Generic Test Procedure for MCPTT CT communication in E-UTRA' take place. | - | - | - | - |
| 1b1 | IF in RRC_IDLE state having a pre-established session, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.13 Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session' take place. | - | - | - | - |
| 2 | The SS (MCPTT Server) sends an initial SIP INVITE requesting the establishment of an MCPTT group call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCPTT client) sends SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| 4 | The SS starts timer Timer_1 = 5 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 5a1 to 5c1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that may take place if the UE responds reliably or unreliably to a SIP INVITE with a SIP 183 (Session Progress) | - | - | - | - |
| 5a1 | Check: Does the UE (MCPTT client) send SIP 183 (Session Progress) unreliably? | --> | SIP 183 (Session Progress) | - | P |
| 5a2 | The SS stops Timer_1. | - | - | - | - |
| 5b1 | Check: Does the UE (MCPTT client) send SIP 183 (Session Progress) reliably? | --> | SIP 183 (Session Progress) | - | P |
| 5b2 | The SS stops Timer_1. | - | - | - | - |
| 5b3 | The SS (MCPTT Server) acknowledges the receipt of SIP 183 (Session Progress) | <-- | PRACK | - | - |
| 5b4 | The UE (MCPTT Client) responds PRACK with SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 5c1 | Check: Does Timer_1 expire? | - | - | - | P |
| 5A | Check: Does the UE (MCPTT client) notify the User of the incoming call request? (NOTE 1) | - | - | - | P |
| 6 | Make UE (MCPTT User) accept the call (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCPTT client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 8 | The SS (MCPTT server) sends a SIP ACK to acknowledge the session establishment | <-- | SIP ACK | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.5.4	Specific message contents

All message contents are as specified in clause 5.5 with condition GROUP-CALL where applicable and with the following clarifications:

Table 5.3.5.4-1: SIP INVITE (step 2, Table 5.3.5.3-1)

Derivation Path: Table 5.5.2.5.2-1 with condition MANUAL and GROUP-CALL and MCPTT

Table 5.3.5.4-1A: SIP 183 (Session Progress) (step 5a1, Table 5.3.5.3-1)

Derivation Path: Table 5.5.2.16.3.1-1 with condition MCPTT

Table 5.3.5.4-2: SIP 183 (Session Progress) (step 5b1, Table 5.3.5.3-1)

Derivation Path: Table 5.5.2.16.3.1-1 with condition 100rel and MCPTT

Table 5.3.5.4-3: SIP 200 (OK) (step 7, Table 5.3.5.3-1)

Derivation Path: Table 5.5.2.17.1.1-1 with condition INVITE-RSP and MCPTT