3GPP TSG-RAN5 Meeting #91-e	R5-212484

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1851 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IEs of MANAGE UE POLICY COMMAND msg |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Some of the values is still FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Replace the FFS with values of common configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Common configuration of NR V2X TCs is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4, 4.7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	MANAGE UE POLICY COMMAND

Table 4.7.4-1: MANAGE UE POLICY COMMAND

| Derivation Path: TS 24.501 Table D.5.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| MANAGE UE POLICY COMMAND message identity |  | ’0000 0001’B | MANAGE UE POLICY COMMAND message |  |
| UE policy section management list |  | See Table 4.7.5.1-1 |  |  |

<End of modified section 1>

<Start of modified section 2>

4.7.5.1	V2X information elements of MANAGE UE POLICY COMMAND message

–	UE policy section management list

Table 4.7.5.1-1: UE policy section management list

| Derivation Path: TS 24.501 Figure D.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management list IEI |  | Not Present | This IE is LV-E type in MANAGE UE POLICY COMMAND message |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  | See Table 4.7.5.1-2 |  |  |

–	UE policy section management list contents

Table 4.7.5.1-2: UE policy section management list contents

| Derivation Path: TS 24.501 Figure D.6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management sublist (PLMN-1) |  | See Table 4.7.5.1-3 |  |  |
| UE policy section management sublist (PLMN-2) |  | Not Present |  |  |

–	UE policy section management sublist

Table 4.7.5.1-3: UE policy section management sublist

| Derivation Path: TS 24.501 Figure D.6.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management sublist contents |  | See Table 4.7.5.1-4 |  |  |

–	UE policy section management sublist contents

Table 4.7.5.1-4: UE policy section management sublist contents

| Derivation Path: TS 24.501 Figure D.6.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Instruction 1 |  | See Table 4.7.5.1-5 |  |  |
| Instruction 2 |  | Not Present |  |  |

–	Instruction

Table 4.7.5.1-5: Instruction

| Derivation Path: TS 24.501 Figure D.6.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UPSC |  | '00 00 00 01'H | 2 bytes, value set by PCF |  |
| UE policy section contents |  | See Table 4.7.5.1-6 |  |  |

–	UE policy section contents

Table 4.7.5.1-6: UE policy section contents

| Derivation Path: TS 24.501 Figure D.6.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part 1 |  | See Table 4.7.5.1-7 |  |  |
| UE policy part 2 |  | Not Present |  |  |

–	UE policy part

Table 4.7.5.1-7: UE policy part

| Derivation Path: TS 24.501 Figure D.6.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B |  | URSP |
|  |  | ‘0010’B |  | ANDSP |
|  |  | ‘0011’B |  | V2XP |
| UE policy part contents |  | See Table 4.7.5.4-1 |  | V2XP |

<End of modified section 2>