3GPP TSG-RAN5 Meeting #91-e	R5-212907

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1192 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DRX reference configuration in Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, In Reference DRX configuration DRX.10, TAT is set to 500ms, which is even shorter than DRX cycle (640ms). So it is impossible for SS to send MAC-CE in DRX on duration to keep UL synchonization.As a result UE has to perform RACH procedure to resume UL sync. Then conformant UE may fail the test due to the additional delay caused by unintended RACH procedure. 2. Furthermore, inactivity timer in DRX.9 and DRX.10 (100ms and 1920ms respectively) is longer than DRX cycle. In test SS shall provide UL grant at least every DRX cycle to UE, otherwise additional delay will be introduced due to lack of UL resource for measurement report. According to 36.321 cl.5.7, it means that drx-InactivityTimer will be restarted every 40ms/640ms TS 321 cl.5.7: - if the PDCCH indicates a new transmission (DL, UL or SL): - except for an NB-IoT UE configured with a single DL and UL HARQ process and when PDCCH indicates the transmission is not for multiple TBs: - start or restart drx-InactivityTimer.. However, according to 36.133 cl.5: Note 1: For the performance requirements specified hereafter, the state when no DRX is used is defined as follows: - DRX and eDRX_CONN parameters are not configured; or - DRX or eDRX_CONN parameters are configured and ○ drx-InactivityTimer is running; or ○ drx-RetransmissionTimer is running; or ○ mac-ContentionResolutionTimer is running; or ○ a Scheduling Request sent on PUCCH/SPUCCH is pending; or ○ an uplink grant for a pending HARQ retransmission can occur and there is data in the corresponding HARQ buffer; or ○ a PDCCH/SPDCCH indicating a new transmission addressed to the C-RNTI of the UE has not been received after successful reception of a Random Access Response for the explicitly signaled preamble (only applicable to UEs in RRC_CONNECTED). Otherwise - It is the state when DRX is used.. Then it is impossible to verify inter-RAT measurement delay requirements with DRX since UE will always apply requirements corresponding to "no DRX" |
| Summary of change: |  | - Change TAT for DRX.10 to infinity. - Change inactivity timer for DRX.10 to psf2, i.e. 2ms,which is the configuration value used in DRX.4. - Update the configuration of DRX.9/10 in Annex H.3.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5, H.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

A.5	Reference DRX configurations

The reference DRX configurations for the NR serving cell are captured in Table A.5-1. The reference DRX configurations for the E-UTRA serving cell for NSA and inter-RAT test cases are captured in Table A.5-2.

Table A.5-1: DRX configurations for NR serving cell

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.1 | DRX.2 | DRX.3 | DRX.6 | DRX.7 | DRX.8 | DRX.11 |
| drx-onDurationTimer | ms | 1 | 1 | 6 | 1 | 6 | 6 | 6 |
| drx-InactivityTimer | ms | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerDL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerUL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-LongCycleStartOffset | ms | 40 | 640 | 40 | 320 | 640 | 320 | 20 |
| shortDRX | - | disabled | disabled | disabled | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | 500 | 500 | Infinity | 500 | Infinity | Infinity | Infinity |
| Note 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |  |  |

Table A.5-2: DRX configurations for E-UTRA serving cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.4 | DRX.5 | DRX.9 | DRX.10 |
| drx-onDurationTimer | ms | psf2 | psf6 | psf2 | psf6 |
| drx-InactivityTimer | ms | psf2 | psf1920 | psf2 | psf2 |
| drx-RetransmissionTimerDL | slot | psf16 | psf16 | psf16 | psf16 |
| drx-LongCycleStartOffset | ms | sf160, 0 | sf320. 0 | sf40, 0 | sf640. 0 |
| shortDRX | - | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | Infinity | Infinity | 500 | Infinity |
| Note 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 36.331 [29] |  |  |  |  |  |  |

<<Unchanged skipped>>

MAC-MainConfig: DRX configuration for E-UTRAN serving cell

Table H.3.7-2: MAC-MainConfig

| Derivation Path: 36.508, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig ::= SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf2 |  | DRX.4 OR DRX.9 |
|  |  | psf6 |  | DRX.5 OR DRX.10 |
| drx-InactivityTimer |  | psf2 |  | DRX.4 OR DRX.9 OR DRX.10 |
|  |  | psf1920 |  | DRX.5 |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf160 |  | 0 |  | DRX.4 |
| sf320 |  | 0 |  | DRX.5 |
| sf40 |  | 0 |  | DRX.9 |
| sf640 |  | 0 |  | DRX.10 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  | DRX.4 OR DRX.5 OR DRX.10 |
|  |  | sf500 |  | DRX.9 |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.4 | DRX Configuration 4 according to TS 38.133 [6] A.3.3.4 |
| DRX.5 | DRX Configuration 5 according to TS 38.133 [6] A.3.3.5 |
| DRX.9 | DRX Configuration 9 according to TS 38.133 [6] A.3.3.9 |
| DRX.10 | DRX Configuration 10 according to TS 38.133 [6] A.3.3.10 |

<End of modified section>