3GPP TSG-RAN5 Meeting #91-e	R5-213276

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2269 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR V2X test case 12.1.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR V2X TC 12.1.7.2 “PC5-only operation / Sidelink UE capability transfer via PC5 RRC / Peer UE side”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.7, 12.1.7.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR XXXX |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 12	NR sidelink

## 12.1	PC5-only operation

…

### <Start of modified section>

#### 12.1.7.2	PC5-only operation  / Sidelink UE capability transfer via PC5 RRC / Peer UE side

12.1.7.2.1	Test Purpose (TP)

(1)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE receives a UECapabilityEnquirySidelink message from peer UE }

then { UE sends a UECapabilityInformationSidelink message to peer UE }

}

12.1.7.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.9.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.9.2.4]

The peer UE shall set the contents of UECapabilityInformationSidelink message as follows:

1>	include UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink;

1>	compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilterSidelink, and prioritized in the order of frequencyBandListFilterSidelink (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on).

1>	include into supportedBandCombinationListSidelinkNR as many band combinations as possible from the list of "candidate band combinations", starting from the first entry;

1>	include the received frequencyBandListFilterSidelink in the field appliedFreqBandListFilter of the requested UE capability;

1>	submit the UECapabilityInformationSidelink message to lower layers for transmission.

NOTE:	If the UE cannot include all band combinations due to message size or list size constraints, it is up to UE implementation which band combinations it prioritizes.

12.1.7.2.3	Test description

12.1.7.2.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1

-	System information combination [FFS] as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cell 1.

-	SS-UE

-	Operating as NR sidelink communication receiving device on the resources (i.e. the frequency included in sl-FreqInfoList in SIB12 of the PCell which is NR Cell1) that UE is expected to use for transmission.

UE:

- UE is authorised to perform NR sidelink communication.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1 and PC5 RRC connection is established with SS-UE … and Test Loop Function (On) with UE test loop mode [FFS] defined in 38.509 [6], subclause [FFS].

12.1.7.2.3.2	Test procedure sequence

[Editor’s Note: For the below test procedure sequence it is assumed that the initiating UE side (= UE under test) as well as the peer UE (= SS-UE) are both in coverage.]

Table 12.1.7.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE receives UECapabilityEnquirySidelink message from SS-UE on SL-SRB3 | <-- | NR RRC: UECapabilityEnquirySidelink- | - | - |
| 2 | Check: Does the UE send a UECapabilityInformationSidelink message ? | --> | NR RRC: UECapabilityInformationSidelink | 1 | P |

12.1.7.2.3.3	Specific message contents

Table 12.1.7.2.3.3-1: UECapabilityInformationSidelink (step 2, Table 12.1.7.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ueCapabilityInformationSidelink-r16 SEQUENCE { |  |  |  |
| accessStratumReleaseSidelink-r16 | rel16 |  |  |
| pdcp-ParametersSidelink-r16 | Checked |  |  |
| rlc-ParametersSidelink-r16 | Checked |  |  |
| supportedBandCombinationListSidelinkNR-r16 | Checked |  |  |
| supportedBandListSidelink-r16 | Checked |  |  |
| appliedFreqBandListFilter-r16 | Checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

### <End of modified section>