Title: 	Discussion on test point selection for TX UE co-existence spurious emission for inter-band EN-DC within FR1

Introduction

Generic initial test conditions, such as environmental conditions, test frequencies, channel bandwidths, subcarrier spacing (SCS) were discussed in previous RAN5 presentation [1] [2]. The general agreement is that FR1 testing mostly follows similar requirement as for LTE. A recent paper [3] also referenced to an agreed common uplink configuration table for all TX test cases.

In another RAN5 paper (R5-185892), it is proposed that NR test point selection shall follow what is specified in SA scenarios.

The purpose of this paper is to discuss test point selection of E-UTRA and NR carriers for UE co-existence spurious emission measurement for inter-band EN-DC within FR1. Considering there are so many EN-DC combos expected to be supported, it is reasonable to carefully investigate the possibility of reducing test points. Since the objective of 38.521-3 specification is provide conformance testing requirement for NR in the case of EN-DC scenarios, the number of LTE test points can be reduced.

Discussion

From TS 38.521-1, the initial conditions used for NR general/UE co-ex spurious emission is given below:

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest supported SCS per test channel BW, |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| 2 |  | CP-OFDM QPSK | Outer_1RB_Left |
| 3 |  | CP-OFDM QPSK | Outer_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration |  |  |  |

Test Environment

Same Test Environment as for standalone NR testing should be selected.

Proposal 1: Propose Test Environment as Normal for spurious emission UE co-existence test cases for EN-DC with FR1.

Test Frequencies selection

Generally, both Low range and High range shall be selected for both NR carrier and E-UTRA carrier for inter-band EN-DC spurious emission UE co-existence. However, the test frequency selection for inter-band EN-DC shall take the intermodulation products caused by E-UTRA carrier and NR carrier into consideration, which is different from NR standalone testing.

To ensure that intermodulation products could fall into all protected bands, the test configuration should be analysed per EN-DC configuration. For each inter-band EN-DC configuration, Low, Mid and High range shall be used flexibly by both NR carrier and E-UTRA carrier to produce the desired intermodulation products. For some EN-DC configurations, a specified test frequency may need to be calculated so that the desired intermodulation products occur.

Proposal 2: Low range and High range frequency shall be selected by default for E-UTRA carrier and NR carrier to test UE co-existence spurious emission.

Proposal 2a: Additionally, Low, Mid and High range frequency shall be used flexibly by both NR carrier and E-UTRA carrier to produce the desired intermodulation products. But for some EN-DC configurations, a specified test frequency may need to be calculated so that the desired intermodulation products occur.

Test Channel Bandwidth selection

Generally, the highest channel bandwidth supported by E-UTRA carrier and NR carrier shall be selected for inter-band EN-DC UE co-existence spurious emission test case. But for some EN-DC configurations, a specified test channel bandwidth may be needed in order to test the desired intermodulation products.

Proposal 3: Highest channel bandwidth shall be selected by default for E-UTRA carrier and NR carrier. But for some EN-DC configurations, a specified test channel bandwidth may be needed in order to test the desired intermodulation products.

RB allocation and RB location selection

For inter-band EN-DC, 1 RB allocation for E-UTRA carrier and 1 RB allocation for NR carrier shall be selected for UE co-existence spurious emission. For default test points, 1 RB at the left edge of both carriers shall be used for Low range frequency, and 1 RB at the right edge of both carriers shall be used for High range frequency.

For the purpose of covering all protected bands, additional 1 RB location for E-UTRA carrier and 1 RB location for NR carrier shall be selected flexibly in order to test the desired intermodulation products.

Proposal 4: 1 RB allocation for E-UTRA carrier and 1 RB allocation for NR carrier shall be selected for inter-band UE co-existence spurious emission.

Proposal 4a: For default test points, 1 RB at the left edge of both carriers shall be used for Low range frequency, and 1 RB at the right edge of both carriers shall be used for High range frequency. Additional 1 RB location for E-UTRA carrier and 1 RB location for NR carrier shall be selected flexibly in order to test the desired intermodulation products.

Modulation selections

For LTE and NR SA spurious emissions measurement, the modulation QPSK and CP-OFDM are used for the uplink modulation, respectively.

For inter-band EN-DC, there is no particular reason to deviate from the selected modulation in LTE and NR SA scenarios.

Proposal 5: Propose the same modulation scheme as for LTE and NR SA testing for E-UTRA carrier and NR carrier for EN-DC spurious emission test cases.

Number of Test Points for each EN-DC combo

Table 2.6-1 Maximum Number of Test Points for UE co-existence spurious emission for each EN-DC combo

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of RB allocations | Maximum Number of Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 1 | 1 | 2 |
| Note: The maximum number of test point is 2 if only default points are applied. |  |  |  |  |  |

Conclusion

In conclusion, the following test points selection are proposed for inter-band EN-DC in UE coexistence spurious emission measurement:

Proposal 1: Propose Test Environment as Normal for spurious emission UE co-existence test cases for EN-DC with FR1.

Proposal 2: Low range and High range frequency shall be selected by default for E-UTRA carrier and NR carrier to test UE co-existence spurious emission.

Proposal 2a: Additionally, Low, Mid and High range frequency shall be used flexibly by both NR carrier and E-UTRA carrier to produce the desired intermodulation products. But for some EN-DC configurations, a specified test frequency may need to be calculated so that the desired intermodulation products occur.

Proposal 3: Highest channel bandwidth shall be selected by default for E-UTRA carrier and NR carrier. But for some EN-DC configurations, a specified test channel bandwidth may be needed in order to test the desired intermodulation products.

Proposal 4: 1 RB allocation for E-UTRA carrier and 1 RB allocation for NR carrier shall be selected for inter-band UE co-existence spurious emission.

Proposal 4a: For default test points, 1 RB at the left edge of both carriers shall be used for Low range frequency, and 1 RB at the right edge of both carriers shall be used for High range frequency. Additional 1 RB location for E-UTRA carrier and 1 RB location for NR carrier shall be selected flexibly in order to test the desired intermodulation products.

Proposal 5: Propose the same modulation scheme as for LTE and NR SA testing for E-UTRA carrier and NR carrier for EN-DC spurious emission test cases.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v15.0.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.521-1 v0.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] 3GPP TR 38.817-01 v0.3.0 General aspects for UE RF for NR (Release 15)

[5] R5-181895, “TP for adding Mid channel BW definition in TS 38.508-1”

[6] R5-180886, “Discussion_NR_FR1_TestPoints_For_SEM”

[7] R5-191451r1, “Resolving ambiguity in LTE anchor agnostic approach”