3GPP TSG-RAN WG5 Meeting #91-e	R5-21abcd

# Electronic Meeting, 17th May – 28th May 2021

Agenda Item:	TTCN sidebar

Source:	MCC TF160

Title:	UL Grant settings for IMS-capable LTE UEs

Document for:	Discussion and endorsement

# 1	Introduction

TTCN CR R5s210279 from Anritsu raised the issue of IMS-capable CatM1 UE failing LTE/CatM1 test cases in the preamble due to SIP timer expiry in UE when trying to send the REGISTER message for IMS registration. This issue is addressed in the present paper.

# 2	Current UL Grant settings in TTCN

Table 2-1 summarizes the current UL Grant settings used in the TTCN implementation of the LTE test suites.

Table 2-1: UL Grant settings

| UE Category | CE mode A TC | Grant size type | TBS {Imcs ; Nprb} | UL Grant Allocation | Total Grant size (bits) |
| --- | --- | --- | --- | --- | --- |
| CatM1 | n/a | default | 1000 {14 ; 4} | Type1 | 1000 |
|  | n/a | BigGrant | 1000 {14 ; 4} | Type5 {20ms ; 10x} | 10000 |
| Cat0 | n/a | default | 1000 {14 ; 4} | Type1 | 1000 |
|  | n/a | BigGrant | 1000 {14 ; 4} | Type5 {20ms ; 10x} | 10000 |
| Cat1/1bis | n/a | default | 4008 {9 ; 25} | Type1 | 4008 |
|  | n/a | BigGrant | 4968 {12 ; 25} | Type5 {20ms ; 2x} | 9936 |
| CatX with X≥2 | yes | default | 1000 {14 ; 4} | Type1 | 1000 |
|  |  | BigGrant | 1000 {14 ; 4} | Type5 {20ms ; 10x} | 10000 |
|  | no | default | 4008 {9 ; 25} | Type1 | 4008 |
|  |  | BigGrant | 10680 {20 ; 25} | Type1 | 10680 |

Note: For CE mode B only TC 7.1.4.24c, other settings are used.

As per TS 36.523-3 subclause 7.2, BigGrant (USE_BIG_GRANTS) is applied “for test cases with IMS signalling within the test body, unless explicitly specified in test”.

In practice, Table 2-2 shows which test cases use BigGrant in TTCN.

Table 2-2: Test cases using BigGrant

| 34.229-1 | 36.523-1 |
| --- | --- |
| all TCs | all IMS emergency TCs (including eCall) |
|  | all SRVCC TCs |
|  | all SSAC & SCM TCs |
|  | 9.2.1.1.28, 9.2.1.1.28a, 9.2.1.1.28b |
|  | 10.4.2 |

# 3	Issue with IMS REGISTER

The issue was described by Anritsu as follows:

UE fails to sends the REGISTER message within 10 seconds(t_WaitForUPlaneSignalling) expiry especially when there is a retransmission of first IP Packet of REGISTER.

Let us say REGISTER message size is 1800 bytes where IP MTU size configured as 1400.

> REGISTER message segmented and sent in two IP packets: first with 1400 bytes and second with 450 (including IP headers)

>> First IP packet: 1400 *8(11200 bits) leads to 12 segments at RLC (due to 1000 TB size for CAT-M1)

>>> Time difference between each RLC segment would be 320ms (BSR  timer expiry).

>>>Total time to send first IP packet 3.8 seconds ( 12 RLC segments * BSR timer)

>> TCP ACK will sent after 3.8 seconds, but, it can take another 400ms to reach UE.

>>>So, overall time can take more than 4 seconds( “3.8 + 0.4” > 4 seconds). This leads to retransmission of first IP Packet again.

>> If first IP packet retransmitted, then, it will be another 3.8 seconds.

(t_WaitForUPlaneSignalling)

DelayinUE-ToTriggerICMPv6 + ICMPv6 signalling + TCP Socket Connection + First IP Packet + TCP ACK for first IP Packet + retransmission of first IP Packet + Second IP Packet + TCP ACK For second IP Packet

0.3seconds           +          0.3 seconds   +  0.3 seconds                    + 3.8 seconds      + 0.3 seconds                          + 3.8 seconds                                    + 1.2 second               + 0.3 seconds

è               Overall delay more than 10 seconds

As per RFC 3261, maximum time UE can wait is 4 seconds for non-INVITE requests.

Timer    Value            Section               Meaning

----------------------------------------------------------------------

T1       500ms default    Section 17.1.1.1     RTT Estimate

T2       4s               Section 17.1.2.2     The maximum retransmit

interval for non-INVITE

requests and INVITE

response

# 4	Resolution

Proposal1: For a CatM1 or Cat0 UE, when this UE also supports IMS, then use BigGrant.

From Table 2-1, it means using the yellow highlighted settings:

Proposal2: For a CatX UE with X≥2, when this UE also supports IMS and the test case is a CE mode A test case, then use BigGrant.

From Table 2-1, it means using the green highlighted settings:

Table 2-1: UL Grant settings

| UE Category | CE mode A TC | Grant size type | TBS {Imcs ; Nprb} | UL Grant Allocation | Total Grant size (bits) |
| --- | --- | --- | --- | --- | --- |
| CatM1 | n/a | default | 1000 {14 ; 4} | Type1 | 1000 |
|  | n/a | BigGrant | 1000 {14 ; 4} | Type5 {20ms ; 10x} | 10000 |
| Cat0 | n/a | default | 1000 {14 ; 4} | Type1 | 1000 |
|  | n/a | BigGrant | 1000 {14 ; 4} | Type5 {20ms ; 10x} | 10000 |
| Cat1/1bis | n/a | default | 4008 {9 ; 25} | Type1 | 4008 |
|  | n/a | BigGrant | 4968 {12 ; 25} | Type5 {20ms ; 2x} | 9936 |
| CatX with X≥2 | yes | default | 1000 {14 ; 4} | Type1 | 1000 |
|  |  | BigGrant | 1000 {14 ; 4} | Type5 {20ms ; 10x} | 10000 |
|  | no | default | 4008 {9 ; 25} | Type1 | 4008 |
|  |  | BigGrant | 10680 {20 ; 25} | Type1 | 10680 |

Notes:

- No change proposed for Cat1/1bis since default grant size is same as CatX UE with X≥2.

- It may seem a bit odd to have bigger grant size for IMS-capable CatM1 UE than for IMS-capable CatX UE with X≥2.

# 5	E-UTRA ASP semantic updates

Some semantic description gives the impression that some UL Grant Allocation type cannot be used for certain type of test cases, while there is no hard restriction in practice. It is better to remove the ambiguity and the misalignment with TS 36.523-3 subclause 7.2:

type union UL_GrantConfig_Type { Null_Type OnSR_Reception, /* SS transmits UL Grant as configured by DciInfoUL_Type at every reception of SR; */ PeriodicGrant_Type Periodic, /* SS transmits UL Grant as configured by DciInfoUL_Type periodically; * MAC tests testing Grants might set the period as infinite and num grant as 1 */ PeriodicGrant_Type PeriodicOnSR_Reception, /* SS transmits UL Grant as configured by DciInfoUL_Type periodically; the periodic grant transmission * is started/restarted on reception of a SR from UE * to enable large UL data transmission for lower category UEs (Cat<=1) */ Null_Type None /* disable any grant transmission */ };