3GPP TSG-RAN WG5 Meeting #92-e	R5-214562

Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0148 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of assistance data information elements for Multi-RTT, DL-AoD and DL-TDOA positioning methods |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2021-7-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN#90-e, WI of UE Conformance Test Aspects - NR Positioning Support has been approved, related contents should be introduced into 37.571-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify the values in Table 8.4.1.6-1: NR-DL-PRS-AssistanceData, Table 8.4.1.6-2: NR-SelectedDL-PRS-IndexList, Table 8.4.1.6-3: NR-DL-PRS-Info and Table 8.4.1.7-1: NR-PositionCalculationAssistance. - Corrected the values in NR-Multi-RTT-RequestLocationInformation and NR DL-TDOA REQUEST LOCATION INFORMATION |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The assistance data elements used for protocol conformance test cases for NR positioning in clause 9 are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

-	NR Multi-RTT REQUEST LOCATION INFORMATION

Table 8.4-5: NR-Multi-RTT-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.12.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 1 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-Multi-RTT-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| } |  |  |  |

-	NR DL-AoD REQUEST LOCATION INFORMATION

Table 8.4-6: NR-DL-AoD-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.11.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-AoD-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

-	NR DL-TDOA REQUEST LOCATION INFORMATION

Table 8.4-7: NR-DL-TDOA-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.10.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 1 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

8.4.1.6	Multi-RTT Assistance Data Elements

This clause defines the Multi-RTT assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages.

-	NR DL-PRS ASSISTANCE DATA

Table 8.4.1.6-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 1 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Sub-tests 20 and 21 only; and as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] SEQUENCE { |  | entry 3 | Sub-test 21 only as defined in clause 8.2.11 |
| dl-PRS-ID-r16 | 3 |  |  |
| nr-PhysCellID-r16 | 4 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 4 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 4 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 4 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 4 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 3 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 3 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Sub-test 21 only as defined in clause 8.2.11 |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| mnc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 3 Identifier defined in TS 38.508-1 [30] Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF NR-SSB-Config-r16 { | 2 entries |  |  |
| NR-SSB-Config-r16[1] SEQUENCE { |  | entry 1 | Sub-test 21 only as defined in clause 8.2.11 |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency | As defined in 38.508 -1 [30] subclause 6.2.3. |  |
| ss-PBCH-BlockPower-r16 | 0 |  |  |
| halfFrameIndex-r16 | 0 |  |  |
| ssb-periodicity-r16 | ms20 |  |  |
| ssb-PositionsInBurst-r16 | Not present |  |  |
| ssb-SubcarrierSpacing-r16 | Subcarrier spacing of SSB for NR Cell 3 |  |  |
| sfn-SSB-Offset-r16 | 0 |  |  |
| } |  |  |  |
| NR-SSB-Config-r16[2] SEQUENCE { |  | entry 2 | Sub-test 21 only as defined in clause 8.2.11 |
| nr-PhysCellID-r16 | 4 |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 4 frequency | As defined in 38.508 -1 [30] subclause 6.2.3. |  |
| ss-PBCH-BlockPower-r16 | 0 |  |  |
| halfFrameIndex-r16 | 0 |  |  |
| ssb-periodicity-r16 | ms20 |  |  |
| ssb-PositionsInBurst-r16 | Not presnet |  |  |
| ssb-SubcarrierSpacing-r16 | Subcarrier spacing of SSB for NR Cell 4 |  |  |
| sfn-SSB-Offset-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	NR-SelectedDL-PRS-IndexList

Table 8.4.1.6-2: NR-SelectedDL-PRS-IndexList

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-SelectedDL-PRS-IndexList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-SelectedDL-PRS-PerFreq-r16 { | 2 entries |  |  |
| NR-SelectedDL-PRS-PerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-SelectedDL-PRS-FrequencyLayerIndex-r16 | 0 |  |  |
| nr-SelectedDL-PRS-IndexListPerFreq-r16 ::= SEQUENCE (SIZE (1.. nrMaxTRPsPerFreq-r16)) OF NR-SelectedDL-PRS-IndexPerTRP-r16 { | 3 entries |  |  |
| NR-SelectedDL-PRS-IndexPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-SelectedTRP-Index-r16 | 0 |  |  |
| dl-SelectedPRS-ResourceSetIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxSetsPerTrp-r16)) DL-SelectedPRS-ResourceSetIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceSetIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceSetIndex-r16 | 0 |  |  |
| dl-SelectedPRS-ResourceIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxResourcesPerSet-r16)) OF DL-SelectedPRS-ResourceIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceIdIndex-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-SelectedDL-PRS-IndexPerTRP-r16[2] SEQUENCE { |  | entry 2 | Sub-tests 20 and 21 only; and as defined in clause 8.2.10 and clause 8.2.11. |
| nr-SelectedTRP-Index-r16 | 1 |  |  |
| dl-SelectedPRS-ResourceSetIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxSetsPerTrp-r16)) OF DL-SelectedPRS-ResourceSetIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceSetIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceSetIndex-r16 | 0 |  |  |
| dl-SelectedPRS-ResourceIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxResourcesPerSet-r16)) OF DL-SelectedPRS-ResourceIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceIdIndex-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-SelectedDL-PRS-IndexPerTRP-r16[3] SEQUENCE { |  | entry 3 | Sub-test 21 only as defined in clause 8.2.11 |
| nr-SelectedTRP-Index-r16 | 2 |  |  |
| dl-SelectedPRS-ResourceSetIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxSetsPerTrp-r16)) OF DL-SelectedPRS-ResourceSetIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceSetIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceSetIndex-r16 | 0 |  |  |
| dl-SelectedPRS-ResourceIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxResourcesPerSet-r16)) OF DL-SelectedPRS-ResourceIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceIdIndex-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-SelectedDL-PRS-PerFreq-r16[2] SEQUENCE { | entry 2 |  | Sub-test 21 only as defined in clause 8.2.11 |
| nr-SelectedDL-PRS-FrequencyLayerIndex-r16 | 1 |  |  |
| nr-SelectedDL-PRS-IndexListPerFreq-r16 ::= SEQUENCE (SIZE (1.. nrMaxTRPsPerFreq-r16)) OF NR-SelectedDL-PRS-IndexPerTRP-r16 { | 1 entry |  |  |
| NR-SelectedDL-PRS-IndexPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-SelectedTRP-Index-r16 | 0 |  |  |
| dl-SelectedPRS-ResourceSetIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxSetsPerTrp-r16)) OF DL-SelectedPRS-ResourceSetIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceSetIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceSetIndex-r16 | 0 |  |  |
| dl-SelectedPRS-ResourceIndexList-r16 ::= SEQUENCE (SIZE (1.. nrMaxResourcesPerSet-r16)) OF DL-SelectedPRS-ResourceIndex-r16 { | 1 entry |  |  |
| DL-SelectedPRS-ResourceIndex-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-SelectedPRS-ResourceIdIndex-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	NR-DL-PRS-Info

Table 8.4.1.6-3: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 | The choice of subcarrier spacing is specified in TS 38.508-1[30] clause 6.2.3. |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Not present |  |  |
| dl-PRS-ResourceTimeGap-r16 | Not present |  |  |
| dl-PRS-NumSymbols-r16 | n2 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 0 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  |  |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.4.1.7	DL-AoD Assistance Data Elements

This clause defines the DL-AoD assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in addition to those defined in Table 8.4.1.6-1, Table 8.4.1.6-2 and Table 8.4.1.6-3

-	NR-PositionCalculationAssistance

Table 8.4.1.7-1: NR-PositionCalculationAssistance

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-PositionCalculationAssistance-r16 ::= SEQUENCE { |  |  |  |
| nr-TRP-LocationInfo-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-TRP-LocationInfoPerFreqLayer-r16 { | 2 entries |  |  |
| NR-TRP-LocationInfoPerFreqLayer-r16[1] SEQUENCE { |  | entry1 |  |
| referencePoint-r16 |  |  |  |
| trp-LocationInfoList-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfoElement-r16 { | 3 entries |  |  |
| TRP-LocationInfoElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | Not present | Same as the reference point location |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| TRP-LocationInfoElement-r16[2] SEQUENCE { |  | entry 2 | Sub-tests 20 and 21 only; and as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 2 as defined in 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| TRP-LocationInfoElement-r16[3] SEQUENCE { |  | entry 3 | Sub-test 21 only as defined in clause 8.2.11 |
| dl-PRS-ID-r16 | 3 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 4 as defined in TS 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-TRP-LocationInfoPerFreqLayer-r16[2] SEQUENCE { |  | entry 2 |  |
| referencePoint-r16 | Not present | Same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list. |  |
| trp-LocationInfoList-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfoElement-r16 { | 1 entry |  |  |
| TRP-LocationInfoElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 3 as defined in TS 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| } |  |  |  |

8.4.1.8	DL-TDOA Assistance Data Elements

The DL-TDOA assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages are defined in Table 8.4.1.6-1, Table 8.4.1.6-2, Table 8.4.1.6-3 and Table 8.4.1.7-1.