3GPP TSG-RAN WG5 Meeting #92-e	R5-214757

# Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2347 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5G Power saving TC 8.1.5.10.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5G UAI TC 8.1.5.10.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding NR5G UAI TC 8.1.5.10.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_UE_pow_sav-UEConTest WP will not reach 100% completion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.5.10	UE Assistance Information

##### 8.1.5.10.1	UE Assistance Information/ Release Preference

8.1.5.10.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide its release preference and a Preferred RRC State is provided (UE determines that it would prefer to transition out of RRC_CONNECTED) and timer T346f is not running }

then { the UE transmits a UEAssistanceInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide its release preference and a Preferred RRC State is provided (UE determines that it would prefer to transition out of RRC_CONNECTED) and timer T346f is running }

then { the UE does not transmit a UEAssistanceInformation message }

}

(3)

with { UE in NR RRC_CONNECTED state and is configured with connectedReporting and having transmitted a UEAssistanceInformation message to inidicate preference to transition out of RRC_CONNECTED state }

ensure that {

when { UE determines that it would prefer to remain in RRC_CONNECTED state and timer T346f is not  running }

then { the UE transmits a UEAssistanceInformation message }

}

8.1.5.10.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.5.9, 5.7.4.2 and 5.7.4.3. Unless state explicitly these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

The UE shall:

…

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

[TS 38.331, clause 5.7.4.2]

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

…

Upon initiating the procedure, the UE shall:

1>	if configured to provide its release preference and timer T346f is not running:

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide a release preference according to 5.7.4.2 or 5.3.5.3:

2>	include releasePreference in the UEAssistanceInformation message;

2>	set preferredRRC-State to the desired RRC state on transmission of the UEAssistanceInformation message;

8.1.5.10.1.3	Test Description

8.1.5.10.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.1.3.2	Test procedure sequence

Table 8.1.5.10.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure releasePreferenceConfig with releasePreferenceProhibitTimer IE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘idle’. | <-- | SET UAI REQUEST | - | - |
| - | EXCEPTION: Step 4 and 5 can be performed in any order. | - | - | - | - |
| 4 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 5 | Check: Does the UE transmits an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘idle’ within the ReleasePreference-r16 IE? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | SS starts timer T346f = 30s. | - | - | - | - |
| 7 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘inactive’. | <-- | SET UAI REQUEST | - | - |
| 8 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 9 | Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘inactive’ within the ReleasePreference-r16 IE? | --> | NR RRC: UEAssistanceInformation | 2 | F |
| 9 | SS waits for timer T346f expiry. | - | - | - | - |
| - | EXCEPTION: Step 10a may be performed if the UE cached the preferredRRC-State indication from upper layers. |  |  |  |  |
| 10a | The UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘inactive’ within the ReleasePreference-r16 IE. | --> | NR RRC: UEAssistanceInformation | - | - |
| 11 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = connected. | <-- | SET UAI REQUEST | - | - |
| 12 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 13 | Check: Does the UE transmits an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘connected’ within the ReleasePreference-r16 IE? | --> | NR RRC: UEAssistanceInformation | 3 | F |
| 14 | The SS transmits an RRCReconfiguration message to configure releasePreferenceConfig with connectedReporting. | <-- | NR RRC: RRCReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 16 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘connected’. | <-- | SET UAI REQUEST | - | - |
| - | EXCEPTION: Step 18 and 19 can be performed in any order. | - | - | - | - |
| 17 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 18 | Check: Does the UE transmits an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘connected’ within the ReleasePreference-r16? | --> | NR RRC: UEAssistanceInformation | 3 | P |

8.1.5.10.1.3.3	Specific message contents

Table 8.1.5.10.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 ::= SEQUENCE { |  |  |  |
| releasePreferenceConfig-r16 ::= SEQUENCE { |  |  |  |
| releasePreferenceProhibitTimer-r16 | s30 |  |  |
| connectedReporting | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.1.3.3-2: UEAssistanceInformation (Step 5, step 10a and step 18, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| releasePreference-r16::= SEQUENCE { |  |  |  |  |
| preferredRRC-State-r16 |  | idle |  | Step 5 |
|  |  | inactive |  | Step 10a |
|  |  | connected |  | Step 18 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.10.1.3.3-1: RRCReconfiguration (step 14, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 ::= SEQUENCE { |  |  |  |
| releasePreferenceConfig-r16 ::= SEQUENCE { |  |  |  |
| releasePreferenceProhibitTimer-r16 | s30 |  |  |
| connectedReporting | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |