3GPP TSG-RAN5 Meeting #92-e	R5-214828

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2385 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MDT NR TC 8.1.6.3.4.1-Inter System_CEF_bluetooth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In R16, the NR UE also needs to support Inter-System MDT features of CEF (Connection Establishment Failure) measurement reporting. Thus, relevant new test cases need to be introduced in RAN5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added a new sub clause 8.1.6.3.4.1 to describe the test case of NR Inter-System MDT CEF measurement reporting of bluetooth information. - Complete the editorial text of the subtiles 8.1.6.3.2/8.1.6.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of relevent test case for NR Inter-System MDT CEF measurement reporting of bluetooth information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2. CR 0168 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.6.3.2	Inter-System MDT / Logged MDT

…

8.1.6.3.3	Inter-System MDT / Radio Link Failure

<End of modified section 1>

<Start of modified section 2>

8.1.6.3.4	Inter-System MDT / Connection Establishment Failure

8.1.6.3.4.1	Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection

8.1.6.3.4.1.1	Test Purpose (TP)

(1)

with { UE detects connection establishment failure }

ensure that {

when { UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailableBT-r16 set true }

}

(2)

with { UE has sent the RRCSetupComplete message with logMeasAvailableBT-r16 set true }

ensure that {

when { UE receives a UEInformationRequest message with logMeasReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the Bluetooth beacon(s) that included in bt-NameListConfig}

}

8.1.6.3.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.3.5.9, 5.3.3.7 and 5.7.10.3.

[TS 38.331, clause 5.3.5.9]

The UE shall:

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.4.1.3	Test description

8.1.6.3.4.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, Bluetooth beacon 1 (Cell 40), Bluetooth beacon 2 (Cell 41)

(Editor’s Note: configurations of Bluetooth beacons are expected to be added in 38.508-1.)

UE:

None

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.4.1.3.2	Test procedure sequence

Table 8.1.6.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging | - | - | - | - |
| 4 | The SS is configured to not transmit Msg2 in 2-step RACH procedure. | - | - | - | - |
| 5-6 | Steps 1 to 2 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. | - | - | - | - |
| 7 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 8 | The SS is configured to normal that it would transmit Msg2 in 2-step RACH procedure. | - | - | - | - |
| 9-11 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. | - | - | - | - |
| 12 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailableBT included? | --> | NR RRC: RRCSetupComplete 5GMM: Service Request | 1 | P |
| 13-16 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. | - | - | - | - |
| 17 | The SS sends a UEInformationRequest message to get logMeasReportReq-r16. | <-- | UEInformationRequest | - | - |
| 18 | Check: Does the UE transmit a UEInformationResponse message with logMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | UEInformationResponse | 2 | P |

8.1.6.3.4.1.3.3	Specific message contents

Table 8.1.6.3.4.1.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.1.3.2-1)

| Derivation path: 38.508 clause 4.6.1 table 4.6.1-5AA LoggedMeasurementConfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16-IEs SEQUENCE { |  |  |  |
| traceReference-r16 | Not checked |  |  |
| traceRecordingSessionRef-r16 | Not checked |  |  |
| tce-Id-r16 | Not checked |  |  |
| absoluteTimeInfo-r16 | Not checked |  |  |
| bt-NameList-r16 SEQUENCE { SIZE (1..maxBT-Name-r16)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 3GPP TS 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| { |  |  |  |
| reportType CHOICE { |  |  |  |
| Periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms2560 | 2.56 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-2: RRCSetupComplete (step 12, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension EQUENCE{ |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| logMeasAvailable-r16 | Not checked |  |  |
| logMeasAvailableBT-r16 | true |  |  |
| logMeasAvailableWLAN-r16 | Not checked |  |  |
| connEstFailInfoAvailable-r16 | true |  |  |
| rlf-InfoAvailable-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-3: UEInformationRequest (step 17, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A, condition FAIL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| connEstFailReportReq-r16 SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | Not checked |  |  |
| logMeasReportReq-r16 | True |  |  |
| connEstFailReportReq-r16 | True |  |  |
| ra-ReportReq-r16 | Not checked |  |  |
| rlf-ReportReq-r16 | Not checked |  |  |
| mobilityHistoryReportReq-r16 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| criticalExtensionsFuture | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-4: UEInformationResponse (step 18, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Not checked |  |  |
| traceReference-r16 | Not checked |  |  |
| traceRecordingSessionRef-r16 | Not checked |  |  |
| tce-Id-r16 | Not checked |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF SEQUENCE { | Not checked |  |  |
| LocationInfo-r16 SEQUENCE { |  |  |  |
| LogMeasResultListBT-r16 ::= SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF SEQUENCE { | 1 entry | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 | Not checked |  |  |
| numberOfConnFail-r16 | Not checked | INTEGER (1..8) |  |
| perRAInfoList-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 2>