3GPP TSG-RAN5 Meeting #92-e	R5-214972

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2419 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MDT test case 8.1.6.1.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the table 8.1.6.1.2.5.3.3-2 & 8.1.6.1.2.5.3.3-5, they refer to the wrong table number in TS 38.508-1; - Editorial error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the wrong table number in the table 8.1.6.1.2.5.3.3-2 & 8.1.6.1.2.5.3.3-5; - Modify them. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE can't pass this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.6.1.2.5	Logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger

8.1.6.1.2.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state camping normally on an NR cell where reportType is set to eventTriggered and eventType is set to eventL1 }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements only when the conditions indicated by the eventL1 are met}

}

8.1.6.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, 5.7.10.3 and 6.2.2.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

- set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 6.2.2]

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration.

8.1.6.1.2.5.3	Test description

8.1.6.1.2.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.5.3.2	Test procedure sequence

Table 8.1.6.1.2.5.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.5.3.2-1: Time instances of cell power level in FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -98 | Power level is such that entry condition for eventL1 is satisfied Ms + Hys < Thresh |

Table 8.1.6.1.2.5.3.2-2: Time instances of cell power level in FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Power level is such that entry condition for eventL1 is satisfied Ms + Hys < Thresh |

Table 8.1.6.1.2.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to eventL1, to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30s to allow UE to activate logging | - | - | - | - |
| 4-6 | Steps 1 to 3 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | - |
| 8-11 | Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message with logMeasReportReq present. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message including logMeasReport? | --> | NR RRC: UEInformationResponse | 1 | - |
| 14 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 15 | The SS changes Cell 1 levels according to the row "T1" in table 8.1.6.1.2.5.3.2-1/2. | - | - | - | - |
| 16 | Wait 30s to allow UE to activate logging | - | - | - | - |
| 17-19 | Steps 1 to 3 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed. | - | - | - | - |
| 20 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |
| 21-24 | Steps 5 to 8 of of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 25 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 26 | Check: Does the UE transmit a UEInformationResponse message. | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.2.5.3.3	Specific message contents

Table 8.1.6.1.2.5.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.5.3.2-3)

Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-5AA with condition EVENTL1

Table 8.1.6.1.2.5.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension { |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 |  | Not Present |  |  |
| logMeasAvailableBT-r16 |  | Not Present |  |  |
| logMeasAvailableWLAN-r16 |  | Not Present |  |  |
| connEstFailInfoAvailable-r16 |  | Not Present |  |  |
| rlf-InfoAvailable-r16 |  | Not Present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.5.3.3-3: UEInformationRequest (step 12 and step 25, Table 8.1.6.1.2.5.3.2-3)

Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.5.3.3-4: UEInformationResponse (step 13, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.5.3.3-5: RRCSetupComplete (step 20, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension { |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 |  | True |  |  |
| logMeasAvailableBT-r16 |  | Not Present |  |  |
| logMeasAvailableWLAN-r16 |  | Not Present |  |  |
| connEstFailInfoAvailable-r16 |  | Not Present |  |  |
| rlf-InfoAvailable-r16 |  | Not Present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.5.3.3-6: UEInformationResponse (step 26, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: 38.508-1 clause 4.6.1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF SEQUENCE { |  | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r16[x] |  | Not checked |  |  |
| relativeTimeStamp-r16 [x] |  | SS record the value |  |  |
| servCellIdentity-r16 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-16 [x] SEQUENCE { |  |  |  |  |
| resultsSSB-Cell-r16 |  | MeasQuantityResults of Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |  |
| best-ssb-Index |  | Not checked |  |  |
| best-ssb-Results |  | Not checked |  |  |
| numberOfGoodSSB |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells-r16 [x] SEQUENCE {} |  | Not present |  |  |
| anyCellSelectionDetected-r16[x] |  | Not present |  |  |
| } |  |  |  |  |
| logMeasAvailable-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |