3GPP TSG-RAN5 Meeting #92-e	R5-215191

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2010 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for n24 and n99 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specifications for Rel-17 n24 and n99 bands in RAN4 have been completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for Tx-Rx spacing of -101.5 and -120.5 MHz added for SCS of 15, 30 and 60 kHz for n24 Test frequencies added for SCS of 15, 30 and 60 kHz for n99 Test frequencies for NR standalone signalling testing added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies for RF test cases for n24 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A, 4.3.1.0B, 4.3.1.0C, 4.3.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.1	Test frequencies

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8] and TS 38.101-3 [9].

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1 and Table 4.3.1.0A-2 for FR1 and FR2 respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 156, 257 |
| n2 | 15 |
| n3 | 20 |
| n5 | 15 |
| n7 | 156, 257 |
| n8 | 15 |
| n12 | 10 |
| n14 | 10 |
| n20 | 15 |
| n24 | 10 |
| n25 | 15 |
| n26 | 108 |
| n28 | 15 |
| n29 | 102 |
| n30 | 10 |
| n34 | 10 |
| n38 | 15 |
| n39 | 20 |
| n40 | 30 |
| n41 | 60 |
| n48 | 204, 405 |
| n50 | 20 |
| n51 | 5 |
| n53 | 10 |
| n65 | 15 |
| n66 | 20 |
| n70 | 15 |
| n71 | 10 |
| n74 | 15 |
| n75 | 152 |
| n76 | 52 |
| n77 | 50 |
| n78 | 50 |
| n79 | 60 |
| n80 | 203 |
| n81 | 153 |
| n82 | 153 |
| n83 | 153 |
| n84 | 153, 6, 257 |
| n86 | 203 |
| n99 | 103 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 20MHz. Note 7: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 50MHz. Note 8: This Mid test channel bandwidth is chosen since it is more commonly used. |  |

Table 4.3.0A.1-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 200 |
| n258 | 200 |
| n260 | 200 |
| n261 | 200 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs. |  |

#### 4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1 and Table 4.3.1.0B-2 for FR1 and FR2 respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 5 |
| n2 | 5 |
| n3 | 5 |
| n5 | 5 |
| n7 | 5 |
| n8 | 5 |
| n12 | 5 |
| n14 | 5 |
| n20 | 5 |
| n24 | 5 |
| n25 | 5 |
| n26 | 5 |
| n28 | 5 |
| n29 | 52 |
| n30 | 5 |
| n34 | 5 |
| n38 | 5 |
| n39 | 5 |
| n40 | 54,105 |
| n41 | 10 |
| n48 | 54, 105 |
| n50 | 54,105 |
| n51 | 5 |
| n53 | 5 |
| n65 | 5 |
| n66 | 5 |
| n70 | 5 |
| n71 | 5 |
| n74 | 5 |
| n75 | 52 |
| n76 | 52 |
| n77 | 10 |
| n78 | 10 |
| n79 | 40 |
| n80 | 53 |
| n81 | 53 |
| n82 | 53 |
| n83 | 53 |
| n84 | 53 |
| n86 | 53 |
| n99 | 53 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 UEs. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 50 |
| n258 | 50 |
| n260 | 50 |
| n261 | 50 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 UEs.. |  |

#### 4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1 and Table 4.3.1.0C-2 for FR1 and FR2 respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 206, 507 |
| n2 | 20 |
| n3 | 308, 409 |
| n5 | 20 |
| n7 | 206, 507 |
| n8 | 20 |
| n12 | 15 |
| n14 | 10 |
| n20 | 20 |
| n24 | 10 |
| n25 | 206, 409 |
| n26 | 20 |
| n28 | 206, 308 |
| n29 | 102 |
| n30 | 10 |
| n34 | 15 |
| n38 | 20 |
| n39 | 40 |
| n40 | 80 |
| n41 | 100 |
| n48 | 403, 1004 |
| n50 | 80 |
| n51 | 5 |
| n53 | 10 |
| n65 | 20 |
| n66 | 40 |
| n70 | 151/252 |
| n71 | 20 |
| n74 | 20 |
| n75 | 202 |
| n76 | 52 |
| n77 | 100 |
| n78 | 100 |
| n79 | 100 |
| n80 | 301 |
| n81 | 201 |
| n82 | 201 |
| n83 | 201, 308 |
| n84 | 201, 507 |
| n86 | 401 |
| n99 | 101 |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: For UEs with limited UE channel bandwidth capability, if the above defined high channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel-15 UEs. Note 6: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 20MHz. Note 7: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 50MHz. Note 8: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 30MHz. Note 9: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 40MHz. |  |

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 400 |
| n258 | 400 |
| n260 | 400 |
| n261 | 400 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined high channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel-15 UEs. |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 4.3.1.1.1	NR operating bands in FR1

<<Unchanged text skipped>>

4.3.1.1.1.21 to 4.3.1.1.1.23	FFS

4.3.1.1.1.24	Reference test frequencies for NR operating band n24

Table 4.3.1.1.1.24-1a: Test frequencies for NR operating band n24 and SCS 15 kHz; Tx-Rx spacing of -101.5 MHz

| UL/DL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1531.0 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1632.5 | 326500 | 1539.53 | 307906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1635.0 | 327000 | 1631.67 | 326334 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low Mid High | 1531.0 | 306200 | 1526.32 | 305264 | 0 | 15 | 3823 | 305810 | 2 | 1 | 2 (4) | 5 |
|  |  | Uplink | Low Mid High | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-1b: Test frequencies for NR operating band n24 and SCS 15 kHz; Tx-Rx spacing of -120.5 MHz

| UL/DL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1531.0 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1649.0 | 329800 | 1646.75 | 329350 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1651.5 | 330300 | 1558.53 | 311706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low Mid High | 1531.0 | 306200 | 1526.32 | 305264 | 0 | 15 | 3823 | 305810 | 2 | 1 | 2 (4) | 5 |
|  |  | Uplink | Low Mid High | 1651.5 | 330300 | 1646.82 | 329364 | 0 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-2a: Test frequencies for NR operating band n24 and SCS 30 kHz; Tx-Rx spacing of -101.5 MHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid High | 1531.0 | 306200 | 1526.68 | 305336 | 0 | 30 | 3829 | 306290 | 6 | 0 | 3 (3) | 6 |
|  |  | Uplink | Low Mid High | 1632.5 | 326500 | 1628.18 | 325636 | 0 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-2b: Test frequencies for NR operating band n24 and SCS 30 kHz; Tx-Rx spacing of -120.5 MHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid High | 1531.0 | 306200 | 1526.68 | 305336 | 0 | 30 | 3829 | 306290 | 6 | 0 | 3 (3) | 6 |
|  |  | Uplink | Low Mid High | 1651.5 | 330300 | 1647.18 | 329436 | 0 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-3a: Test frequencies for NR operating band n24 and SCS 60 kHz without CORESET#0; Tx-Rx spacing of -101.5 MHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low Mid High | 1531.0 | 306200 | 1527.04 | 305408 | 0 | 15 | - | 305768 |
|  |  | Uplink | Low Mid High | 1632.5 | 326500 | 1628.54 | 325708 | 0 | - | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-3b: Test frequencies for NR operating band n25 and SCS 60 kHz without CORESET#0; Tx-Rx spacing of -120.5 MHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low Mid High | 1531.0 | 306200 | 1527.04 | 305408 | 0 | 15 | - | 305768 |
|  |  | Uplink | Low Mid High | 1651.5 | 330300 | 1647.54 | 329508 | 0 | - | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.25	Reference test frequencies for NR operating band n25

<<Unchanged text skipped>>

4.3.1.1.1.96 to 4.3.1.1.1.98	FFS

4.3.1.1.1.99	Reference test frequencies for NR operating band n99 (SUL)

Table 4.3.1.1.1.99-1: Test frequencies for NR operating band n99 and SCS 15 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 |
|  |  |  | Mid | 1632.5 | 326500 | 1539.53 | 307906 | 504 |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |
| 10 | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |

Table 4.3.1.1.1.99-2: Test frequencies for NR operating band n99 and SCS 30 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 |

Table 4.3.1.1.1.99-3: Test frequencies for NR operating band n99 and SCS 60 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------