3GPP TSG-RAN5 Meeting #92-e	R5-215211

# Electronic Meeting, 16th – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1115 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.5B.2.2.1 SEM for intra-band non-contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The notation of E-UTRA uplink RB allocation was not in line with that in other test cases, and the corresponding explanation was missing. - The initial procedure was wrongly described as LTE anchor agnostic. - In test procedure, there was a lack of the step to test maximum power reduction. And the emission power shall be measured according to exceptional test requirements in clause 6.5B.2.2.1.5 rather than that in standalone scenarios. - The whole subclause of Message Contents was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Unify the notation of E-UTRA uplink RB allocation to align with other test cases. - Correct initial procedures to enable transmission on E-UTRA carrier. - Add the step to measure maximum power reduction, and update the step of measuring power of emission. - Supplement the subclause of Message Contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would not be performed properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5B.2.2.1	Spectrum emissions mask for intra-band non-contiguous EN-DC

Editor's note:	The following aspects are either missing or not yet determined:

-	Testing of none simultaneous LTE-NR transmission is FFS.

6.5B.2.2.1.1	Test purpose

To verify that the power of any UE emissions shall not exceed specified level for the specified channel bandwidth.

6.5B.2.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.1.3	Minimum conformance requirements

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.1.

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask and therefore LTE anchor agnostic approach is not applied.

6.5B.2.2.1.4	Test description

6.5B.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in TS 38.508-1 [6] clause 4.3.1.4.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in TS 38.508-1 [6] clause 4.3.1.4.3 and are shown in Table 6.5B.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1.4.3 for cases with single E-UTRA carrier |  |  | Low with maxWgap (NR low – E-UTRA high); High with maxWgap (E-UTRA low – NR high) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 |  |  | Lowest for E-UTRA, lowest for NR; Highest for E-UTRA, highest for NR |  |  |  |
| Test SCS as specified as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 |  |  | Lowest, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | NR Uplink Configuration |  | E-UTRA Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) | Modulation | RB allocation (NOTE 3) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | QPSK | Outer_1RB_Right |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | QPSK | Outer_1RB_Left |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full | QPSK | Outer_Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | QPSK | Outer_1RB_Right |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | QPSK | Outer_1RB_Left |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer_Full | QPSK | Outer_Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer_ Full | 16QAM | Outer_Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer_ Full | 16QAM | Outer_Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer_ Full | 16QAM | Outer_Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | QPSK | Outer_1RB_Right |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right | QPSK | Outer_1RB_Left |
| 18 | Default |  | CP-OFDM QPSK | Outer_ Full | QPSK | Outer_Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 21 | Default |  | CP-OFDM 16 QAM | Outer_ Full | 16QAM | Outer_Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 24 | Default |  | CP-OFDM 64 QAM | Outer_ Full | 16QAM | Outer_Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 27 | Default |  | CP-OFDM 256 QAM | Outer_ Full | 16QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2. The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.2.1.4.2	Test Procedure

- For NR carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

- For E-UTRA carrier, SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

- Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting in this step for the UE to reach PUMAX level.

- For a UE supporting dynamic power sharing, measure the mean power over all component carriers. For a UE not supporting dynamic power sharing, measure the power of each component carrier individually. The measure transmitted power shall meet the requirements in clause 6.2B.2.2.5. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms). For TDD slots with transient periods are not under test.

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to clause 6.5B.2.2.1.5. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency

NOTE 1: When switching to DFT-s-OFDM waveform, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.2.1.4.3	Message Content

Message contents are according to TS 36.508-1 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.2.2.1.4.3-1: RRCConnectionReconfiguration: nr-Config-r15.

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.1.4.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.1.4.3-3: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.2.2.1.5	Test requirement

-For NR carrier frequency masks not overlapping with E-UTRA carrier frequency masks, the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8].

-For E-UTRA carrier frequency masks not overlapping with NR carrier frequency masks, the power of any UE emission shall fulfil requirements in Table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable.

-For NR carrier frequency masks overlapping with E-UTRA carrier frequency masks, the most relaxed limit is used between requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8] and Table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable.

<End of Changes>