Source: 	oppo, Huawei

Title: 	Discussion on test point selection for Relative power tolerance for CA

Document for:	Discussion and endorsement

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation.

The purpose of this contribution is to provide the complete analaysis for each parameter included in the Test Configuration Table and propose test points selection for Relative power tolerance for CA.

Discussion

Relative power tolerance for CA is defined in [2] where its scope is described as follows:

For intra-band contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in subclause 6.3A.1.0 and the total power is limited by PUMAX as defined in subclause 6.2A.4.0. The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.4.3.3-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.4.3.3-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subframes;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.4.3.3-1 for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

For intra-band non-contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in subclause 6.3A.1 and the total power is limited by PUMAX as defined in subclause 6.2A.4.0. The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.4.3-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.4.3-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subrames;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.4.3-1 for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3A.4.

For inter-band CA:

No requirements unique to CA operation are defined.

Proposal 1: Relative power tolerance test is no need for Inter-band CA.

The test point selection from the relevant NR single carrier (Table 6.3.4.3.4.1-1 in [1]) and LTE intra-band contiguous CA (Table 6.3.5A.2.1.4.1-1 in [3]) and LTE intra-band non-contiguous CA (Table 6.3.5A.2.3.4.1-1 in [3] ) test case is extracted as below:

Table 6.3.4.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Ch BW | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB Allocation | Modulation | RB allocation (NOTE 1) |
| 5MHz | N/A for Relative power tolerance test case |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-1 See Table 6.3.4.3.5-2 See Table 6.3.4.3.5-7 |
| 10MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 15MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 20MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 25MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 30MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 40MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 50MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 60MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 80MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 90MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 100MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| Note 1: The starting resource block shall be RB# 0 |  |  |  |  |

Table 6.3.5A.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] clause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] clause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (Note 3) (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 6 13 2 16 | P_5@20 P_5@20 P_1@24 P_8@17 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 25 | 50 | N/A for this test | QPSK | 6 13 2 16 | P_5@20 P_5@20 P_1@24 P_8@17 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 50 | 50 |  | QPSK | 6 13 2 16 | P_5@45 P_5@45 P_1@49 P_8@42 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 75 | 25 |  | QPSK | 6 13 2 16 | P_5@70 P_5@70 P_1@74 P_8@67 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 75 | 75 |  | QPSK | 6 13 2 16 | P_5@70 P_5@70 P_1@74 P_8@67 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 25 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 50 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 75 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| 100 | 100 |  | QPSK | 6 13 2 16 | P_5@95 P_5@95 P_1@99 P_8@92 | S_1@0 S_8@0 S_1@0 S_8@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. Note 3: The UL allocation is changed as part of the test procedure. The Test Configuration Table entries list the combinations used, with the sequence of usage as determined by the test procedure for each sub-test. |  |  |  |  |  |  |  |  |

Table 6.3.5A.2.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap , Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC NRB_alloc (LCRB @ RBstart) | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | QPSK | 6 13 2 16 | P_5@0 P_5@0 P_1@0 P_8@0 | S_1@0 S_8@0 S_1@0 S_8@0 |
| 2 | 25 | 25 | 5 | N/A | N/A | QPSK | 6 13 2 16 | P_5@0 P_5@0 P_1@0 P_8@0 | S_1@0 S_8@0 S_1@0 S_8@0 |
| 34 | 50 | 50 | 25 | N/A | N/A | QPSK | 6 13 2 16 | P_5@0 P_5@0 P_1@0 P_8@0 | S_1@0 S_8@0 S_1@0 S_8@0 |
| 45 | 100 | 100 | 5 | N/A | N/A | QPSK | 6 13 2 16 | P_5@0 P_5@0 P_1@0 P_8@0 | S_1@0 S_8@0 S_1@0 S_8@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. Note 3: The UL allocation is changed as part of the test procedure. The Test Configuration Table entries list the combinations used, with the sequence of usage as determined by the test procedure for each sub-test. Note 4: Not applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration Note 5: Applicable if only Bandwidth Combination Set 0 is supported by the UE for the CA Configuration. |  |  |  |  |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

According to general comment in [2] core spec, the power control cases shall be tested “under normal conditions” only:

The requirements on power control accuracy apply under normal conditions.

It is reasonable to use this principle for NR CA power control test cases.

Proposal 2: Define Test Environment as Normal Conditions (NC) for Relative Power Tolerance for CA.

Test Subcarrier Spacing

Lowest and Highest SCS are tested in Relative Power Tolerance for single carrier case in FR1, it’s reasonable to select the same SCS for CA test cases regardless of number of CCs.

Proposal 3: Select Lowest and Highest SCS supported for UE for Relative Power Tolerance measurement for CA.

  - Test Frequencies

LTE intra-band contiguous CA test case uses mid frequency but NR single carrier uses Low frequency. It is hard to say which frequency should be the worst case. Because Mid frequency is more common and been selected by many CA cases. We suggest to use Mid frequency for intra-band contiguous CA in this case.

Low Wgap is defined for Tx intra-band non-contiguous CA test cases in LTE, Low Wgap and Maximum Wgap are both tested in LTE. However in NR, testing with max Wgap can represent the worst case and avoid costing double test time. So it’s reasonable to test with Maximum Wgap for Relative Power Tolerance measurement for intra-band non-contiguous CA.

Proposal 4: Select Mid range as test frequencies for Relative Power Tolerance measurement for intra-band contiguous CA. Test frequencies for each CC depend on specific CA configurations with max Wgap for Absolute Power Tolerance measurement for intra-band non-contiguous CA.

Test Channel Bandwidth

In LTE CA, Lowest NRB_agg and Highest NRB_agg is selected. We believe this is also reasonable for NR CA.

Proposal 5: Test the Lowest NRB_agg and Highest NRB_agg as Channel Bandwidth for Relative Power Tolerance measurement for CA.

Uplink Modulations and RB allocation

In NR single carrier and LTE CA, DFT-s-OFDM QPSK is selected. We suggest to use the same configutation for NR CA.

Proposal 6: Use DFT-s-OFDM QPSK for Relative Power Tolerance measurement for CA.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for Relative power control for CA:

Proposal 1: Relative power tolerance test is no need for Inter-band CA.

Proposal 2: Define Test Environment as Normal Conditions (NC) for Relative Power Tolerance for CA.

Proposal 3: Select Lowest and Highest SCS supported for UE for Relative Power Tolerance measurement for CA.

Proposal 4: Select Mid range as test frequencies for Relative Power Tolerance measurement for intra-band contiguous CA. Test frequencies for each CC depend on specific CA configurations with max Wgap for Absolute Power Tolerance measurement for intra-band non-contiguous CA.

Proposal 5: Test the Lowest NRB_agg and Highest NRB_agg as Channel Bandwidth for Relative Power Tolerance measurement for CA.

Proposal 6: Use DFT-s-OFDM QPSK for Relative Power Tolerance measurement for CA.

References

[1] 3GPP TS 38.521-1 V17.5.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 17)

[2] 3GPP TS 38.101-1 v16.8.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3] 3GPP TS 36.521-1 V16.9.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing (Release 16)