3GPP TSG-RAN5 Meeting #92-e	R5-214508

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2018 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to NR cell configurations for SIG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In subclause 6.3.1, an Editor’s Note remains, which should be resolved. Prose specification and TTCN implementation for 5GS Rel-15 has almost completed, and the need for NR cell configuration types, similar to those specified in TS 36.508 subclauses 6.3.3 & 6.3.4 for E-UTRA cells, has not been identified/requested. It can therefore be considered as not needed for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Editor’s Note has been deleted. The title of subclause 6.3.1 has been updated to better reflect the scope of this subclause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Issues captured via Editor’s Notes, remain unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Reference system configurations

### 6.3.1	Default System Information configurations

#### 6.3.1.1	Intra-frequency neighbouring cell list in SIB3 for NR cells

Intra-frequency neighbouring cell list for signalling test cases is defined in table 6.3.1.1-1. This table is referred to in the default contents of IE intraFreqNeighCellList in SIB3 defined in table 4.6.2-2.

Table 6.3.1.1-1: Intra-frequency neighbouring cell lists for NR cells

| cell ID | Test Frequency | intra-frequency neighbouring cell list |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |  |
|  |  |  | 1 | 1 | 3 |
| NR Cell 1 | NRf1 | 3 | NR Cell 2 | NR Cell 4 | NR Cell 11 |
| NR Cell 2 | NRf1 | 3 | NR Cell 1 | NR Cell 4 | NR Cell 11 |
| NR Cell 4 | NRf1 | 3 | NR Cell 1 | NR Cell 2 | NR Cell 11 |
| NR Cell 11 | NRf1 | 3 | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| NR Cell 3 | NRf2 | 1 | NR Cell 23 | - | - |
| NR Cell 23 | NRf2 | 1 | NR Cell 3 | - | - |

Editor’s Note: The intra-frequency NR neighbouring cell list for signalling NAS test cases when cells are on same PLMN is FFS.

#### 6.3.1.2	Inter-frequency carrier frequency list in SIB4 for NR cells

Inter-frequency NR carrier frequency list for signalling test cases is defined in table 6.3.1.2-1. This table is referred to in the default contents of IE interFreqCarrierFreqList in SIB4 defined in table 4.6.2-3.

Table 6.3.1.2-1: Inter-frequency carrier frequency lists for NR cells

| cell ID | Test Frequency | interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  |  | 1 | 2 | 3 |
| NR Cell 1 NR Cell 2 NR Cell 4 NR Cell 11 | NRf1 (Note 2) | 3 | NRf2 | NRf3 | NRf5 |
| NR Cell 3 NR Cell 23 | NRf2 (Note 2) | 3 | NRf1 | NRf3 | NRf5 |
| NR Cell 6 | NRf3 (Note 2) | 3 | NRf1 | NRf2 | NRf5 |
| NR Cell 10 | NRf5 (Note 3) | 3 | NRf1 | NRf2 | NRf3 |
| Note 1: Depending on the Band under test, NRf3 may not be applicable. Note 2: In case of Test frequency NRf1, NRf2 and NRf3, dl-CarrierFreq NRf5 as part of inter-frequency list is applicable only in case of multi-band scenarios. Note 3: Test frequency NRf5 is applicable only in case of multi-band scenarios. |  |  |  |  |  |

Editor’s Note: The inter-frequency NR carrier frequency list for signalling NAS test cases when cells are on same PLMN is FFS.

#### 6.3.1.3	E-UTRA carrier frequency list in SIB5 for NR cells

The frequency mapping of E-UTRA cells are defined as per TS 36.508 [2] clause 4.4.2 and TS 36.508 [2] clause 6.3.2 for NGC NAS test cases, E-UTRA frequency mapping is according to clause 6.2.3.3. E-UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.3-1. This table is referred to in the default contents of IE carrierFreqListEUTRA in SIB5 defined in table 4.6.2-4.

Table 6.3.1.3-1: E-UTRA carrier frequency lists for NR cells

| interFreqCarrierFreqList |  |
| --- | --- |
| number of entries | carrierFreq[n] |
|  | 1 |
| 1 | E-UTRA f1 |
| 2 | E-UTRA f2 |
| 3 | E-UTRA f3 |
| 4 | E-UTRA f4 |
| Note 1: E-UTRAf1, E-UTRAf2, E-UTRAf3, E-UTRAf4 are according to clause 6.2.3.3 Note 2: Depending on the Band under test, E-UTRA f2 or E-UTRA f3 or E-UTRA f4 may not be applicable. |  |

Table 6.3.1.3-2: Void

### 6.3.2	Default configurations for NAS test cases

The default configurations specified in this subclause apply only to NAS test cases. They apply to all NAS test cases unless otherwise specified.

#### 6.3.2.1	Simulated network scenarios for NAS test cases

Simulated network scenarios for NAS test cases to be tested are specified in the pre-test conditions of each individual test case.

Any combination is allowed with the following restrictions:

-	NGC Cell B shall not be used if NGC Cell D is used

-	a maximum 3 cells on the same frequency can be used, i.e. only 3 cells out of NGC Cell A, NGC Cell B, NGC Cell C and NGC Cell D may be used simultaneously in each individual test case when cells in the test case are in different PLMNs (refer to Table 6.3.2.2-3).

#### 6.3.2.2	Simulated NAS cells

Simulated NAS cells and default NAS parameters are specified in Table 6.3.2.2-1. Unless otherwise specified in a test case, default radio parameters of the NAS cells are specified as per Table 6.3.2.2-2.

Unless otherwise specified, the default parameters specified in clause 4.4.2 will also apply to all NAS cells.

Table 6.3.2.2-1: Default NAS parameters for simulated NAS cells

| NAS cell ID | Tracking Area |  |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC |  | MNC |  |  | AMF Region ID | AMF Set ID | AMF Pointer |  |
| NGC Cell A | TAI-1 | (Note 3) |  |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.10.1 [26]. |
| NGC Cell B | TAI-2 | (Note 3) |  |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NGC Cell C | TAI-3 | (Note 3) |  |  | 3 | TAI-3 | 252 | 1 | 1 |  |
| NGC Cell D | TAI-4 | (Note 3) |  |  | 4 | TAI-4 | 252 | 1 | 1 |  |
| NGC Cell E | TAI-12 | 002 |  | 101 | 3 | TAI-12 | 244 | 1 | 1 |  |
| NGC Cell F | TAI-11 | 003 | 101 |  | 2 | TAI-11 | 239 | 1 | 1 |  |
| NGC Cell G | TAI-7 | (Note 4) |  | 02 | 1 | TAI-7 | 238 | 1 | 1 |  |
| NGC Cell H | TAI-8 | (Note 4) |  | 02 | 2 | TAI-8 | 237 | 1 | 1 |  |
| NGC Cell I | TAI-9 | 002 |  | 101 | 1 | TAI-9 | 244 | 1 | 1 |  |
| NGC Cell J | TAI-10 | 003 |  | 101 | 1 | TAI-10 | 236 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure for initial access or mobility (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.8.3). Note 4: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.8.3). |  |  |  |  |  |  |  |  |  |  |

Table 6.3.2.2-2: Default radio parameters for simulated NAS cells when cells are in same PLMN and access stratum is NR

| NAS cell ID | Frequency | NR Cell ID (Note 1) |
| --- | --- | --- |
| NGC Cell A | NRf1 | NR Cell 1 |
| NGC Cell B | NRf1 | NR Cell 2 |
| NGC Cell C | NRf1 | NR Cell 4 |
| NGC Cell D | NRf1 | NR Cell 11 |
| NGC Cell E | NA | NA |
| NGC Cell F | NRf2 | NR Cell 3 |
| NGC Cell G | NA | NA |
| NGC Cell H | NA | NA |
| NGC Cell I | NA | NA |
| NGC Cell J | NRf2 | NR Cell 12 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2. Note 2: For signalling tests, simultaneous co-existence of NGC Cells B and D is not allowed (in line with Table 4.4.2-1) |  |  |

Table 6.3.2.2-3: Default PLMN and radio parameters for simulated NAS cells when cells are in different PLMNs and access stratum is NR

| NAS cell ID | PLMN | Frequency | NR Cell ID (Note 1) |
| --- | --- | --- | --- |
| NGC Cell A | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 1 |
| NGC Cell B | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 2 |
| NGC Cell C | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 4 |
| NGC Cell D | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 11 |
| NGC Cell E | MCC=002 MNC=101 | NRf2 | NR Cell 3 |
| NGC Cell F | MCC=003 MNC=101 | NRf4 | NR Cell 14 |
| NGC Cell G | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 12 |
| NGC Cell H | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 23 |
| NGC Cell I | MCC=002 MNC=101 | NRf3 | NR Cell 6 |
| NGC Cell J | MCC=002 MNC=101 | NRf3 | NR Cell 13 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2 Note 2: for signalling tests, simultaneous co-existence of NGC Cells B and D is not allowed (In line with Table 4.4.2-1) |  |  |  |