3GPP TSG-RAN5 Meeting #92-e	R5-215436

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1437 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MsgA configuration to the TS 38.533 annexes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several 2-step RACH RRM test cases have been added to TS 38.533. These test cases need the MsgA configuration from TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MsgA configuration has been added to the TS 38.133 Annexes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 2-step RACH work plan cannot be completed, since the required configuration for the test cases will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# A.7	Reference PRACH configurations

This section provides the typical PRACH configurations used for all RRM test cases defined in this test specification. Parameters not listed in this section can be derived from the configuration of each test.

## A.7.1	PRACH configurations for FR1

Table A.7.1-1 defines the PRACH configurations for FR1. Each of the PRACH configurations defined in Table A.7.1-1 have different applicabilities:

-	PRACH.1 FR1 for SSB-based contention based random access in FR1.

-	PRACH.2 FR1 for SSB-based non-contention based random access in FR1.

-	PRACH.3 FR1 for CSI-RS based non-contention based random access in FR1.

-	PRACH.4 FR1 for CSI-RS based non-contention based random access in FR1 to convey BFR.

Table A.7.1-1 Parameters for PRACH Configurations for FR1

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR1 | PRACH.2 FR1 | PRACH.3 FR1 | PRACH.4 FR1 |  |
| prach-ConfigurationIndex | 102 | 102 | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | - | - | - | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | - | oneFourth | oneFourth | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  |
| ra-ContentionResolutionTimer | sf48 | - | - | - | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl1 | 10 slots |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| csirs-ResourceList | N/A | N/A | present | presnt | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | N/A | 50 | 50 | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A | PRACH occasion index 1 is allowed |
| Note: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |

## A.7.2	PRACH configurations for FR2

Table A.7.2-1 defines the PRACH configurations for FR2. Each of the PRACH configurations defined in Table A.7.2-1 have different applicabilities:

-	PRACH.1 FR2 for SSB-based contention based random access in FR2.

-	PRACH.2 FR2 for SSB-based non-contention based random access in FR2.

-	PRACH.3 FR2 for CSI-RS based non-contention based random access in FR2.

-	PRACH.4 FR2 for CSI-RS based non-contention based random access in FR2 to convey BFR.

Table A.7.2-1 Parameters for PRACH Configurations for FR2

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR2 | PRACH.2 FR2 | PRACH.3 FR2 | PRACH.4 FR2 |  |
| prach-ConfigurationIndex | 190 | 190 | 190 | 190 | Preamble format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | N/A | N/A | N/A | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | N/A | oneFourth | oneFourth | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  |
| ra-ContentionResolutionTimer | sf48 | N/A | N/A | N/A | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 | Max number of RA preamble transmission performed before declaring a failure |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl40 |  |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0 |
| csirs-ResourceList | N/A | present | present | present | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | 50 | 50 | 50 | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A | PRACH occasion index 1 is allowed |
| Note: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |

# A.7A	Reference MsgA configurations

This section provides the typical PRACH and PUSCH configurations for MsgA used for all RRM test cases defined in this test specification. Parameters not listed in this section can be derived from the configuration of each test.

## A.7A.1	MsgA configurations for FR1

Table A.7A.1-1 defines the MsgA configurations for FR1. Each of the MsgA configurations defined in Table A.7A.1-1 have different applicabilities:

-	MsgA.1 FR1 for SSB-based 2-step contention based random access in FR1.

-	MsgA.2 FR1 for SSB-based 2-step non-contention based random access in FR1.

Table A.7A.1-1 Parameters for MsgA Configurations for FR1

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR1 | MsgA.2 FR1 |  |
| msgA-prach-ConfigurationIndex | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 14 | 14 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |

## A.7A.2	MsgA configurations for FR2

Table A.7A.2-1 defines the MsgA configurations for FR2. Each of the MsgA configurations defined in Table A.7A.2-1 have different applicabilities:

-	MsgA.1 FR2 for SSB-based contention based random access in FR2.

-	MsgA.2 FR2 for SSB-based non-contention based random access in FR2.

Table A.7A.2-1 Parameters for MsgA Configurations for FR2

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR2 | MsgA.2 FR2 |  |
| msgA-prach-ConfigurationIndex | 190 | 190 | Preamble Format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 10 | 10 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

< End of changes >