3GPP TSG-RAN5 Meeting #92-e	R5-215484

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5350 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for CA_21A-28A in test case 7.3A.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Table 5.4.2A.1-2, BCS0 of CA_21A-28A does not contain 15 MHz CHBW for Band 28 as an allowed configuration. However, in Reference Sensitivity test case 7.3A.3 the test of 15 MHz for Band 28 is specified in Table 7.3A.3.4.1-26a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test configuration in Table 7.3A.3.4.1-26a has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference sensitivity of CA_21A-28 cannot be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 7.3A.3.4.1	Initial conditions

Same as in clause 7.3A.1.4.1 with the following exceptions:

-	Instead of Table 7.3A.1.4.1-1 use Table 7.3A.3.4.1-1.

-	Instead of clause 7.3A.1.4.3 use clause 7.3A.3.4.3.

Table 7.3A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 7.1-2. |  |  |  |  | A: Mid range for PCC and SCC, unless otherwise specified in the Tables 7.3A.3.4.1-2 to 7.3A.3.4.1-33 |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Highest NRB_agg, unless otherwise specified in the Tables 7.3A.3.4.1-2 to 7.3A.3.4.1-33 |  |
| Network signalling value |  |  |  |  | NS_01 Unless given by Table 7.3.3.-3 for the band with active uplink carrier |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  |  | UL Allocation |  |
| PCC NRB | SCCs NRB | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC NRB_alloc |
| 25 | 25 | QPSK | 25 | 25 | QPSK | 25 |
| 25 | 25 | QPSK | 25 | 25 | QPSK | 8 |
| 50 | 50 | QPSK | 50 | 50 | QPSK | 50 |
| 50 | 50 | QPSK | 50 | 50 | QPSK | 25 |
| 50 | 50 | QPSK | 50 | 50 | QPSK | 20 |
| 50 | 50 | QPSK | 50 | 50 | QPSK | 16 |
| 50 | 75 | QPSK | 50 | 75 | QPSK | 50 |
| 50 | 75 | QPSK | 50 | 75 | QPSK | 25 |
| 50 | 75 | QPSK | 50 | 75 | QPSK | 20 |
| 50 | 75 | QPSK | 50 | 75 | QPSK | 16 |
| 50 | 100 | QPSK | 50 | 100 | QPSK | 50 |
| 50 | 100 | QPSK | 50 | 100 | QPSK | 25 |
| 50 | 100 | QPSK | 50 | 100 | QPSK | 20 |
| 75 | 25 | QPSK | 75 | 25 | QPSK | 75 |
| 75 | 25 | QPSK | 75 | 25 | QPSK | 20 |
| 75 | 50 | QPSK | 75 | 50 | QPSK | 75 |
| 75 | 50 | QPSK | 75 | 50 | QPSK | 50 |
| 75 | 50 | QPSK | 75 | 50 | QPSK | 25 |
| 75 | 50 | QPSK | 75 | 50 | QPSK | 20 |
| 75 | 50 | QPSK | 75 | 50 | QPSK | 18 |
| 75 | 75 | QPSK | 75 | 75 | QPSK | 75 |
| 75 | 75 | QPSK | 75 | 75 | QPSK | 50 |
| 75 | 75 | QPSK | 75 | 75 | QPSK | 25 |
| 75 | 75 | QPSK | 75 | 75 | QPSK | 20 |
| 75 | 100 | QPSK | 75 | 100 | QPSK | 75 |
| 75 | 100 | QPSK | 75 | 100 | QPSK | 50 |
| 75 | 100 | QPSK | 75 | 100 | QPSK | 25 |
| 75 | 100 | QPSK | 75 | 100 | QPSK | 20 |
| 100 | 25 | QPSK | 100 | 25 | QPSK | 25 |
| 100 | 50 | QPSK | 100 | 50 | QPSK | 100 |
| 100 | 50 | QPSK | 100 | 50 | QPSK | 75 |
| 100 | 50 | QPSK | 100 | 50 | QPSK | 50 |
| 100 | 50 | QPSK | 100 | 50 | QPSK | 25 |
| 100 | 50 | QPSK | 100 | 50 | QPSK | 20 |
| 100 | 75 | QPSK | 100 | 75 | QPSK | 100 |
| 100 | 75 | QPSK | 100 | 75 | QPSK | 75 |
| 100 | 75 | QPSK | 100 | 75 | QPSK | 50 |
| 100 | 75 | QPSK | 100 | 75 | QPSK | 25 |
| 100 | 75 | QPSK | 100 | 75 | QPSK | 25 |
| 100 | 75 | QPSK | 100 | 75 | QPSK | 20 |
| 100 | 100 | QPSK | 100 | 100 | QPSK | 20 |
| 100 | 100 | QPSK | 100 | 100 | QPSK | 25 |
| 100 | 100 | QPSK | 100 | 100 | QPSK | 45 |
| 100 | 100 | QPSK | 100 | 100 | QPSK | 50 |
| 100 | 100 | QPSK | 100 | 100 | QPSK | 75 |
| 100 | 100 | QPSK | 100 | 100 | QPSK | 100 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.3.5-2 for UE supporting one uplink carrier is tested per Test CA configuration. Note 3: The UL resource blocks shall be located as close as possible to the downlink PCC but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 4: The frequencies of PCC and SCC shall be switched and tested in each configuration, according to the UE declared capability for UL support (within CA operation) in the individual bands. |  |  |  |  |  |  |

Table 7.3A.3.4.1-1a: Void

Table 7.3A.3.4.1-2: Test frequencies and Test CC combinations for CA_1A-3A

| CBW |  |  |  |  |  | Band 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5MHz |  | 15MHz |  | 20MHz |  |  |
|  |  |  |  |  |  | High range |  |  |  |  |  |  |
| Band 1 as PCC (Note 1) |  | 20MHz |  | Low range |  | X2 |  | X2 |  | X2 |  |
|  |  |  |  | Mid range |  | - |  | - |  | X3 |  |  |
| Band 1 as SCC | 20MHz |  | Low range |  | - |  | - |  | X |  |
| Note 1: For Band 1 as PCC, the exceptions described in Table 7.3A.0-0bA are tested. Note 2: This is the case that the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. The appropriate Uplink RB allocation value according to Table 7.3A.0-0bB should be applied. Note 3: This is the case that the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. The appropriate Uplink RB allocation value according to Table 7.3A.0-0bB should be applied. |  |  |  |  |  |  |  |  |  |  |

< Unchanged tables omitted >

Table 7.3A.3.4.1-26: Test frequencies and Test CC combinations for CA_19A-21A

| CBW |  |  | Band 21 |  |
| --- | --- | --- | --- | --- |
|  |  |  | 10MHz | 15MHz |
|  |  |  | High range | Mid range |
| Band 19 | 10MHz | High range |  | X |
|  | 15MHz | Mid range | X | X |

Table 7.3A.3.4.1-26a: Test frequencies and Test CC combinations for CA_21A-28A

| CBW |  |  | Band 28 |
| --- | --- | --- | --- |
|  |  |  | 10MHz |
|  |  |  | High range |
| Band 21 | 10MHz | High range | X |
|  | 15MHz | Mid range | X |

< End of changes >