3GPP TSG-RAN5 Meeting #92-e	R5-215490

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1371 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Power Class 1.5 for LTE Band 41and NR Band n41 A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_B41_Bn41_PC29dBm-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of PC1.5 for n41 to Additional Maximum Power Reduction testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated minimum conformance requirements, initial conditions, test procedure, message contents and test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices transmitting 29dBm will not be tested for conformance to A-MPR requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3.2, 6.2.3.4.1, 6.2.3.4.2, 6.2.3.4.3.4, 6.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< unchanged sections skipped>>>

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09 not complete.

- PC1 requirements for NR operating bands other than Band n14 are not fully defined in RAN4 Rel-15 and Rel-16 specifications due to the lack of MPR requirements.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49 and NS_50 to all types of NR Power Class 2 and 3 UE release 15 and forward.

#### 6.2.3.3	Minimum conformance requirements

##### 6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 |  |  |  |
| NS_13 | 6.5.3.3.18 | n26 |  |  |  |
| NS_14 | 6.5.3.3.19 | n26 |  |  |  |
| NS_15 | 6.5.3.3.20 | n26 |  |  |  |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 15, 20, 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.26-1 | Table 6.2.3.3.26-1 |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.27-1 | Table 6.2.3.3.27-1 |
| NS_50 | 6.5.3.3.3.16 | n39 | 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

##### 6.2.3.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

Allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A2 relative to 23 dBm for power class 3, PC2 A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A1 relative to 23 dBm for power class 3, PC2 A3 relative to 26 dBm for power class 2, and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.3.2-2: A-MPR' values Access

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16-QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64-QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256-QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256-QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.2.

##### 6.2.3.3.3	A-MPR for NS_10

<<< unchanged sections skipped>>>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-26. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

Table 6.2.3.4.1-1: Test Configuration table for NS_06 and NS_35

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_35 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 16 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
|  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing |  | Modulation (NOTE 2) |  |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |
| 2 (Note 3) | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |
| 3 (Note 3) | 2496 + BWChannel /2 + |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Inner Full |
| 4 (Note 3) | MAX(10 MHz, 0.45 × BWChannel) |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |
| 5 (Note 3) | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Inner Full |
| 7 (Note 3) | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |
| 8 (Note 4) | Low |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |
| 9 (Note 4) | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |
| 10 (Note 4) | 2496 + BWChannel /2 + |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Inner Full |
| 11 (Note 4) | MAX(10 MHz, 0.45 × BWChannel) |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |
| 12 (Note 4) | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Right |
| 13 | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Inner Full |
| 14 (Note 4) | High |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |
| 15 | Low |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Left |
| 16 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Left |
| 17 | 2496 + BWChannel /2 + |  |  |  |  | DFT-s-OFDM QPSK |  |  | Inner Full |
| 18 | MAX(10 MHz, 0.45 × BWChannel) |  |  |  |  | DFT-s-OFDM QPSK |  |  | Outer Full |
| 19 | High |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Right |
| 20 | High |  |  |  |  | DFT-s-OFDM QPSK |  |  | Inner Full |
| 21 | High |  |  |  |  | DFT-s-OFDM QPSK |  |  | Outer Full |
| 22 | Low |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Left |
| 23 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Left |
| 24 | 2496 + BWChannel /2 + |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Inner Full |
| 25 | MAX(10 MHz, 0.45 × BWChannel) |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Outer Full |
| 26 | High |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Right |
| 27 | High |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Inner Full |
| 28 | High |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Outer Full |
| 29 | Low |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Left |
| 30 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Left |
| 31 | 2496 + BWChannel /2 + MAX(10 MHz, 0.45 × BWChannel) |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Outer Full |
| 32 | High |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Right |
| 33 | High |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Outer Full |
| 34 | Low |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Left |
| 35 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Left |
| 36 | 2496 + BWChannel /2 + MAX(10 MHz, 0.45 × BWChannel) |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Outer Full |
| 37 | High |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Right |
| 38 | High |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Outer Full |
| 39 | Low |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |
| 40 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |
| 41 | 2496 + BWChannel /2 + |  |  |  |  | CP-OFDM QPSK |  |  | Inner Full |
| 42 | MAX(10 MHz, 0.25 × BWChannel) |  |  |  |  | CP-OFDM QPSK |  |  | Outer Full |
| 43 | High |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Right |
| 44 | High |  |  |  |  | CP-OFDM QPSK |  |  | Inner Full |
| 45 | High |  |  |  |  | CP-OFDM QPSK |  |  | Outer Full |
| 46 | Low |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Left |
| 47 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Left |
| 48 | 2496 + BWChannel /2 + |  |  |  |  | CP-OFDM 16 QAM |  |  | Inner Full |
| 49 | MAX(10 MHz, 0.25 × BWChannel) |  |  |  |  | CP-OFDM 16 QAM |  |  | Outer Full |
| 50 | High |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Right |
| 51 | High |  |  |  |  | CP-OFDM 16 QAM |  |  | Inner Full |
| 52 | High |  |  |  |  | CP-OFDM 16 QAM |  |  | Outer Full |
| 53 | Low |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Left |
| 54 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Left |
| 55 | 2496 + BWChannel /2 + MAX(10 MHz, 0.25 × BWChannel) |  |  |  |  | CP-OFDM 64 QAM |  |  | Outer Full |
| 56 | High |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Right |
| 57 | High |  |  |  |  | CP-OFDM 64 QAM |  |  | Outer Full |
| 58 | Low |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Left |
| 59 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Left |
| 60 | 2496 + BWChannel /2 + MAX(10 MHz, 0.25 × BWChannel) |  |  |  |  | CP-OFDM 256 QAM |  |  | Outer Full |
| 61 | High |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Right |
| 62 | High |  |  |  |  | CP-OFDM 256 QAM |  |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. |  |  |  |  |  |  |

Table 6.2.3.4.1-3: Test Configuration table for NS_03, NS_03U and NS_100

<<< unchanged sections skipped>>>

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-30.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-30. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

4.	For network signalling value “NS_04” and UEs supporting Power Class 2 and Power Class 1.5, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.3.4.3.4-2.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.3.4.1-1 to Table 6.2.3.4.1-20, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1, with the following exceptions for each network signalling value.

For almost contiguous allocation testing, message contents are according to TS 38.508-1 [5] subclause 4.6.1 with the following exceptions:

Table 6.2.3.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| } |  |  |  |

6.2.3.4.3.1	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

<<< unchanged sections skipped>>>

6.2.3.4.3.4	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.4-1: AdditionalSpectrumEmission Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.2.3.4.3.4-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1.5 UE |

6.2.3.4.3.5	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

<<< unchanged sections skipped>>>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 2 | 28+TT | 17.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 10 | 26 | 0 | 0 | 0 | 26 | 2 | 2 | 28+TT | 23-TT |
| 11 | 26 | 0.5 | 0 | 0 | 25.5 | 2 | 2 | 28+TT | 22.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 13 | 26 | 0 | 0 | 0 | 26 | 2 | 2 | 28+TT | 23-TT |
| 14 | 26 | 0.5 | 0 | 0 | 25.5 | 2 | 2 | 28+TT | 22.5-TT |
| 15 | 26 | 0 | 6 | 0 | 20 | 2.5 | 2 | 28+TT | 17-TT |
| 16 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 17 | 26 | 0 | 0 | 0 | 26 | 2 | 2 | 28+TT | 23-TT |
| 18 | 26 | 1 | 0 | 0 | 25 | 2 | 2 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 20 | 26 | 0 | 0 | 0 | 26 | 2 | 2 | 28+TT | 23-TT |
| 21 | 26 | 1 | 0 | 0 | 25 | 2 | 2 | 28+TT | 22-TT |
| 22 | 26 | 0 | 6 | 0 | 20 | 2.5 | 2 | 28+TT | 17-TT |
| 23 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 24 | 26 | 1 | 0 | 0 | 25 | 2 | 2 | 28+TT | 22-TT |
| 25 | 26 | 2 | 0 | 0 | 24 | 2 | 2 | 28+TT | 21-TT |
| 26 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 27 | 26 | 1 | 0 | 0 | 25 | 2 | 2 | 28+TT | 22-TT |
| 28 | 26 | 2 | 0 | 0 | 24 | 2 | 2 | 28+TT | 21-TT |
| 29 | 26 | 0 | 6.5 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 30 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 31 | 26 | 2.5 | 0 | 0 | 23.5 | 2 | 2 | 28+TT | 20.5-TT |
| 32 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 33 | 26 | 2.5 | 0 | 0 | 23.5 | 2 | 2 | 28+TT | 20.5-TT |
| 34 | 26 | 0 | 8 | 0 | 18 | 4 | 2 | 28+TT | 14-TT |
| 35 | 26 | 4.5 | 0 | 0 | 21.5 | 2 | 2 | 28+TT | 18.5-TT |
| 36 | 26 | 4.50 | 0 | 0 | 21.5 | 2 | 2 | 28+TT | 18.5-TT |
| 37 | 26 | 4.5 | 0 | 0 | 21.5 | 2 | 2 | 28+TT | 18.5-TT |
| 38 | 26 | 4.5 | 0 | 0 | 21.5 | 2 | 2 | 28+TT | 18.5-TT |
| 39 | 26 | 0 | 7.5 | 0 | 18.5 | 4 | 2 | 28+TT | 14.5-TT |
| 40 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 41 | 26 | 1.5 | 0 | 0 | 24.5 | 2 | 2 | 28+TT | 21.5-TT |
| 42 | 26 | 3 | 0 | 0 | 23 | 2 | 2 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 44 | 26 | 1.5 | 0 | 0 | 24.5 | 2 | 2 | 28+TT | 21.5-TT |
| 45 | 26 | 3 | 0 | 0 | 23 | 2 | 2 | 28+TT | 20-TT |
| 46 | 26 | 0 | 7.5 | 0 | 18.5 | 4 | 2 | 28+TT | 14.5-TT |
| 47 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 48 | 26 | 2 | 0 | 0 | 24 | 2 | 2 | 28+TT | 21-TT |
| 49 | 26 | 3 | 0 | 0 | 23 | 2 | 2 | 28+TT | 20-TT |
| 50 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 51 | 26 | 2.0 | 0 | 0 | 24 | 2 | 2 | 28+TT | 21-TT |
| 52 | 26 | 3.0 | 0 | 0 | 23 | 2 | 2 | 28+TT | 20-TT |
| 53 | 26 | 0 | 7.5 | 0 | 18.5 | 4 | 2 | 28+TT | 14.5-TT |
| 54 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 55 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 56 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 57 | 26 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 19.5-TT |
| 58 | 26 | 0 | 10 | 0 | 16 | 5 | 2 | 28+TT | 11-TT |
| 59 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 60 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 61 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 62 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 1.5 and Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0 | 3.5 | 0 | 22.5 | 2 | 2 | 28+TT | 20.5-TT |
| 2 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 20.5-TT |
| 3 | 23 | -3 | 0.2 | 0 | 0 | 25.8 | 2 | 2 | 28+TT | 23.8-TT |
| 4 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2 | 2 | 28+TT | 22.8-TT |
| 5 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2 | 2 | 28+TT | 20.5-TT |
| 6 | 23 | -3 | 0.2 | 0 | 0 | 25.8 | 2 | 2 | 28+TT | 23.8-TT |
| 7 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2 | 2 | 28+TT | 22.8-TT |
| 8 | 23 | 0 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2 | 2 | 25+TT | 16-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 23 | 2 | 2 | 25+TT | 20.5-TT |
| 11 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2 | 2 | 25+TT | 21-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 3.5 | 2 | 25+TT | 20.5-TT |
| 13 | 23 | 0 | 0 | 0 | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 14 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2 | 2 | 25+TT | 20.5-TT |
| 15 | 23 | 0 | 0 | 4 | 0 | 19 | 2 | 2 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 1 | 0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 17 | 23 | 0 | 0 | 0 | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 19 | 23 | 0 | 1 | 0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 20 | 23 | 0 | 0 | 0 | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 21 | 23 | 0 | 1 | 0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 22 | 23 | 0 | 0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 0 | 2 | 0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 24 | 23 | 0 | 1 | 0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 25 | 23 | 0 | 2 | 0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 26 | 23 | 0 | 2 | 0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 27 | 23 | 0 | 1 | 0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 28 | 23 | 0 | 2 | 0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 29 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 31 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 32 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 33 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 34 | 23 | 0 | 0 | 6.0 | 0 | 17 | 5 | 2 | 25+TT | 12+TT |
| 35 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 38 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 0 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 40 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 41 | 23 | 0 | 1.5 | 0 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 43 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 44 | 23 | 0 | 1.5 | 0 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 0 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 48 | 23 | 0 | 2 | 0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 49 | 23 | 0 | 3 | 0 | 0 | 20 | 2.55 | 2 | 25+TT | 17.5-TT |
| 50 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 51 | 23 | 0 | 2 | 0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 52 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 53 | 23 | 0 | 0 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 55 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 56 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 57 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 58 | 23 | 0 | 0 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 59 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 61 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 62 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n25, n66, n70 and n86

<<< unchanged sections skipped>>>

### 6.2.4	Configured transmitted power