3GPP TSG-RAN5 Meeting #92-e	R5-215512

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2449 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current Prose the Service REQ content at Steps 2B amd 11B is checked for presence of IE PDU Session Status. This doesn’t affect the test purpose, and does not add any value. Similarly, the Prose specifies special message content for Service Accept at Steps 4 and 13 and this is unnecessary as this doesn’t add any value o the TP. It is proposed to void tables 10.1.1.1.3.3-1 and Table 10.1.1.1.3.3-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Voided tables 10.1.1.1.3.3-1 and Table 10.1.1.1.3.3-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase implementation is not optimized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 10.1.1.1	PDU session authentication and authorization / During the UE-requested PDU session procedure

10.1.1.1.1	Test Purpose (TP)

(1)

with { UE is establishing UE-requested PDU session by sending PDU Session establishment Request message }

ensure that {

when { UE receives a PDU SESSION AUTHENTICATION COMMAND message }

then { UE transmits a PDU SESSION AUTHENTICATION COMPLETE message }

}

(2)

with { PDU session authentication and authorization procedure is performed during the UE-requested PDU session establishment procedure }

ensure that {

when { UE receives EAP-failure message in the PDU SESSION ESTABLISHMENT REJECT message }

then { UE consider that the PDU session is not established }

}

(3)

with { PDU session authentication and authorization procedure is performed during the UE-requested PDU session establishment procedure }

ensure that {

when { UE receives EAP-success message in the PDU SESSION ESTABLISHMENT ACCEPT message }

then { UE consider that the PDU session is established }

}

10.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 6.3.1.2.1, 6.3.1.2.2 and 6.4.1.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501 clause 6.3.1.2.1]

In order to initiate the PDU EAP message reliable transport procedure, the SMF shall create a PDU SESSION AUTHENTICATION COMMAND message.

The SMF shall set the PTI IE of the PDU SESSION AUTHENTICATION COMMAND message to "No procedure transaction identity assigned".

The SMF shall set the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message to the EAP-request message provided by the DN or generated locally.

The SMF shall send the PDU SESSION AUTHENTICATION COMMAND message, and the SMF shall start timer T3590 (see example in figure 6.3.1.1).

Upon receipt of a PDU SESSION AUTHENTICATION COMMAND message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE passes to the upper layers the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message. Apart from this action, the authentication and authorization procedure initiated by the DN is transparent to the 5GSM layer of the UE.

[TS 24.501 clause 6.3.1.2.2]

When the upper layers provide an EAP-response message responding to the received EAP-request message, the UE shall create a PDU SESSION AUTHENTICATION COMPLETE message.

The UE shall set the EAP message IE of the PDU SESSION AUTHENTICATION COMPLETE message to the EAP-response message.

The UE shall transport the PDU SESSION AUTHENTICATION COMPLETE message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5. Apart from this action, the authentication and authorization procedure initiated by the DN is transparent to the 5GSM layer of the UE.

Upon receipt of a PDU SESSION AUTHENTICATION COMPLETE message, the SMF shall stop timer T3590 and provides the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMPLETE message to the DN or handles it locally.

[TS 24.501 clause 6.4.1.4]

If the connectivity with the requested DN is rejected by the network, the SMF shall create a SM PDU SESSION ESTABLISHMENT REJECT message.

The SMF shall set the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message to indicate the reason for rejecting the PDU session establishment.

The 5GSM cause IE typically indicates one of the following SM cause values:

#8	operator determined barring;

#26	insufficient resources;

#27	missing or unknown DNN;

#28	unknown PDU session type;

#29	user authentication or authorization failed;

10.1.1.1.3	Test description

10.1.1.1.3.1 	Pre-test conditions

System Simulator:

NGC Cell A

UE:

None.

Preamble:

The UE is in state 1N-A with PDU session Active state using the generic procedure NR RRC_IDLE according to TS 38.508-1 [4].

10.1.1.1.3.2	Test procedure sequence

Table 10.1.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to request connectivity to an additional PDU session. (see Note 1) | - | - | - | - |
| 2-2A | Steps 2-3 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. | - | - | - | - |
| 2B | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message with service type IE set to “signalling”. | --> | SERVICE REQUEST | - | - |
| 3-3A | Steps 5 and 6 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. | - | - | - | - |
| 4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 4A | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message to request an additional PDU session. Note: PDU SESSION ESTABLISHMENT REQUEST is included in UL NAS transport. UL NAS transport message is included in dedicatedNAS-Message of ULInformationTransfer message. DNN information is included in UL NAS transport message. | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 6 | The SS transmits PDU SESSION AUTHENTICATION COMMAND including an EAP-Request message. | <-- | PDU SESSION AUTHENTICATION COMMAND |  |  |
| 7 | Check: Does the UE transmit a PDU SESSION AUTHENTICATION COMPLETE containing EAP-Response message? | --> | PDU SESSION AUTHENTICATION COMPLETE | 1 | P |
| 8 | The SS transmits PDU SESSION ESTABLISHMENT REJECT message with 5GSM cause #29 including an EAP-Failure message. | <-- | PDU SESSION ESTABLISHMENT REJECT |  |  |
| 9 | The SS releases the RRC connection. | - |  | - | - |
| 10 | Cause the UE to request connectivity to an additional PDU session. (see Note 1) | - | - | - | - |
| 11-11A | Steps 2-3 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. | - | - | - | - |
| 11B | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message with service type IE set to “signalling”. | --> | SERVICE REQUEST | - | - |
| 12-12A | Steps 5 and 6 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. | - | - | - | - |
| 13 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 13A | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 14 | The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message to request an additional PDU session. Note: PDU SESSION ESTABLISHMENT REQUEST is included in UL NAS transport. UL NAS transport message is included in dedicatedNAS-Message of ULInformationTransfer message DNN information is included in UL NAS transport message. | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 2 | P |
| 15 | The SS transmits PDU SESSION AUTHENTICATION COMMAND including an EAP-Request message. | <-- | PDU SESSION AUTHENTICATION COMMAND |  |  |
| 16 | Check: Does the UE transmit a PDU SESSION AUTHENTICATION COMPLETE containing EAP-Response message? | --> | PDU SESSION AUTHENTICATION COMPLETE | - | - |
| 17 | The SS transmits RRCReconfiguration message containing PDU SESSION ESTABLISHMENT ACCEPT message containing an EAP-Success message. | <-- | PDU SESSION ESTABLISHMENT ACCEPT |  |  |
| 18 | The UE transmits RRCReconfigurationComplete message to confirm the establishment of DRB. | - | - | 3 | P |
| - | EXCEPTION: Step 19a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 19a1 | If initiated by the UE, the generic procedure for IP address allocation in the user plane, specified in subclause 4.5.6, takes place performing IP address allocation in the user plane. | - | - | - | - |
| 20 | The SS releases the RRC connection. | - |  | - | - |
| Note 1: The request of connectivity to an additional PDU session may be performed by MMI or AT command +CGACT. |  |  |  |  |  |

10.1.1.1.3.3	Specific message contents

Table 10.1.1.1.3.3-1Void

Table 10.1.1.1.3.3-2:  Void

Table 10.1.1.1.3.3-3: PDU SESSION ESTABLISHMENT REQUEST (step 5 and 14, Table 10.1.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | PSI-1 | UE assigns a particular PSI not yet used between 1 and 15 |  |
| PTI |  | PTI-1 | UE assigns a particular PTI not yet used between 1 and 254 |  |

Table 10.1.1.1.3.3-4: UL NAS Transport (step 5 and 14, Table 10.1.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0001’B | N1 SM information |  |
| PDU session ID |  | PSI-1 |  |  |
| Request type |  | ‘001’B | Initial request |  |
| S-NSSAI |  | Not Present |  |  |
| DNN |  | DNN-1 (New DNN name) | The requested DNN is different from default DNN. |  |

Table 10.1.1.1.3.3-5: PDU SESSION ESTABLISHMENT REJECT (step 8, Table 10.1.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | PSI-1 |  |  |
| PTI |  | PTI-1 |  |  |
| 5GSM cause |  | ‘00011 101’ | User authentication or authorization failed |  |

Table 10.1.1.1.3.3-6: PDU SESSION ESTABLISHMENT ACCEPT (step 17, Table 10.1.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | PSI-1 |  |  |
| PTI |  | PTI-1 |  |  |
| Authorized QoS rules |  |  |  |  |
| QoS rule |  |  |  |  |
| QoS rule identifier |  | ‘0000 0001’B |  |  |
| Rule operation code |  | ‘001’B | Create new QoS rule |  |
| DQR bit |  | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters |  | ‘0001’B | 1 packet filter |  |
| Packet filter list |  | See table 4.8.2.1-1 | Packet filter list #1 |  |
| Packet filter direction |  | ‘11’B | bidirectional |  |
| Packet filter identifier |  | ‘0000’B | Id 0 |  |
| Component type 1 ID |  | ‘0000 0001’B | Match-all type |  |
| QoS rule precedence |  | ‘0000 0000’B | 0 |  |
| QoS flow identifier (QFI) |  | ’00 0011’B | QFI 3 |  |
| EAP message |  |  |  |  |
| QoS flow description |  |  |  |  |
| QFI |  | ‘00 0011’B | QFI 3 |  |
| Operation code |  | ‘001’B | Create new QoS flow description |  |
| E bit |  | ‘1’B | Parameters list is included |  |
| Number of parameters |  | ’00 0001’B | 1 parameters |  |
| 5QI |  | ‘0000 1001’B | 5QI 9 |  |
| DNN |  | DNN-1 |  |  |