3GPP TSG-RAN5 Meeting #92-e                                                      R5-215527

Electronic Meeting, 17th May – 28th May

Title: 		        Occupied bandwidth for Intra-Band Contiguous EN-DC measurement

uncertainty

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.2.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Current test specifications TS 38.521-3 ([1]) and Work plan for 5GS_NR_LTE-UEConTest work item ([2]) consider test 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC incomplete due to measurement uncertainties for ENBW > 100 MHz:

This document proposes a way forward to complete measurement uncertainty pending for such test case.

# 2.	Discussion

6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC is defined as an LTE non-anchor agnostic test case.

Measurement uncertainties for 6.5B.1.1 has been defined as 1.5% of aggregated channel bandwidth (refer to section F.1.2 in [1]).

On the other hand, there is no Intra-band contiguous UL band combination including LTE UL CA. This means that only one LTE CC and one NR CC are required at this stage, i.e maximum ENBW = 120 MHz.

In order to define measurement uncertainties for wider ENBW, corresponding values for FR2 SA could be considered: we can see that measurement uncertainties for Occupied bandwidths FR2 standalone test case have been defined as 1.2% of the channel bandwidths for channel bandwidth equal to 200 MHz.

Hence, it seems reasonable to extend current measurement uncertainty definition from 100 ENBW=100MHz up to ENBW=200MHz in case new band combinations with higher LTE UL CA is defined in the future.

Proposal 1: Extend current measurement uncertainty definition from 100 ENBW=100MHz up to ENBW=200MHz in case new band combinations with higher LTE UL CA is defined in the future.

Couple of CR including this change are defined in [4] and [5].

# 3.	Conclusion

This document proposes a way forward to complete measurement uncertainty for Occupied bandwidth for Intra-Band Contiguous EN-DC. The following proposal is made:

Proposal 1: Extend current measurement uncertainty definition from 100 ENBW=100MHz up to ENBW=200MHz in case new band combinations with higher LTE UL CA is defined in the future.

# 4.	References

[1] 38.521-3 v17.1.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[2] R5-212657, “WP UE Conformance Test Aspects - 5G system with NR and LTE”, Ericsson, RAN5#91-e

[3] 38.521-2 v16.8.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[4] R5-215528, “Occupied bandwidth for Intra-Band Contiguous EN-DC measurement uncertainty”, Keysight Technologies, RAN5#92-e

[5] R5-215529, “Occupied bandwidth for Intra-Band Contiguous EN-DC measurement uncertainty”, Keysight Technologies, RAN5#92-e