3GPP TSG-RAN WG5 Meeting #92-e	R5-215747

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0181 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.7.1 - MCPTT Group Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to 24.481 clause 7.2.8 an MCPTT group document needs to contain a <supported-services> element - In context of test case 5.2 the group document for group B shall not contain user A (to avoid potential notifications for the client under test) and it shall contain at least one user not being contained in group A (with regard to creation of the temporary group) - There is no reason to require that the group document for group creation contains more XML elements than needed; in general for test case 5.2 it should be sufficient that the document contains only the list of group members and the <supported-services> element. - Table 5.5.7.1-3: According to 24.481 clause 6.3.14.2 the GMOP group regroup creation request shall contain a group document with an <on-network-temporary> element which does not have any temporary-MCPTT-group-requestor attribute (the temporary-MCPTT-group-requestor is added by the GMS serving the client after the GMS has identified the client from the access token) - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - <supported-services> element added to Table 5.5.7.1-1 and Table 5.5.7.1-3 - Table 5.5.7.1-2: Group B with user C and user D - Unnecessary elements removed from Table 5.5.7.1-2 - Temporary-MCPTT-group-requestor removed from Table 5.5.7.1-3 - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid group documents |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See also document R5w210211 discussed at TTCN Workshop #54 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-214652 and the outcome of 1 revision to this document |  |  |  |  |  |  |  |  |

#### 5.5.7.1	MCPTT Group Configuration

The structure of a group configuration document is specified in TS 24.481 [11] clause 7, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 6.3.

The structure of the configuration document is based on several XML schemas. To distinguish the schemas the prefixes of their corresponding name spaces are used in the 'Information Element' column as according to table 7.2.2-2 of TS 24.481 [11].

Table 5.5.7.1-1: MCPTT Group Configuration Defaults

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| list-service[1] |  |  |  | Group 1 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_Group_A_ID |  | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 |  | TS 24.483 [13] clause 6.2.7 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 |  | TS 24.483 [13] clause 6.2.8 |  |  |  |
| list |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  | group member 1 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_A |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "3" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCPTT_User_A_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| entry[2] |  |  |  | group member 2 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_B |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "2" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCPTT_User_B_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| entry[3] |  |  |  | group member 3 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_C |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "1" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCPTT_User_C_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| cp:ruleset |  |  |  |  |  |  |  |  |  |
| cp:rule |  |  |  |  |  |  |  |  |  |
| cp:id attribute |  | "rule1" |  |  |  |  |  |  |  |
| cp:actions |  |  |  |  |  |  |  |  |  |
| cp:on-network-allow-getting-member-list |  | "true" |  | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures |  |  |  |  |  |
| cp:allow-initiate-conference |  | "true" |  |  |  |  |  |  |  |
| cp:join-handling |  | "true" |  |  |  |  |  |  |  |
| cp:allow-MCPTT-emergency-call |  | "true" |  | Indicates whether an MCPTT emergency group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 6.2.19 |  |  |  |
| cp:allow-imminent-peril-call |  | "true" |  | Indicates whether an MCPTT imminent peril group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 6.2.20 |  |  |  |
| cp:allow-MCPTT-emergency-alert |  | "true" |  | Indicates whether an MCPTT emergency alert is possible on the MCPTT group |  | TS 24.483 [13] clause 6.2.21 |  |  |  |
| cp:on-network-allow-getting-affiliation-list |  | "true" |  | Indicates that the identity is allowed to get the list of MCPTT users affiliated to the MCPTT group in on-network MCPTT procedures |  |  |  |  |  |
| cp:on-network-allow-conference-state |  | "true" |  | indicates that the identity is allowed to subscribe to the conference event package of an MCPTT group session of the MCPTT group in on-network MCPTT procedures |  |  |  |  |  |
| oxe:supported-services |  |  |  |  |  |  |  |  |  |
| oxe:service |  |  |  |  |  | TS 24.481 [11] |  |  |  |
| oxe:enabler |  | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |  |  |  |  |
| oxe:group-media |  |  |  |  |  |  |  |  |  |
| mcpttgi:mcptt-speech |  | Present |  |  |  |  |  |  |  |
| mcpttgi:owner |  | px_MCPTT_Group_A_Owner_Organization |  | Group’s owner (Mission Critical Organisation). |  | TS 24.483 [13] clause 6.2.15 |  |  |  |
| mcpttgi:preferred-voice-encodings |  |  |  |  |  |  |  |  |  |
| mcpttgi:encoding- |  |  |  |  |  |  |  |  |  |
| mcpttgi:name[1] |  | px_MCPTT_Group_A_preferred_VCodec |  | Preferred voice codec is a RTP payload. MCPTT clients shall support the AMR-WB codec. |  | RFC 4566 [27] TS 26.171 [66] TS 24.483 [13] clause 6.2.16 |  |  |  |
| mcpttgi:level-within-group-hierarchy |  | "0" |  | Indicates the level within a group hierarchy (only applicable for group-broadcast group). |  | TS 24.483 [13] clause 6.2.17 |  |  |  |
| mcpttgi:level-within-user-hierarchy |  | "0" |  | Indicates the level within user hierarchy (only applicable for user-broadcast group). |  | TS 24.483 [13] clause 6.2.18 |  |  |  |
| mcpttgi:protect-media |  | "true" |  | Indicates whether confidentiality and integrity of media is required on the MCPTT group |  | TS 24.483 [13] clause 6.2.22 |  |  |  |
| mcpttgi:protect-floor-control-signalling |  | "true" |  | Indicates whether confidentiality and integrity of floor control signalling is required on the MCPTT group |  | TS 24.483 [13] clause 6.2.23 |  |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id |  | tsc_MCPTT_Group_A_ProSeLayer2GroupID |  | Indicates the Prose layer-2 group ID |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |  |  |
| mcpttgi:off-network-IP-multicast-address |  | "0.0.0.0" |  | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |  |  |
| mcpttgi:off-network-ProSe-relay-service-code |  | "123456" |  | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |  |  |
| mcpttgi:off-network-in-progress-emergency-state-cancellation-timeout |  | "PT18H12M15S" |  | Indicates the timeout value for the cancellation of an in progress emergency for an MCPTT group call. "PT18H12M15S" corresponds to 65535 seconds what is maximum allowed value according to TS 24.483 [13] |  | TS 24.483 [13] clause 6.2.31 |  |  |  |
| mcpttgi:off-network-in-progress-imminent-peril-state-cancellation-timeout |  | "PT18H12M15S" |  | Indicates the timeout value for the cancellation of an in progress imminent peril for an MCPTT group call. "PT18H12M15S" corresponds to 65535 seconds what is maximum allowed value according to TS 24.483 [13] |  | TS 24.483 [13] clause 6.2.32 |  |  |  |
| mcpttgi:off-network-hang-timer |  | "PT5S" |  | Indicates the group call hang timer. "PT5S" corresponds to 5 seconds |  | TS 24.483 [13] clause 6.2.33 |  |  |  |
| mcpttgi:off-network-maximum-duration |  | "PT1M" |  | Indicates the max duration of group calls. "PT1M" corresponds to 1 minute |  | TS 24.483 [13] clause 6.2.34 |  |  |  |
| mcpttgi:off-network-queue-usage |  | "true" |  | Indicates if queuing is enabled or not |  | TS 24.483 [13] clause 6.2.34A |  |  |  |
| mcpttgi:off-network-ProSe-signalling-PPPP |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.36 |  |  |  |
| mcpttgi:off-network-ProSe-media-PPPP |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.37 |  |  |  |
| mcpttgi:off-network-ProSe-emergency-call-signalling-PPPP |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.38 |  |  |  |
| mcpttgi:off-network-ProSe-emergency-call-media-PPPP |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.39 |  |  |  |
| mcpttgi:off-network-ProSe-imminent-peril-call-signalling-PPPP |  | "7" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.40 |  |  |  |
| mcpttgi:off-network-ProSe-imminent-peril-call-media-PPPP |  | "7" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.41 |  |  |  |
| mcpttgi:multi-talker-control |  | "false" |  | "true" indicates that multi-talker control is enabled for the group "false" indicates that multi-talker control is disabled for the group |  |  |  |  |  |
| mcpttgi:max-number-simultaneous-talkers |  | "1" |  | Indicates the maximum number of parallel talkers in a MCPTT group session in on-network MCPTT procedures |  |  |  |  |  |
| mcpttgi:audio-mixing-entity |  | Not present |  | Absence of the <audio-mixing-entity> element indicates that audio mixing is performed in the network |  |  |  |  |  |

Table 5.5.7.1-2: Group Creation Document

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service [1] |  |  |  |  |
| uri-attribute | px_MCPTT_Group_B_ID | uri of the MCPTT group | TS 24.481 [11] |  |
| display-name | px_MCPTT_Group_B_name | group display name | TS 24.481 [11] |  |
| list |  |  |  |  |
| entry[1] |  | User-C |  |  |
| uri-attribute | px_MCPTT_ID_User_C | User ID allowed to participate in this group | TS 24.481 [11] |  |
| display-name | Not present | User display name | TS 24.481 [11] |  |
| entry[2] |  | User-D |  |  |
| uri-attribute | px_MCPTT_ID_User_D | User ID allowed to participate in this group | TS 24.481 [11] |  |
| display-name | Not present | User display name | TS 24.481 [11] |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  | TS 24.481 [11] |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| oxe:group-media |  |  |  |  |
| mcpttgi:mcptt-speech | Present |  |  |  |

Table 5.5.7.1-3: Temporary Group Creation Document

| Derivation Path: TS 24.481 [11] clause 7.3.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| gmop:document |  |  |  |  |
| gmop:request |  |  |  |  |
| gmop:group-regroup-creation |  |  |  |  |
| group |  |  |  |  |
| list-service[1] |  |  |  |  |
| uri attribute | px_MCPTT_Group_T_ID | MCS temporary group identity |  |  |
| display-name | Not present |  |  |  |
| list | Not present | Temporary group contains constituent groups but no group members |  |  |
| mcpttgi:on-network-temporary |  |  | TS 24.481 [11] |  |
| constituent-MCPTT-group-IDs |  |  |  |  |
| constituent-MCPTT-group-ID[1] | px_MCPTT_Group_A_ID | MCS group ID of a constituent MCS group of the temporary MCS group |  |  |
| constituent-MCPTT-group-ID[2] | px_MCPTT_Group_B_ID | MCS group ID of a constituent MCS group of the temporary MCS group |  |  |
| oxe:supported-services |  |  |  |  |
| oxe:service |  |  | TS 24.481 [11] |  |
| oxe:enabler | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| oxe:group-media |  |  |  |  |
| mcpttgi:mcptt-speech | Present |  |  |  |