3GPP TSG-RAN5 Meeting #92-e	R5-216021

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0385 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean-up of parameter settings and message contents in 8.4.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Table 8.4.2.2.1-1 TDD Slot Configuration FR1.120-2 is an editorial error from FR2.120-2. CSI-IM resource Type is not aligned to CoreSpec TS38.101-4 Aperiodic Report Slot Offset is not aligned to CoreSpec TS38.101-4 Missing Note1 will lead to vague measurement channels to be used - 8.4.2.2.1.4.3_1 Test case uses FR2.120-2. Based on their config pattern, default SR config in TS 38.508-1 will fall on DL slot instead of UL slot. Message Contents will be missing and remained TBD. - Table 8.4.2.2.1.5-1 The test requirement of  in Test 1, 0.99 is correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 8.4.2.2.1-1 Change the table number from 8.4.2.2.1-1 to 8.4.2.2.1.3-1. Modified TDD Slot Configuration from FR1.120-2 to FR2.120-2. Changed CSI-IM resource Type: Periodic -> Aperiodic Changed Aperiodic Report Slot Offset: 7 -> 6 Added Note1 which indicates what measurement channels to be used for the tests - Table 8.4.2.2.1-2 Change the table number from 8.4.2.2.1-2 to 8.4.2.2.1.3-2. - 8.4.2.2.1.4.3_1 Added schedulingRequestResourceConfig message exception. Removed TBD as TS38.508-1 clause 5.4.2.6 is updated for FR2 RI Added different message contents accordingly as set by parameter table - Table 8.4.2.2.1.5-1 Modified the test requirement of  in Test 1 from 0.09 to 0.99. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 CR dependent: R4-2111893 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is outcome of 1 revision of R5-215347 r1: Add declaration of RAN4 dependency |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

8.4	Reporting of Rank Indicator (RI)

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

8.4.1	1RX requirements

(Void)

8.4.2	2RX requirements

8.4.2.1	FDD

(Void)

8.4.2.2	TDD

8.4.2.2.1	2Rx TDD FR2 RI reporting for both SA and NSA

-8.4.2.2.1.1	Test Purpose

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

8.4.2.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

8.4.2.2.1.3	Minimum requirement

The minimum performance requirement in Table 8.4.2.2.1.3-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 8.4.2.2.1.3-1, and using the downlink physical channels specified in Annex C.2.2, the minimum requirements are specified in Table 8.4.2.2.1.3-2.

Table 8.4.2.2.1.3-1: RI Test (TDD)

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 | 100 | 100 |
| Subcarrier spacing |  |  | kHz | 120 | 120 | 120 |
| Duplex Mode |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 | FR2.120-2 | FR2.120-2 |
| SNR |  |  | dB | 0 | 16 | 16 |
| Propagation channel |  |  |  | TDLA30-35 | TDLA30-35 | TDLA30-35 |
| Antenna configuration |  |  |  | ULA Low 2x2 | ULA Low 2x2 | XP High 2x2 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 4, (8,-) | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | CSI-RS interval and offset |  | slot | 8/1 | 8/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig interval and offset |  | slot | Not configured | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  |  | 0 | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | CSI-IM RE pattern |  |  | Pattern 1 | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8,13) | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured | Not configured | Not configured |
| ReportConfigType |  |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  |  | Table 1 | Table 1 | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  | 111111111 | 111111111] | 111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 6 | 6 | 6 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 1.375 | 1.375 | 1.375 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |
| Note 1: Measurements channels are specified in Table A.4-1. TBS.1-1 is used for Rank 1 case. TBS.1-2 is used for Rank 2 case. |  |  |  |  |  |  |

Table 8.4.2.2.1.3-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 1.05 |
| 2 | 1.0 | N/A | N/A |

The normative reference for this requirement is TS 38.101-4 [5] clause 8.4.2.2.

8.4.2.2.1.4	Test Description

8.4.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE antenna is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 8.1.2-1 and Table 8.4.2.2.1.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 8.4.2.2.1.4.3.

8.4.2.2.1.4.2	Test Procedure

1.	Set the UE in a direction that satisfies the 3 normative criteria specified in Annex H.0. If no direction found mark the test as inconclusive.

2.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table 8.4.2.2.1.3-1 as appropriate. Measure theaccording to Annex G.3.3.

3.	The SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and fixed RI as defined in Table 8.4.2.2.1.3-1. The SS sends downlink MAC padding bits on the DL RMC.

4.	Propagation conditions are set according to Annex B.2.

5.	The SS shall transmit an RRC Connection Reconfiguration message to set codebookSubsetRestriction as for UE reported RI according to Table 8.4.2.2.1.3-1.

6.	The UE shall transmit RRC Connection Reconfiguration Complete message.

7.	Propagation conditions are set according to Table 8.4.2.2.1.3-1.

8.	The SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC. Measure according to Annex G.3.3.
If the ratio ( / ) satisfies the requirement in Table 8.4.2.2.1.5-1, then pass the UE for this test and go to step 9. Otherwise, declare a FAIL verdict.

9.	If all tests have not been done, then repeat the same procedure (steps 1 to 8) with test conditions according to the Table 8.4.2.2.1.3-2 for the other Tests as appropriate. Otherwise, declare a PASS verdict.

8.4.2.2.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2 with the following exceptions:

8.4.2.2.1.4.3_1	Message exceptions for SA

Table 8.4.2.2.1.4.3_1-1: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| Sl80 | 7 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.2.1.4.3_1-2: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | Aperiodic |  | CSI-RS for CSI Acquisition, CSI-IM-Resource |
|  | Periodic |  | CSI-RS for Tracking or Beam Refinement |
| } |  |  |  |

Table 8.4.2.2.1.4.3_1-3: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.2, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| Two SEQUENCE { |  |  |  |
| twoTX-CodebookSubsetRestriction | 010000 | Fixed rank 2 | Test 1 |
|  | 000011 | Fixed rank 1 | Test 2, Test 3 |
|  | 010011 | Following rank | Test 1, Test 2, Test 3 |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 11111111 | Non restriction |  |

Table 8.4.2.2.1.4.3_1-4: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 6 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands9 | 111111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| subbandSize | value2 |  |  |
| } |  |  |  |

8.4.2.2.1.4.3_2	Message exceptions for NSA

Same as 8.4.2.2.1.4.3_1.

8.4.2.2.1.5	Test Requirements

Table 8.4.2.2.1.5-1: Test Requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.04 | 1.04 |
| 2 | 0.99 | N/A | N/A |

<<End of change>>