3GPP TSG-RAN5 Meeting #92-e	R5-216096

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0336 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of FR2 demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Achievable SNRBB needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SNRBB and related appilicability of test points have been updated in clauses 7.1.1_1 and 8.1.1_1. It has been clarified that SNRBB is valid for modulations up to 64 QAM in table headers and associated editor’s note for modulations above 64 QAM has been added. Header of Table 7.1.1_1-2 and Table 8.1.1_1-2 has been corrected. References to Tables 7.1.1_1-2 and 8.1.1_1-2 have been added in the initial conditions of respective test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testability and assumption of achievable SNR will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1, 7.2.2.2.1_1.3.1, 7.3.2.2.1.4.1, 7.3.2.2.2.4.1, 8.1.1_1, 8.2.2.2.1.1.4.1, 8.2.2.2.2.1.4.1, 8.4.2.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper in R5-214202. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of three revisions of R5-214203. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.1.1_1	Applicability of test requirements due to maximum achievable SNR

### Editor’s note: The applicability of test requirements with a modulation above 64 QAM is FFS.

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: Current assumption of maximum testable SNRBB under fading conditions for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | [24.0] | [20.8] | [17.8] |
| n258 mid | [24.0] | [20.8] | [17.8] |
| n259 mid | [13.6] | [10.3] | [7.0] |
| n260 mid | [17.7] | [14.5] | [11.4] |
| n261 mid | [24.0] | [20.8] | [17.8] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | - | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | - | - | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | - | x | x |
|  | 2-4 | 100 | Yes | 15.8 | x | x | - | - | x |
|  | 2-5 | 50 | Yes | 16 | x | x | - | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | - | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | - | x |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |

< Unchanged sections omitted >

7.2.2.2.1_1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 7.2-1 and Table 7.2.2.2.1.0-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.2.2.2.1.4.3.

< Unchanged sections omitted >

7.2.2.2.1_2.3	Test Description

Same test description as in clause 7.2.2.2.1_1.3 with following exception:

-	Table 7.2.2.2.1_2.4-1 instead of Table 7.2.2.2.1_1.4-1

< Unchanged sections omitted >

7.3.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 7.3-1 and Table 7.3.2.2.1.3-1as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.3.2.2.1.4.3.

< Unchanged sections omitted >

7.3.2.2.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 7.3-1 and Table 7.3.2.2.2.3-1as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.3.2.2.1.4.3.

< Unchanged sections omitted >

### 8.1.1_1	Applicability of test requirements due to maximum achievable SNR

### Editor’s note: The applicability of test requirements with a modulation above 64 QAM is FFS.

The current assumption of maximum testable SNRBB for PC3, Quiet Zone size ≤ 30 cm under fading conditions is specified in Table 7.1.1_1-1.

The current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm without fading conditions is specified in Table 8.1.1_1-1.

Table 8.1.1_1-1: Current assumption of maximum testable SNRBB without fading for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | [28.7] | [25.5] | [22.5] |
| n258 mid | [28.7] | [25.5] | [22.5] |
| n259 mid | [18.4] | [15.2] | [12.1] |
| n260 mid | [22.5] | [19.3] | [16.3] |
| n261 mid | [28.7] | [25.5] | [22.5] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 8.1.1_1-2 for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 8.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 8.2.2.2.1.1 | 1 | 100 | No | 9 | x | x | x | x | x |
|  | 2 | 100 | No | 15 | x | x | x | x | x |
| 8.2.2.2.2.1 | 1 | 100 | Yes | 7 | x | x | x | x | x |
|  | 2 | 100 | Yes | 13 | x | x | - | x | x |
| 8.4.2.2.1 | 1 | 100 | Yes | 0 | x | x | x | x | x |
|  | 2 | 100 | Yes | 16 | x | x | - | - | x |
|  | 3 | 100 | Yes | 16 | x | x | - | - | x |

< Unchanged sections omitted >

8.2.2.2.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, AWGN noise source and the UE antenna is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 8.1.2-1 and Table 8.2.2.2.1.1.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 8.2.2.2.1. 1.4.3.

< Unchanged sections omitted >

8.2.2.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE antenna is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 8.1.2-1 and Table 8.2.2.2.2.1.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 8.2.2.2.2.1.4.3.

< Unchanged sections omitted >

8.4.2.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of 38.521-2.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 8.1.2-1 and Table 8.4.2.2.1-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 8.4.2.2.1.4.3.

< End of changes >