3GPP TSG-RAN WG5 Meeting #92-e	R5-214676r2

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0206 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCPTT test case 6.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Result of discussion paper about Pre test conditions R5-213432 - use of a GNSS simulator – no specific location needs to be reported to fulfil the test purpose. - SIP 200 (OK) to SIP MESSAGE are missing - Specific message content contradicts test purpose - Incorrect naming in specific message content - According to 24.379 clause 13.2.2 and clause 13.2.3 the participating MCPTT function shall include MCPTT Info with <mcptt-request-uri> element - As the test case does not use any emergency condition, the setting of the EmergencyLocationInformation in the location-information configuration as steps 1 and 4 is not relavant. - minimumIntervalLength and TriggeringCriteria are mandatory in Location-Info for configuration - The timers started at step 1A and step 3B are not used for anything but restarted at step 3A; on the other hand the Wait timer for step 1C may be started together with the PeriodicReport timer (i.e. before connection release). - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pre test conditions adapted to decision in R5-213432: “Test case can be implemented and verified without use of a GNSS simulator when longitude and latitude are "any value" for Location-Info sent by the UE. Step 0 removed as location coordinates are not relevant for the test case - SIP 200 (OK) message added - Missing messages added - Specific message content adapted and corrected - MCPTT info added to SIP MESSAGE at steps 1, 2, 4 and 5 - EmergencyLocationInformation kept as per default message content in tables 6.3.2.3.3-8 and 6.3.2.3.3-8 - minimumIntervalLength and TriggeringCriteria as per default message content in table 6.3.2.3.3-8 - start/stop of timers corrected - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test case specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision1: Format corrections Revision 2: Further correntions |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	On-network / Location/ On-demand Location Information Request

6.3.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client)registered and authorized for MCPTT service and not in an emergency state and having received a location information configuration message providing configuration parameters for both NonEmergencyLocationInformation and EmergencyLocationInformation }

ensure that {

when { UE (MCPTT Client) receives a location information request before the minimumReportInterval timer has expired  }

then { UE (MCPTT Client)does not wait for the minimumReportInterval timer to expire, and,  sends"immediately"  a SIP MESSAGE containing location information report including information in accordance with the NonEmergencyLocationInformation configuration and the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request, and, resets the minimumReportInterval timer }

}

(2)

with { UE (MCPTT Client)registered and authorized for MCPTT service and not in an emergency state and having received a location information configuration message }

ensure that {

when { UE (MCPTT Client) receives a new location information configuration message and ConfigScope set to 'Full' }

then { UE (MCPTT Client) stores the contents of the <Configuration> elements thereby overriding the previous configuration }

}

6.3.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clauses 13.3.1, 13.3.2, 13.3.3, 13.3.4.2, F.3.3. Unless otherwise stated, these are Rel-13 requirements.

[TS 24.379 clause 13.3.2]

The MCPTT client sends a location report when one of the trigger criteria is fulfilled or when it receives a request from the participating MCPTT function to send a location report. To send the location report the MCPTT client can use an appropriate SIP message that it needs to send for other reasons, or it can include the location report in a SIP MESSAGE request.

To send a location report, the MCPTT client includes in the SIP MESSAGE request an application/vnd.3gpp.mcptt-location-info+xml MIME body as specified in clause F.3. The MCPTT client populates the elements in accordance with its reporting configuration. Further location information may also be included in the P-Access-Network-Info header field.

[TS  24.379 clause 13.3.2]

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

2)	a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml"; and

3)	 an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Configuration> root element included in the <location-info> root element;

then the MCPTT client:

1)	shall store the contents of the <Configuration> elements;

…

3)	shall start the minimumReportInterval timer.

[TS  24.379 clause 13.3.3]

Upon receiving a SIP MESSAGE request containing

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

2)	a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml"; and

3)	 an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Request> element included in the <location-info> root element;

then the MCPTT client:

1)	shall send a location report as specified in subclause 13.3.4; and

2)	shall reset the minimumReportInterval timer.

[TS  24.379 clause 13.3.4.2]

If the MCPTT client does not need to send a SIP request for other reasons, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. The MCPTT client;

1)	shall include in the Request-URI, the SIP URI received in the P-Asserted-Identity header field in the received SIP MESSAGE request for location report configuration;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml";

3)	shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body and in the <location-info> root element include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

…

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcptt along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [6];

5)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer;

…

7)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4].

[TS 24.379 clause F.3.3]

<Configuration> element has a <ConfigScope> attribute that can assume the values "Full" and "Update". The value "Full" means that the Configuration> element contains the full location configuration which replaces any previous location configuration. The value "Update" means that the location configuration is in addition to any previous location configuration. To remove configuration elements a "Full" configuration is needed. The <Configuration> element contains the following child elements:

6.3.2.3	Test description

6.3.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 and Cell 2 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state on Cell 1 (Serving cell).

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.3.2.3.2	Test procedure sequence

Table 6.3.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Void. | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the connection establishment for the purpose of MCPTT sending the message(s) below are described in TS 36.579-1 [2], subclause 5.4.4 ‘Generic Test Procedure for MCPTT CT communication in E-UTRA’. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 1 | The SS sends a location reporting configuration containing configuration parameters for both NonEmergencyLocationInformation and EmergencyLocationInformation, and, minimumReportInterval timer set to 20 sec and, periodic trigger criteria with PeriodicReport reporting interval set to 15 sec. NOTE: The handling of the PeriodicReport is not tested in this test. It is added here for the purpose of verifying that the UE restarts the minimumReportInterval – when the Periodic report trigger fires before the expiration of the minimumReportInterval the UE needs to wait for its expiry before sending the periodic report. | <--- | SIP MESSAGE | - | - |
| 1AA | The UE (MCPTT Client) responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 1A | Start timer t_Wait=10s (NOTE 1) | - | - | - | - |
| 1B | Start timer t_PeriodicReport=15s (PeriodicReport timer set in step 1). | - | - | - | - |
| - | EXCEPTION: The SS waits 2 seconds before it releases the RRC connection | - | - | - | - |
| 1C | t_Wait expires . | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the connection establishment for the purpose of MCPTT sending the message(s) below are described in TS 36.579-1 [2], subclause 5.4.4 'Generic Test Procedure for MCPTT CT communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 2 | The SS sends an on-demand location reporting request. | <--- | SIP MESSAGE | - | - |
| 2A | The UE (MCPTT Client) responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE before t_PeriodicReport expires containing the location information report with the correct ReportID providing only the parameters set in the NonEmergencyLocationInformation configuration provided in step 1? | ---> | SIP MESSAGE | 1 | P |
| 3AA | The SS (MCPTT Server) responds with SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 3A | Start timer t_MinimumReportInterval=20s | - | - | - | - |
| 3B | Stop timer t_PeriodicReport | - | - | - | - |
| - | EXCEPTION: The SS waits 2 seconds before it releases the RRC connection | - | - | - | - |
| 3C | Void. | - | - | - | - |
| 3D | t_MinimumReportInterval expires. | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the connection establishment for the purpose of MCPTT sending the message(s) below are described in TS 36.579-1 [2], subclause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 3E | Check: Does the UE (MCPTT Client) send a SIP MESSAGE containing the periodic trigger identification along with location information report? | ---> | SIP MESSAGE | 1 | P |
| 3F | The SS (MCPTT Server) responds with SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a location reporting configuration with new configuration changing some of the parameters and not including periodic trigger criteria, and, ConfigScope set to 'Full'. | <--- | SIP MESSAGE | - | - |
| 4A | The UE (MCPTT Client) responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 5 | The SS sends on-demand location reporting request. | <--- | SIP MESSAGE | - | - |
| 5A | The UE (MCPTT Client) responds with a SIP 200 (OK) | ---> | SIP 200 (OK) | - | - |
| 6 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE containing the location information report with the correct ReportID and in accordance with the latest configuration parameters provided in step 4? | ---> | SIP MESSAGE | 2 | P |
| 6A | The SS (MCPTT Server) responds with SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 7-8 | Void. | - | - | - | - |
| 9 | Check: Does the UE (MCPTT Client) send any SIP MESSAGE in the next 25s? | ---> | SIP MESSAGE | 2 | F |
|  | EXCEPTION: The SS waits 2 seconds before it releases the RRC connection |  |  |  |  |
| NOTE 1: The value of 10s is arbitrary chosen. However, it needs to be long enough to ensure that if the UE did not reset the minimumReportInterval timer after it sent the message in step 3 the UE will send the SIP MESSAGE for the periodic reporting before step 3D and will fail, and, it needs to be short enough so that step 2 happens reasonably before t_PeriodicReport expires. |  |  |  |  |  |

6.3.2.3.3	Specific message contents

Table 6.3.2.3.3-1: SIP MESSAGE (Step 1, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-info as described in Table 6.3.1.3.3-13 |  |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-2 |  |  |  |

Table 6.3.2.3.3-2: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| GeographicalCoordinate | present | Provision of <GeographicalCoordinate> in the report is requested. |  |  |
| minimumIntervalLength | "20" |  |  |  |
| TriggeringCriteria |  |  |  |  |
| PeriodicReport |  |  |  |  |
| extension base | "15" | the value in seconds of the time which determines the periodic provision of the report |  |  |
| TiggerID | "PERIODIC" |  |  |  |

Table 6.3.2.3.3-3: SIP MESSAGE (Step 2, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-info as described in Table 6.3.1.3.3-13 |  |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-4 |  |  |  |

Table 6.3.2.3.3-4: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-3)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.3-1.

Table 6.3.2.3.3-5: SIP MESSAGE (Step 3, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCPTT-Info |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-6 |  |  |  |

Table 6.3.2.3.3-6: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportID attribute | "1" | The same as the ID in the request message. |  |  |
| ReportType attribute | "NonEmergency" |  |  |  |
| CurrentLocation |  | Contains the information corresponding to the location reporting configuration of table 6.3.2.3.3-2 |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-1 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-1 |  |  |

Table 6.3.2.3.3-6A: SIP MESSAGE (Step 3E, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCPTT-Info |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-6B |  |  |  |

Table 6.3.2.3.3-6B: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-6A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| ReportType attribute | "NonEmergency" |  |  |  |
| TriggerID | "PERIODIC" |  |  |  |
| CurrentLocation |  | Contains the information corresponding to the location reporting configuration of table 6.3.2.3.3-2 |  |  |
| CurrentServingEcgi | not present |  |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-1 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-1 |  |  |

Table 6.3.2.3.3-7: SIP MESSAGE (Step 4, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-info as described in Table 6.3.1.3.3-13 |  |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-8 |  |  |  |

Table 6.3.2.3.3-8: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.2-1. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Configuration |  |  |  |  |
| NonEmergencyLocationInformation |  |  |  |  |
| ServingEcgi | present | Provision of <ServingEcgi> in the report is requested. |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| GeographicalCoordinate | not present |  |  |  |

Table 6.3.2.3.3-9: SIP MESSAGE (Step 5, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-info as described in Table 6.3.1.3.3-13 |  |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-10 |  |  |  |

Table 6.3.2.3.3-10: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.3-1. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Request |  |  |  |  |
| RequestID | "2" | The RequestID that the MCPTT Client shall reference in the Report |  |  |

Table 6.3.2.3.3-11: SIP MESSAGE (Step 6, Table 6.3.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | Not present | MCPTT-Info |  |  |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 |  |
| MIME-part-body | Location-info as described in Table 6.3.1.3.3-12 |  |  |  |

Table 6.3.2.3.3-12: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info | present |  |  |  |
| Report | present |  |  |  |
| ReportID | "2" | The same as the ID in the request message. |  |  |
| ReportType | "NonEmergency" |  |  |  |
| CurrentLocation | present | Contains the information corresponding to the location reporting configuration of table 6.3.2.3.3-8 |  |  |
| CurrentServingEcgi | Encrypted <CurrentServingEcgi> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-1 |  |  |
| NeighbouringEcgi | not present |  |  |  |
| MbmsSaId | not present |  |  |  |
| MbsfnArea | not present |  |  |  |
| CurrentCoordinate | not present |  |  |  |

Table 6.3.2.3.3-13: MCPTT-Info in SIP MESSAGE (Table 6.3.2.3.3-1, 6.3.2.3.3-3, 6.3.2.3.3-7, 6.3.2.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |