3GPP TSG-RAN5 Meeting #93-e	R5-217296

# Electronic Meeting, 08th – 19th Nov. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1203 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.2B.2.1 MPR for intra-band contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 6.5B.2.1.3 ACLR for intra-band contiguous EN-DC, the test configuration table keeps the totally same with that for 6.2B.2.1 MPR for intra-band contiguous EN-DC. Besides, test procedures for ACLR have included MPR check. In view of the two reasons, there is no need to test 6.2B.2.1 MPR if ACLR is executed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the MPR applicability to specify MPR test can be skipped if ACLR is tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs would perform needless MPR testing for intra-band contiguous EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.522 CR 0115 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2B.2	UE Maximum Output Power reduction for EN-DC

6.2B.2.0	General

The UE maximum output power reduction (MPR) specified in this clause is applicable for UEs configured with EN-DC when NS_01 is indicated in the MCG and the SCG. The MPR applies subject to indication in the field modifiedMPRbehavior for the SCG [2].

6.2B.2.1	UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC

6.2B.2.1.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.2.1.2	Test applicability

This test case applies to all types of E-UTRA power class 3 and power class 2 UE release 15 and forward, supporting intra-band contiguous EN-DC.

NOTE:	Test execution is not necessary if clause 6.5B.2.1.3 ACLR is executed since MPR requirement is verified in this test case.

6.2B.2.1.3	Minimum conformance requirements

6.2B.2.1.3.1	General

When the UE is configured for intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this clause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [5]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [5]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [5]

MPRENDC is defined in Clause 6.2B.2.1.3.2.

6.2B.2.1.3.2	MPR for power class 3 and power class 2

MPR in this subclause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with ENDC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in subclause 6.2.4 of TS 36.101 [5] and 6.2.2 of TS 38.101-1 [2] apply. For a UE supporting dynamic power sharing for DC_(n)71AA for which dual simultaneous uplink transmissions are mandatory and A-MPR defined in subclause 6.2B.3.1.1 is applied as MPR. The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA =		15;		0 ≤ B < 0.5

10;		0.5 ≤ B < 1.0

8;		1.0 ≤ B < 2.0

6;		2.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.1.

Exception requirements for both NR and E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR. LTE anchor agnostic approach is not applied for this case. E-UTRA test point analysis is included and E-UTRA measurements are performed.

No exception requirements for NR or E-UTRA are defined for this test when transmission on E-UTRA doesn't overlap in time with NR, for a UE that supports dynamic power sharing. LTE anchor agnostic approach is not applied for this case based on the test point analysis in TS 38.905 [7].

6.2B.2.1.4	Test description

6.2B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.4B.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521.1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.2.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1. |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1. |  |  |  |  | Low range, High range |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1. |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1. |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (NOTE 1) | Power config (NOTE 8) |
| 1 | Default | Default | Default | N/A | 16QAM | Outer_Full | DFT-s-OFDM Pi/2 BPSK | Outer_Full | B |
| 2 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | B |
| 3 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 4 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | B |
| 6 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM Pi/2 BPSK | N/A | A |
| 8 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B |
| 9 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B |
| 10 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 11 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 12 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B |
| 13 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 14 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 15 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 16QAM | Outer_Full | B |
| 16 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | Edge_1RB_Right | B |
| 17 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 16QAM | N/A | A |
| 18 (Note 3) | High |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Right | A |
| 19 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | Edge_1RB_Left | B |
| 20 (Note 4) | Low |  |  |  | 16QAM | N/A | DFT-s-OFDM 16QAM | Edge_1RB_Left | A |
| 21 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 16QAM | N/A | A |
| 22 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 64QAM | Outer_Full | B |
| 23 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 64QAM | Edge_1RB_Right | B |
| 24 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 64QAM | Edge_1RB_Left | B |
| 25 | Default |  |  |  | 16QAM | Outer_Full | DFT-s-OFDM 256QAM | Outer_Full | B |
| 26 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM 256QAM | Edge_1RB_Right | B |
| 27 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM 256QAM | Edge_1RB_Left | B |
| 28 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM QPSK | Outer_Full | B |
| 29 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Right | B |
| 30 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM QPSK | N/A | A |
| 31 (Note 3) | High |  |  |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Right | A |
| 32 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | Edge_1RB_Left | B |
| 33 (Note 4) | Low |  |  |  | 16QAM | N/A | CP-OFDM QPSK | Edge_1RB_Left | A |
| 34 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM QPSK | N/A | A |
| 35 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 16QAM | Outer_Full | B |
| 36 (Note 3) | Default |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | Edge_1RB_Right | B |
| 37 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 16QAM | N/A | A |
| 38 (Note 3) | High |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Right | A |
| 39 (Note 4) | Default |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | Edge_1RB_Left | B |
| 40 (Note 4) | Low |  |  |  | 16QAM | N/A | CP-OFDM 16QAM | Edge_1RB_Left | A |
| 41 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 16QAM | N/A | A |
| 42 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 64QAM | Outer_Full | B |
| 43 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 64QAM | Edge_1RB_Right | B |
| 44 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 64QAM | Edge_1RB_Left | B |
| 45 | Default |  |  |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B |
| 46 (Note 3) | Low |  |  |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B |
| 47 (Note 4) | High |  |  |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B |
| 48 (Note 4) | Default |  |  |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Edge_Full_Right is defined as 2 RBs allocated at the right edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 NOTE 7: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. NOTE 8: Power config as specified in Table 6.2B.2.1.4.3-3 and Table 6.2B.2.1.4.3-4 for PC3 UE or Table 6.2B.2.1.4.3-5 and Table 6.2B.2.1.4.3-6 for PC2 UE. NOTE 9: Test IDs with simultaneous E-UTRA and NR UL transmission don’t apply to DC_(n)71AA for a UE supporting dynamic power sharing (A-MPR is applied as MPR and covered by 6.2B.3.1.1). |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.2.1.4.3.

8.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

<End of Changes>