3GPP TSG-RAN5 Meeting #95-e	R5-222849

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2362 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add SI combination for MBS Broadcast test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no SI combination for MBS broadcast testcase |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add SI combination for MBS broadcast testcase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SI combination for MBS broadcast testcase is not configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.4.3.1.2	Combinations of system information blocks for NR standalone and NE-DC

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, several combinations of system information blocks are defined.

Regardless of the combination of system information blocks indicated as being used by a test case, the SS shall broadcast only the NR MIB on the NR Cell(s) configured on an SDL band.

Combination NR-1 is the default combination which applies to the following test case scenarios:

-	NR FDD single cell scenario except RRM test case scenarios

-	NR TDD single cell scenario except RRM test case scenarios

-	NR FDD inter-band DC component carriers cell scenario

-	NR TDD inter-band DC component carriers cell scenario

-	NR FDD and NR TDD inter-band DC component carriers cell scenario

Combination NR-2 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario

-	NR TDD intra-frequency multi cell scenario

-	NR FDD and NR TDD dual mode multi cell roaming scenario

-	NR FDD single cell RRM test case scenario

-	NR TDD single cell RRM test case scenario

Combination NR-3 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario with neighbouring cell related information

-	NR TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination NR-4 applies to the following test case scenarios:

-	NR FDD inter-frequency multi cell scenario

-	NR TDD inter-frequency multi cell scenario

-	NR FDD inter-band multi cell scenario

-	NR TDD inter-band multi cell scenario

-	NR FDD and NR TDD dual mode multi cell non-roaming scenario

-	NR FDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD inter-band carrier aggregation component carriers cell scenario

-	NR TDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario

Combination NR-5 applies to the following test case scenarios:

-	NR FDD intra-band carrier aggregation component carriers cell scenario + NR FDD intra-frequency neighbour.

-	NR FDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR TDD intra-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

Combination NR-6 applies to the following test case scenarios:

-	3GPP inter-RAT NR FDD + E-UTRA FDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA TDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA FDD multi cell scenario

Combination NR-7 applies to the following test case scenarios:

-	NR FDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

-	NR TDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

Combination NR-8 applies to the following test case scenarios:

-	NR FDD ETWS single cell scenario

-	NR TDD ETWS single cell scenario

Combination NR-9 applies to the following test case scenarios:

-	3GPP NR FDD + CMAS single cell scenario

-	3GPP NR TDD + CMAS single cell scenario

Combination NR-10 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS primary notification single cell scenario

-	3GPP NR TDD + ETWS primary notification single cell scenario

Combination NR-11 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS secondary notification single cell scenario

-	3GPP NR TDD + ETWS secondary notification single cell scenario

Combination NR-12 applies to the following test case scenarios:

-	3GPP NR FDD + SNPN only single cell scenario

-	3GPP NR TDD + SNPN only single cell scenario

Combination NR-13 applies to the following test case scenarios:

-	3GPP NR FDD + CAG cell  multi cell scenario

-	3GPP NR TDD + CAG cell  multi cell scenario

Combination NR-14 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

Combination NR-15 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD single cell scenario + positioning test case which require posSIBs.

Combination NR-16 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-17 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-X applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + MBS Broadcast test case which does not require SIB21.

-	3GPP NR TDD single cell scenario + MBS Broadcast test case which does not require SIB21.

Combination NR-Y applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + MBS Broadcast test case which does not require SIB21.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS Broadcast test case which does not require SIB21.

-	3GPP NR FDD inter-frequency multi cell scenario + MBS Broadcast test case which does not require SIB21.

-	3GPP NR TDD inter-frequency multi cell scenario + MBS Broadcast test case which does not require SIB21.

Combination NR-Z applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + MBS Broadcast test case which require SIB21.

-	3GPP NR TDD intra-frequency multi cell scenario + MBS Broadcast test case which require SIB21.

-	3GPP NR FDD inter-frequency multi cell scenario + MBS Broadcast test case which require SIB21.

-	3GPP NR TDD inter-frequency multi cell scenario + MBS Broadcast test case which require SIB21.

Table 4.4.3.1.2-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB1 | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | SIB20 | SIB21 | posSIBs |
| NR-1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-2 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-3 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-4 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-5 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-6 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-7 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-8 | X |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |
| NR-9 | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-10 | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| NR-11 | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| NR-12 | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| NR-13 | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| NR-14 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |
| NR-15 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-16 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-17 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-X | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| NR-Y | X | X |  | X |  |  |  |  |  |  |  |  |  |  | X |  |  |
| NR-Z | X | X |  | X |  |  |  |  |  |  |  |  |  |  | X | X |  |

<End of modified section 1>