3GPP TSG-RAN5 Meeting #96-e	R5-223974

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0216 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assistance data for TC 14.2.1 and 14.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The assistance data for the new DL-TDOA test cases is not yet defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined the assistance data for the dummy cells for 14.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 14.2.1 cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11 (new section) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table 10.1.1-1: General test parameters for the Bluetooth beacons to be simulated for TS 37.571-2 clauses 7 and 9

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Number of Bluetooth beacons |  | 1 |  |
| Transmit power | dBm | -60 |  |
| RF Channel |  | AWGN |  |
| Public address of Bluetooth beacon |  | Any suitable value | For details see [23] |
| Other Bluetooth beacon parameters |  | Any suitable value | For details see [23] |

# 11	DL-TDOA information

## 11.1	DL-TDOA Assistance data for DL-TDOA measurement tests

### 11.1.1	General

This subclause defines the DL-TDOA assistance data that shall be used for the DL-TDOA measurement tests defined in TS 37.571-1 [6].

### 11.1.2	DL-TDOA Assistance Data

This subclause defines the DL-TDOA assistance data elements which shall be provided to the UE in the DL-TDOA measurement tests defined in TS 37.571-1 [6].

Table 11.1.2-3: Sequence data values for 15 instances of sequence for test case 14.2.1

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value PRS muting ingo | Value PRS RE offset | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |  |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘01’ | 0 | 0 |  |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘10’ | 1 | 23 |  |
| Dummy cell | 7 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘01’ | 0 | 46 |  |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘10’ | 1 | 0 |  |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘01’ | 0 | 23 |  |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘10’ | 1 | 15 |  |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘01’ | 0 | 0 |  |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘10’ | 1 | 23 |  |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘01’ | 0 | 0 |  |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘10’ | 1 | 46 |  |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘01’ | 0 | 0 |  |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘10’ | 1 | 23 |  |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘01’ | 0 | 23 |  |

Annex A (normative): GPS data files

< End of changes >