3GPP TSG-RAN5 Meeting #96-e	R5-224600

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0252 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability of NE-DC RRC test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New NE-DC RRC test cases 8.2.3.4.2, 8.2.3.5.2 and 8.2.3.12.2 are added into 38.523-1 , thus relevant applicability definition and test case condition need to be added in 38.523-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added applicability of following test cases in table 4.1-3a 8.2.3.4.2 Measurement configuration control and reporting / Event A1 / Measurement of E-UTRA PSCell / NE-DC 8.2.3.5.2 Measurement configuration control and reporting / Event A2 / Measurement of E-UTRA PSCell / NE-DC 8.2.3.12.2 Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / NE-DC - Added new test case condition for UEs supporting NE-DC in table 4.2-1 - Added new test case condition for UEs supporting NE-DC and Inter-RAT E-UTRA measurements and Event B triggered reporting in table 4.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test case applicability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The hard number for “xyz” and “abc” in Table 4.1-3a and Table 4.2-1 needs to be assigned by MCC |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

4.1	Protocol conformance test cases applicability

< Unchanged sections omitted >

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 8.2.3 | Measurement Configuration Control and Reporting / Handovers |  |  |  |
| 8.2.3.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells |  |  |  |
| 8.2.3.1.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / EN-DC | Rel-15 | C01 | UEs supporting EN-DC |
| 8.2.3.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / RSRQ based measurements |  |  |  |
| 8.2.3.2.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / RSRQ based measurements / EN-DC | Rel-15 | C01 | UEs supporting EN-DC |
| 8.2.3.3 | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of NR cells |  |  |  |
| 8.2.3.3.1 | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of NR cells / EN-DC | Rel-15 | C01 | UEs supporting EN-DC |
| 8.2.3.4 | Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell |  |  |  |
| 8.2.3.4.1 | Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell / EN-DC | Rel-15 | C13 | UEs supporting EN-DC and NR measurements and Event A triggered reporting |
| 8.2.3.4.2 | Measurement configuration control and reporting / Event A1 / Measurement of E-UTRA PSCell / NE-DC | Rel-15 | Cxyz | UEs supporting NE-DC. |
| 8.2.3.5 | Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell |  |  |  |
| 8.2.3.5.1 | Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |
| 8.2.3.5.2 | Measurement configuration control and reporting / Event A2 / Measurement of E-UTRA PSCell / NE-DC | Rel-15 | Cxyz | UEs supporting NE-DC. |
| 8.2.3.6 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cells |  |  |  |
| 8.2.3.6.1 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cells / Intra-frequency measurements / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.6.1a | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.6.1b | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC | Rel-15 | C93 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.6.2 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cells / Intra-frequency measurements / NE-DC | Rel-15 | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |
| 8.2.3.6.2a | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cell / Inter-frequency measurements / NE-DC | Rel-15 | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.6.2b | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cell / Inter-band measurements / NE-DC | Rel-15 | C183 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.7 | Measurement configuration control and reporting / Event A4 (intra-frequency, inter-frequency and inter-band measurements) / Measurement of Neighbour NR cell |  |  |  |
| 8.2.3.7.1 | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.7.1a | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.7.1b | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC | Rel-15 | C93 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.8 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell |  |  |  |
| 8.2.3.8.1 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.8.1a | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC | Rel-15 | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.8.1b | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC | Rel-15 | C93 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.9 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based intra-frequency measurements / Measurement of Neighbour NR cell |  |  |  |
| 8.2.3.9.1 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based intra-frequency measurements / Measurement of Neighbour NR Cell / EN-DC | Rel-15 | C15 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |
| 8.2.3.10 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based inter-frequency measurements / Measurement of Neighbour NR cell |  |  |  |
| 8.2.3.10.1 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based inter-frequency measurements / Measurement of Neighbour NR Cell / EN-DC | Rel-15 | C15 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements) and CSI-RSRP and CSI-RSRQ measurement |
| 8.2.3.11 | Measurement configuration control and reporting / Measurement Gaps |  |  |  |
| 8.2.3.11.1 | Measurement configuration control and reporting / Measurement Gaps / NR FR1 / EN-DC | Rel-15 | C24 | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC within FR1 |
| 8.2.3.11.2 | Measurement configuration control and reporting / Measurement Gaps / NR FR2 / EN-DC | Rel-15 | C25 | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC including FR2 |
| 8.2.3.11.3 | Measurement configuration control and reporting / Measurement Gaps / NR-DC | Rel-15 | C149 | UEs supporting NR-DC and two independent measurement gap configurations for FR1 and FR2 |
| 8.2.3.12 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of NR cells |  |  |  |
| 8.2.3.12.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of NR cells / EN-DC | Rel-15 | C01 | UEs supporting EN-DC |
| 8.2.3.12.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / NE-DC | Rel-15 | Cabc | UEs supporting NE-DC and Inter-RAT E-UTRA measurements and Event B triggered reporting |

< Unchanged sections omitted >

4.2	Protocol conformance test cases Applicability Condition

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

< Unchanged sections omitted >

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| C190 | IF A.4.1-5/1 AND A.4.3.6-1/aa THEN R ELSE N/A | UEs supporting 5G Core and Idle/Inactive Measurements |
| C191 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/bb THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and Idle/Inactive Measurements |
| C192 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.3.6-1/aa THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and Idle/Inactive Measurements |
| C193 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/bb THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and E-UTRA and Idle/Inactive Measurements |
| C194 | IF A.4.1-3/3 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting NE-DC and UL transmission via both MCG path and SCG path for the split DRB |
| Cxyz | IF A.4.1-3/3 THEN R ELSE N/A | UEs supporting NE-DC. |
| Cabc | IF A.4.1-3/3 AND A.4.3.6-1/5 THEN R ELSE N/A | UEs supporting NE-DC and Inter-RAT E-UTRA measurements and Event B triggered reporting |

<End of modified section>