3GPP TSG-RAN5 Meeting #98	R5-230248

# Athens, Greece, 27th Feb - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1528 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of test requirement tables for spurious emission for UE co-existence for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, KDDI, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The test requirements for spurious emission for UE co-existence for EN-DC configurations are specified in different tables in accordance with the different releases in the spec. In R5-225651, the proposal 2 to merge the tables of test conformance requirements for UE co-existence with protected bands in TS 38.521-3. A new column of “Release” in the merged table to indicate all the supported releases is suggested. The following guidelines will be applied in this CR. - If the requirements for a specified configuration have not changed since an early release, the column of “Release” should be noted as all the supported releases in one row. Otherwise, a new row in the table for the later release with new requirements for the specified configuration should be added. - If the Note number in the table is different among releases, an additional information for release should be applied, such as NOTE X_R15, NOTE X_R16, etc. According to RF Action Point AP#97.24 at RAN5#97, it is suggested to merge test requirement tables in clause 6.5B.3.3.2.5 for spurious emission for UE co-existence test case for EN_DC configurations. - Minimum requirements need to be updated for following EN-DC configurations: - DC_18_n77: Need to be added to Table 6.5B.3.3.2.3-2 (Rel-16) as being a Rel-16 configurations in accordance with TS 38.101-1 v16.14.0 - DC_25_n41: Band 48 need to be updated to refer to Note 2 in in Table 6.5B.3.3.2.3-1 (Rel-15), Table 6.5B.3.3.2.3-2 (Rel-16) and Table 6.5B.3.3.2.3-3 (Rel-17) in accordance with TS 38.101-1 v15.20.0, v16.14.0 and v17.8.0, Table 6.5B.3.3.2-1. - DC_39_n79: E-UTRA Band 28 needs to be added to Table 6.5B.3.3.2.3-1 (Rel-15) in accordance with TS 38.101-1 v15.20.0. - There are inconsistencies between test requirement tables for different release tables vs test points analysis and/or minimum requirements. - Test requirements need to be introduced for new TP-analysis |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Merge the tables with different releases for the test requirements for spurious emission for UE co-existence for EN-DC configurations and add a new column of “Release”. - Corrected minimum requirements for: - DC_18_n77: Added to Table 6.5B.3.3.2.3-2 (Rel-16) - DC_25_n41: Band 48 updated to refer to Note 2 in Table 6.5B.3.3.2.3-1 (Rel-15), Table 6.5B.3.3.2.3-2 (Rel-16) and Table 6.5B.3.3.2.3-3 (Rel-17). - DC_39_n79: E-UTRA Band 28 added to Table 6.5B.3.3.2.3-1. - Corrected inconsistencies between releases in the merged test requirement table: - DC_11_n77: Added Band 40 for Rel-15 and Rel-17. Band 40 is included in Rel-15 and Rel-17 minimum requirements and is impacted band in theTP analysis in TR 38.905. - DC_11_n79: Removed not impacted bands and frequency ranges (18, 19,28 and 40 and frequency ranges 2545-2575, and 2595-2645) from Rel-15 in accordance to TP analysis in TR 38.905. Impacted protected bands are the same for Rel-15 as for Rel-16 and Rel-17. - DC_12_n66: Added impacted Bands 53 and n77 in accordance with TP analysis in TR 38.905. - DC_18_n77: Added from Rel-16 (was missed in the Rel-16 table) - DC_25_n41: Removed NR Band n77 from Rel-15 in accordance with minimum requirements in TS 38.101-1 v15.20.0, Table 6.5B.3.3.2-1. - DC_39_n79: Adding impacted band 28 to Rel-15 in accordance with minimum requirements in TS 38.101-1 v15.20.0, Table 6.5B.3.3.2-1. - DC_41_n79: Removed E-UTRA Band 9 from Rel-16 and Rel-17 in accordance with with TP analysis in TR 38.905. - Added test requirements for the following new EN-DC configurations DC_1_n41 (TR 38.905 CR 0279, KDDI) DC_41_n28 (TR 38.905 CR 0279, KDDI) DC_8A_n94A_ULSUP-TDM (TR 38.905 CR 0271, Nokia) DC_20A_n92A_ULSUP-TDM (TR 38.905 CR 0272, Nokia) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The test requirements for spurious emission requirements for UE co-existence for NR bands will be inconvenient to retrieve. - Incorrect minimum requirements remains. - Inconstencies beween releases remains. - Test of new configurations not possible due to missing test requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.2.3, 6.5B.3.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0271, 0272, 0279 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR also impacts EN-DC configurations introduced in Rel-15 and Rel-16 under RAN5 work items 5GS_NR_LTE-UEConTest and NR_CADC_NR_LTE_DC_R16-UEConTest. Introduction of test requirements for DC_8A_n94A_ULSUP-TDM and DC_20A_n92A_ULSUP-TDM are depending on RAN4 CRs R4-2300401, R4-2300402, R4-2300403 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### << Start of changes >>

6.5B.3.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2.3-1 and Table 6.5B.3.3.2.3-2 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the requirements in Table 6.5B.3.3.2.3-1 and Table 6.5B.3.3.2.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.3-1: Spurious emission band UE co-existence limits Rel-15

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band n3, n34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_2_n5 | E-UTRA Bands 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 41, 43, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n66 | E-UTRA Bands 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Bands 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2_n78 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n8 | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67,68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n28 | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2,7 |
| DC_7_n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 65, 66, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n77 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 4, 48, 50, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_19_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n28 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n78 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | FDL_low |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_25_n41 | E-UTRA band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 66, 70,71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA/NR Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_26_n41 | E-UTRA/NR Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 |  | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_30_n5 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_38_n78 | N/A |  |  |  |  |  |  |
| DC_39_n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_41_n77 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 40, 42, 44, 45, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42_n77 | N/A |  |  |  |  |  |  |
| DC_42_n78 | N/A |  |  |  |  |  |  |
| DC_42_n79 | N/A |  |  |  |  |  |  |
| DC_66_n5 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n71 | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7,22, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.3-2: Spurious emission band UE co-existence limits Rel-16

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_1_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_1_n7 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_1_n8 | E-UTRA Band 11, 20, 21, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_2_n5 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  | E-UTRA Band 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 41, 43, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n41 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n66 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25, 41, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_2_n78 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_3_n1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_3_n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_3_n28 | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_3_n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_5_n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | NR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41, 43, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41, 42, 48, 52 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2,7 |  |
| DC_7_n1 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |  |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7,16 |  |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7,16 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 26, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |  |
| DC_7_n8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_8_n1 | E-UTRA Band 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_8_n3 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | E-UTRA Band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
| DC_8_n20 | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8, 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_8_n28 | E-UTRA Band 20, 31, 34, 38, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10, 12 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |  |
| DC_8_n41 | E-UTRA Band 1, 11, 21, 28, 34, 39, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_8_n77 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
| DC_8_n78 | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n79 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_12_n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 48, 50, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n78 | E-UTRA Band 2, 5, 7, 13, 17, 25, 26, 41, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA band 12 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_13_n2 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2,14, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n66 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 30, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  | Frequency range | 799 | - | 803 | -35 | 0.00625 | 5 |  |
| DC_14_n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 2, 25 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  | 5 |
| DC_14_n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  | 5 |
| DC_18_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_20_n1 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 38, 42, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n3 | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 20 E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n7 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42, 52 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_20_n8 | E-UTRA Band 1, 28, 31, 32, 34, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 7, 22, 38, 42, 43 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n28 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n78 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n79 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_25_n41 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 66, 70,71 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_26_n41 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
| DC_26_n77 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_26_n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_26_n79 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_28_n3 | E-UTRA Band 1, 22, 42, 43, 50, 51, 65, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n5 | E-UTRA Band 2, 3, 5, 7, 8, 14, 18, 19, 24, 25, 26, 28, 30, 31, 34, 38, 40, 70, 71 | FDL_low | - | FDL_high | -50 |  |  |  |
|  | E-UTRA Band 4, 22, 32, 41, 42, 43, 45, 48, 50, 51, 52, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
| DC_28_n7 | E-UTRA Band 2, 3, 5, 8, 20, 26, 27, 31, 34, 40, 72 NR band n7 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_28_n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_28_n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_30_n5 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_38_n78 | N/A |  |  |  |  |  |  |  |
| DC_39_n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |  |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |  |
| DC_39_n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |  |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |  |
| DC_40_n1 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_40_n41 | Bands 1, 3, 5, 8, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_40_n78 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_40_n79 | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n77 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 40, 42, 44, 45, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_42_n77 | N/A |  |  |  |  |  |  |  |
| DC_42_n78 | N/A |  |  |  |  |  |  |  |
| DC_42_n79 | N/A |  |  |  |  |  |  |  |
| DC_48_n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_48_n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_66_n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRANR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  | E-UTRA Band 22, 42, 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n5 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 41, 42, 48, 52 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n41 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n71 | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_66_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.3-3: Spurious emission band UE co-existence limits Rel-17

| EN-DC Configuration |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |
| DC_1_n3 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA band 22, 42, 52 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 67 |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_1_n5 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3,34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA band 41, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_1_n7 |  | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_1_n8 |  | E-UTRA Band 11, 20, 21, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1, 8, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_1_n28 |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 1, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_1_n38 |  | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_1_n77 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |
| DC_1_n78 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |
| DC_1_n79 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 8 |
| DC_1_n80 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n5 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 5 |
|  |  | E-UTRA Band 2, 25, 48 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 41, 43, 53 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n7 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | 1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_2_n12 |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 25, 85 NR band n12 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 3 |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 4, 51, 66, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n30 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n38 |  | E-UTRA Band 4, 5, 12, 13, 14,17, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n41 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 43, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2A_n48A |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_2_n66 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n71 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_2_n77 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n78 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_3_n1 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA band 22, 42, 52 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_3_n5 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3,34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 22, 42, 52 Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n7 |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA band 22, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_3_n8 |  | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67,68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 5 |
|  |  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n28 |  | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 13 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_3_n41 |  | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n77 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n79 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_4_n78 |  | E-UTRA Band 5, 7, 26, 28, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_5_n2 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | NR Band n2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |
|  |  | E-UTRA Band 41, 43, 53 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_5_n7 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 65, 66, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 52 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 7, 6 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 7, 6 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 14 |
| DC_5_n12 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43, 50, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Bands 4, 41, 42, 48, 51, 66, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_5_n30 |  | E-UTRA Band 2, 4, 5, 7,12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41, 53 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_5_n66 |  | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |
|  |  | E-UTRA Band 41, 42, 48, 52, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_5_n71 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 5 |
|  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_5_n77 |  | E-UTRA Band 2, 4, 12, 13, 14, 17, 25, 26, 28, 29, 30, 40, 65, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_5_n78 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 7 |
| DC_5_n79 |  | Bands 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |
|  |  | Bands 41, 52 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_7_n1 |  | Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76, n78,n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5,16 |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7,16 |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7,16 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n2 |  | E-UTRA Band 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | 1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n3 |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA band 22, 42, 52 NR band n78, n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n5 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 26, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 52 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 7, 6 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 7, 6 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 14 |
| DC_7_n8 |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n28 |  | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n66 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n71 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 5 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | 1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n77 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n78 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_7_n80 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76. NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_8_n1 |  | E-UTRA Band 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1, 8, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_8_n3 |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 5 |
|  |  | E-UTRA band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |
|  |  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3.12 |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5. 12 |
| DC_8_n20 |  | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 8, 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_8_n28 |  | E-UTRA Band 20, 31, 34, 38, 40, 72 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 9, 11 |
|  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10, 12 |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9, 12 |
| DC_8_n41 |  | E-UTRA Band 1, 11, 21, 28, 34, 39, 45, 50, 51, 65, 73, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_8_n77 |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 3, 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |
| DC_8_n78 |  | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 3, 7, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 12 |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  | 1 |  | 5, 12 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |
| DC_8A_93A_ULSUP-TDM, DC_8A_94A_ULSUP-TDM |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 -50 |  | 1 1 |  |  |
|  |  | E-UTRA band 3, 7, 22, 41, 42, 43 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 5 |
|  |  | E-UTRA 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_11_n77 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_11_n78 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_11_n79 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_12_n2 |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 12, 25, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 3 |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 4, 51, 66, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_12_n5 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43 50, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Bands 4, 41, 42, 48, 51, 66, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_12_n66 |  | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 48, 50, 51, 66, 70 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_12_n7 |  | E-UTRA Band 2, 5, 7, 13, 14, 17, 26, 27, 30, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 50, 51,66 NR Band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_12_n25 |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 48, 66, 70. NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 2, 12, 25, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 15 |
| DC_12_n30 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 66, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_12_n41 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 4, 48, 50, 51, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_12_n77 |  | E-UTRA Band 2, 5, 7. 13, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 66 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_12_n78 |  | E-UTRA Band 2, 5, 7. 13, 17, 25, 26, 41, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 66 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 12 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_13_n2 |  | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2,14, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n5 |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 29, 48, 50, 51, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  | 1 |  |  |
|  |  | E-UTRA Band 24, 30, 41, 53 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n7 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 17,25，26, 27, 29, 50, 51, 66，74, 85 NR Band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_13_n48 |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 24, 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n66 |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 30, 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n71 |  | E-UTRA Band 4, 5, 12, 13, 17, 26, 48, 66, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 24, 25, 30, 41, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 5 |
|  |  | E-UTRA Band 14, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_13_n77 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 29, 41, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 24, 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_14_n2 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 2, 25 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_14_n30 |  | E-UTRA Band 2, 4, 5,12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_14_n77 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_14_n66 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 48 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  | 0.00625 |  | 5 |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  | 0.00625 |  | 5 |
| DC_18_n3 |  | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 65 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_18_n77 |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_18_n78 |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_18_n79 |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n1 |  | E-UTRA Band 1, 11, 21, 28, 40, 42, 65, 74 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n77 |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n78 |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n79 |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_20_n1 |  | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 38, 42, 69 NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_20_n3 |  | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 20 E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 22, 38, 42, 52 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_20_n7 |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42, 52 NR band n78, n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_20_n8 |  | E-UTRA Band 1, 28, 31, 32, 34, 65, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 3, 7, 22, 38, 42, 43 NR Band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_20_n38 |  | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42, 52 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_20_n28 |  | E-UTRA Band 3, 7, 8, 31, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_20_n78 |  | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 38, 69 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 38, 42, 69, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  | 1 |  |  |
| DC_21_n1 |  | E-UTRA Band 1, 18, 19, 28, 40, 42, 65 NR Band n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n28 |  | E-UTRA Band 1, 42, 65, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 3, 18, 19, 34, 40 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
| DC_21_n77 |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n78 |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n79 |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_25_n41 |  | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 48 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_25_n77 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_26_n41 |  | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13 , 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
| DC_26_n77 |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_26_n78 |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_26_n79 |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 5 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |
| DC_28_n3 |  | E-UTRA Band 1, 22, 42, 43, 50, 51, 65, 74, 75, 76, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n5 |  | E-UTRA Band 2, 3, 5, 7, 8, 14, 18, 19, 24, 25, 26, 28, 30, 31, 34, 38, 40, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  |  |  |
|  |  | E-UTRA Band 4, 22, 32, 41, 42, 43, 45, 48, 50, 51, 52, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 9, 11 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 9, 10 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
| DC_28_n7 |  | E-UTRA Band 2, 3, 5, 8, 20, 26, 27, 31, 34, 40, 72 NR band n7 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 9, 10 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 |  | E-UTRA Band 4, 14, 18, 19, 20, 26, 27, 39, 42, 43, 48, 50, 51, 52, 65, 66, 71, 73 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 9, 11 |
|  |  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 70, 72 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 11, 21, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2, 9, 10 |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n77 |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 1, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n78 |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 1, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n79 |  | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 1, 42, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_30_n2 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 43, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_30_n5 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41, 53 NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_30_n66 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_30_n77 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_38_n3 |  |  | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  | E-UTRA Band 22, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_38_n78 |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_39_n40 |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1805 |  |  |  | 1855 |  | -40 |  | 1 |  | 18 |
|  |  | Frequency range |  | 1855 |  |  |  | 1880 |  | -15.5 |  | 5 |  | 5, 7, 18 |
| DC_39_n41 |  | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 5, 7, 19 |
| DC_39_n79 |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1805 |  | - |  | 1855 |  | -40 |  | 1 |  | 18 |
|  |  | Frequency range |  | 1855 |  | - |  | 1880 |  | -15.5 |  | 5 |  | 18 |
| DC_40_n1 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | NR Band n77, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_40_n41 |  | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_40_n78 |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_40_n79 |  | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74 NR band n78 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n28 |  | E-UTRA Band 4, 14, 18, 19, 20, 26, 27, 39, 42, 43, 48, 50, 51, 52, 65, 66, 71, 73 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 70, 72 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 11, 21, 74, 75, 76 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 9, 10 |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  | 6 |  | 14 |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  | 6 |  | 5 |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  | 1 |  | 5 |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_41_n77 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 73, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n78 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n79 |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 42, 44, 45, 65, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high |  | -40 |  | 1 |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_42_n1 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1880 |  | - |  | 1895 |  | -40 |  | 1 |  | 5, 16 |
|  |  | Frequency range |  | 1895 |  | - |  | 1915 |  | -15.5 |  | 5 |  | 5, 7, 16 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 16 |
| DC_42_n3 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_42_n77 |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42_n78 |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42_n79 |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_48_n5 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_48_n12 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 50, 51, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_48_n66 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_48_n71 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 5 |
|  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_48_n77 |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66_n2 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 22, 42, 43, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n5 |  | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 41, 42, 48, 52, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n7 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  | 5 |  | 5, 6, 7 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 5, 6 |
| DC_66_n12 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 50, 53, 70, 71, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 4, 51, 66, 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 12, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_66_n25 |  | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n30 |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n41 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n38 |  | EUTRA 2, 4, 5, 12, 13,14,17, 25, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | Frequency range |  | 2620 |  | - |  | 2645 |  | -15.5 |  | 5 |  | 5, 7, 22 |
|  |  | Frequency range |  | 2645 |  | - |  | 2690 |  | -40 |  | 1 |  | 5, 22 |
| DC_66_n48 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_66_n71 |  | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_66_n77 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_66_n78 |  | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
| DC_71_n2 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 25, 41, 70, NR Band n2, n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 5 |
| DC_71_n38 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 30, 66, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 29 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_71_n41 |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA band 2, 25, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high |  | -38 |  | 1 |  | 5 |
| DC_71_n66 |  | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  |  |
|  |  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.3.2.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

### << End of modified section >>

### << Start of next modified section >>

6.5B.3.3.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.3.1.2.4.2 with the following exceptions:

Instead of Table 6.5B.3.1.2.3-1 --> use Table 6.5B.3.3.2.5-1 and 6.5B.3.3.2.5-2.

In addition to test configurations above, EN-DC only capable UEs and UEs not supporting NR/5GC mode in the tested band needs to be tested according to LTE anchor agnostic approach below.

Same test description as in clause 6.5.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

For Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.3.2.4.1 in TS 38.521-1 [8] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.3.3.2.4.3	Message Contents

Message contents are according to TS 36.508 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.3.3.2.4.3-1: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | Sa2 |  |  |
| specialSubframePatterns | Ssp7 |  |  |
| } |  |  |  |

Table 6.5B3.3.2.4.3-1a: Void

Table 6.5B.3.3.2.4.3-2: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

Table 6.5B.3.3.2.4.3-3: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE AND simultaneous E-UTRA and NR transmission |
|  | 20 |  | Power Class 3 UE AND simultaneous E-UTRA and NR transmission |

6.5B.3.3.2.5	Test Requirement

The test requirements are in Table 6.5B.3.3.2.5-1. For EN-DC only capable devices, in addition to Table 6.5B.3.3.2.5-1, the test requirements as in clause 6.5.3.2.5 in TS 38.521-1 [8] are also needed.

For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the test requirements in Table 6.5B.3.3.2.5-1 can be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.5-1: Requirements for inter-band within FR1 according to UE NR release

| EN-DC Configuration | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n3 | Rel-16 Rel-17 | E-UTRA Band 1, 11, 21, 32, 43, 50, 65, 74, 75 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band 77, 78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1_n5 | Rel-16 | E-UTRA Band 22, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1_n7 | Rel-16 | E-UTRA Band 32, 50, 51, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  |  | Rel-17 |  | NR Band n79 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  |  |  |
| DC_1_n8 | Rel-16 Rel-17 | E-UTRA Band 43 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1_n28 | Rel-15 Rel-16 | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  |  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
| DC_1_n41 | Rel-16 Rel-17 | E-UTRA Band 12, 13, 14, 17, 28, 29, 32, 42, 44, 45, 50, 51, 52, 67, 68, 71, 75, 76, 85 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1_n77 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  |  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  |  | Frequency range | 1915 | - | 1920 | 1.6 | 5 | 5, 7, 8 |
| DC_1_n78 | Rel-15 | E-UTRA Band 3, 11, 21, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
| DC_1_n79 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_2_n5 | Rel-15 | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-16 Rel-17 | E-UTRA Band 42, 48 NR band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n41 | Rel-16 Rel-17 | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Bands 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2_n66 | Rel-15 Rel-16 | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n77 | Rel-17 | E-UTRA Band 4, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n78 | Rel-15 | E-UTRA Band 24, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n1 | Rel-16 Rel-17 | E-UTRA Band 1, 11, 21, 32, 50, 65, 74, 75 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_3_n5 | Rel-16 Rel-17 | E-UTRA Band 5, 7, 8, 18,19, 26, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | Rel-15 Rel-16 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n8 | Rel-15 | E-UTRA Band 20, 28, 38, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | E-UTRA Band 7, 22, 41, 42, 43 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n28 | Rel-15 | NR band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-16 Rel-17 | E-UTRA Band 7, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_3_n41 | Rel-16 Rel-17 | E-UTRA Band 5, 8, 20, 26, 27, 28, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 14, 20 |
|  |  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n77 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 | Rel-15 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_5_n2 | Rel-16 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  |  | Rel-17 |  | E-UTRA Band 41, 43 NR Band n77 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_5_n66 | Rel-15 | E-UTRA Band 5, 6, 7, 8, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  |  | Rel-16 |  | E-UTRA Band 5, 6, 7, 8, 38 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  | Rel-17 |  | E-UTRA Band 26 |  | 859 |  |  | - | 869 |  | -27 |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 41, 42, 52 NR band n77 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_5_n77 |  |  | Rel-17 |  | E-UTRA Band 2, 4, 25, 30, 40, 65, 66, 70 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  |  |  | E-UTRA Band 41 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  | Frequency range |  | 1884.5 |  |  | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |  |
| DC_5_n78 | Rel-15 Rel-16 | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 7 |
| DC_7_n1 | Rel-16 Rel-17 | E-UTRA Band 32, 50, 51, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_7_n3 | Rel-16 Rel-17 | E-UTRA Band 5, 8, 20, 26, 27, 28, 67, 68 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  |  |  |  | E-UTRA band 42, 52 NR Band n77, n78 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_7_n5 |  |  | Rel-16 |  | E-UTRA Band 5, 26, 28, 42 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  | Rel-17 |  | E-UTRA Band 52 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  |  |  |  | NR Band n77, n78 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_7_n8 |  |  | Rel-16 |  | E-UTRA Band 20, 28, 67, 68 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  |  | Rel-17 |  | E-UTRA Band 42, 52 NR band n77, n78 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
| DC_7_n28 | Rel-15 | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7_n66 | Rel-16 Rel-17 | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7_n78 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_8_n1 | Rel-16 Rel-17 | E-UTRA Band 43 or NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_8_n3 | Rel-16 | E-UTRA Band 20, 28, 38, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 7, 22, 41, 42, 43 or NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n20 | Rel-16 | E-UTRA Band 67, 68 | 749 | - | 783 | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 7, 38, 69 | 2570 | - | 2690 | -50 | 1 | 2 |
|  |  | E-UTRA Band 8, 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 | Rel-16 | E-UTRA Band 38, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  |  | Rel-17 |  | E-UTRA Band 41 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range | 491 | - | 616 | -42 | 8 | 5, 17 |
|  |  | Frequency range | 491 | - | 616 | -26.2 | 6 | 14 |
| DC_8_n41 | Rel-16 | E-UTRA Band 28 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n77 | Rel-15 Rel-16 | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  |  |  |  |  | Frequency range |  | 1884.5 |  |  | - | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |
| DC_8_n78 |  |  | Rel-15 |  | E-UTRA band 34, 39, 40, 74 |  | FDL_low |  |  | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  | Rel-16 | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-17 | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  |  |  |  |  | Frequency range |  | 1884.5 |  |  | - | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |  |
| DC_8_n94_ULSUP-TDM | Rel-16 | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Rel-17 | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_11_n77 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_11_n79 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_12_n66 | Rel-15 | E-UTRA Band 41,53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_12_n78 | Rel-16 | E-UTRA Band 2, 7, 25, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 4, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_13_n2 | Rel-16 Rel-17 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_13_n66 | Rel-16 | E-UTRA Band 41, 53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_13_n77 | Rel-17 | E-UTRA Band 2, 4, 10, 25, 41, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_14_n2 | Rel-16 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_14_n66 | Rel-16 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_18_n77 | Rel-16 Rel-17 | E-UTRA Band 1, 3, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n78 | Rel-17 | E-UTRA Band 1, 3, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n1 | Rel-17 | E-UTRA Band 42 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_19_n77 | Rel-15 | E-UTRA Band 1, 3, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | Rel-15 | E-UTRA Band 1, 3, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Rel-17 | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20_n1 | Rel-16 | E-UTRA Band 22, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band 77, 78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n3 | Rel-16 | E-UTRA Band 7, 8 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 38, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n7 | Rel-16 | E-UTRA Band 68 | 776 | - | 783 | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 42, 52 | 3332 | - | 3432 | -50 | 1 | 2 |
|  |  | NR band n77, n78 | 3332 | - | 3432 | -50 | 1 | 2 |
|  |  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_20_n8 | Rel-16 | E-UTRA Band 28 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 7, 38 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n28 | Rel-16 Rel-17 | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20_n78 | Rel-16 | E-UTRA Band 1, 3, 7, 33, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n92_ULSUP-TDM | Rel-16 | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Rel-17 | E-UTRA Band 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21_n1 | Rel-17 | E-UTRA Band 40, 42 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 945 |  | 960 | -50 | 1 |  |
| DC_21_n77 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_21_n79 | Rel-15 | E-UTRA Band 3, 21, 34, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_25_n41 | Rel-15 | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_26_n41 | Rel-15 | E-UTRA Band 3, 5, 18, 19, 26, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Rel-17 | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 34, 39, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n3 | Rel-16 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-17 | E-UTRA Band 7, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_28_n5 | Rel-16 | E-UTRA Band 8, 40, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 24 | 1527 | - | 1559 | -50 | 1 |  |
|  |  | E-UTRA Band 30 | 2351 | - | 2360 | -50 | 1 |  |
|  |  | Frequency range | 557 | - | 672 | -42 | 8 | 5, 17 |
|  |  | Frequency range | 557 | - | 672 | -26.2 | 6 | 14 |
|  |  | Frequency range | 662 | - | 672 | -26.2 | 6 | 5 |
| DC_26_n78 | Rel-16 | E-UTRA Band 1, 3, 34, 39, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n7 | Rel-16 | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 52 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_28_n77 | Rel-15 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-17 | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 | Rel-15 Rel-16 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_28_n79 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_30_n5 | Rel-15 | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_39_n41 | Rel-15 | E-UTRA Band 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 Rel-17 | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_39_n79 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 8, 28, 41, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_40_n1 | Rel-16 Rel-17 | E-UTRA Band 7, 31, 32, 41, 45, 50, 72, 73, 74, 75 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_40_n41 | Rel-15 | Bands 1, 34, 39, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-16 | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_40_n78 | Rel-16 Rel-17 | E-UTRA Band 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 44, 45, 50, 51, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_40_n79 | Rel-16 Rel-17 | E-UTRA Band 1, 34, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_41_n28 | Rel-16 | E-UTRA Band 39, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-17 | E-UTRA Band 2, 3, 25 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 1 | 3, 9 |
| DC_41_n77 | Rel-15 Rel-16 | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n78 | Rel-15 Rel-16 | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_41_n79 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 5, 8, 18, 19, 26, 28, 34, 40, 44, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n5 | Rel-16 | E-UTRA Band 25, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n66 | Rel-16 Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_66_n2 | Rel-16 Rel-17 | E-UTRA Band 4, 10, 22, 24, 50, 66, 70, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | E-UTRA Band 42, 43 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n5 | Rel-15 | E-UTRA Band 5, 6, 7, 8, 26, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Rel-16 | E-UTRA Band 5, 6, 7, 8, 26, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 41, 42, 52 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n41 | Rel-16 Rel-17 | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 43, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 42, 48, NR band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n77 | Rel-17 | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_66_n78 | Rel-15 Rel-16 Rel-17 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 of TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 – 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink. NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB: B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

Table 6.5B.3.3.2.5-2: Void

### << CR implementation: Table below shall be removed completely >>

Table 6.5B.3.3.2.5-3: Void

### << CR implementation: Table below shall be removed completely >>

==============================================================

### << End of changes >>