3GPP TSG-RAN5 Meeting #98	R5-230268

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0426 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for support of multiple CEF reports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.306 table 4.2.18 defines the UE capability parameter “multipleCEF-Report-r17” which indicates whether the UE supports the storage and delivery of multiple CEF (connection establishment failure) reports upon request from the network as specified in TS 38.331. This capability is missing in TS 38.508-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PICS for support of the storage and delivery of multiple CEF reports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capability for support of storage and delivery of multiple CEF reports will remain incomplete in TS 38.508-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0302 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact; Secretary to resolve the ‘xx’ |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.4.4	Additional information

Table A.4.4-1: Additional information

| Item |  |  | Additional information |  |  | Ref. |  |  | Release |  |  | Mnemonic |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Support of ICMP or ICMP IPv6 |  |  | RFC 792 OR RFC 4443, RFC 4884 |  |  | NA |  |  | pc_IP_Ping |  |  | UE supports ICMP or ICMPv6 protocol to enable IP Ping Operation |  |  |
| 2 |  |  | Support of IMS |  |  | 24.229, Annex U |  |  | Rel-15 |  |  | pc_IMS_5GS |  |  |  |  |  |
| 3 |  |  | Support of rachReport |  |  | 38.306, 4.2.17 |  |  | Rel-16 |  |  | pc_rachReport_r16 |  |  | UE supports delivery of rachReport upon request from the network. |  |  |
| 4 |  |  | Support of GNSS |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_GNSS_location_r16 |  |  | UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. |  |  |
| 5 |  |  | Support of UL PDCP Packet Average Delay |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_PDCP_Delay_r16 |  |  | UE supports UL PDCP Packet Average Delay measurement and reporting in RRC_CONNECTED state |  |  |
| 6 |  |  | Support logged MDT |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_loggedMeasurements_r16 |  |  | UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. |  |  |
| 7 |  |  | Support of uncompensated barometric pressure measurement reporting |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_barometer_r16 |  |  | UE supports uncompensated barometric pressure measurement reporting upon request from the network. |  |  |
| 8 |  |  | Support of orientation information reporting |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_orientation_r16 |  |  | UE supports orientation information reporting upon request from the network. |  |  |
| 9 |  |  | Support of speed information reporting |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_speed_r16 |  |  | UE supports speed information reporting upon request from the network. |  |  |
| 10 |  |  | Support of Bluetooth measurements in RRC_CONNECTED state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_immMeasBT_r16 |  |  | UE supports Bluetooth measurements in RRC_CONNECTED state. |  |  |
| 11 |  |  | Support of WLAN measurements in RRC_CONNECTED state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_immMeasWLAN_r16 |  |  | UE supports WLAN measurements in RRC_CONNECTED state. |  |  |
| 12 |  |  | Support of Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_loggedMeasBT_r16 |  |  | UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. |  |  |
| 13 |  |  | Support of WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_loggedMeasWLAN_r16 |  |  | UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. |  |  |
| 14 |  |  | Support of SDT in RRC_INACTIVE state via Random Access Procedure |  |  | 38.306, 4.2.2 |  |  | Rel-17 |  |  | pc_ra_SDT_r17 |  |  | UE supports SDT via Random Access procedure in RRC_INACTIVE state |  |  |
| 15 |  |  | Support of SRB SDT in RRC_INACTIVE state |  |  | 38.306, 4.2.2 |  |  | Rel-17 |  |  | pc_srb_SDT_r17 |  |  | UE supports SRB SDT in RRC_INACTIVE state |  |  |
| 16 |  |  | Support of SDT in RRC_INACTIVE state via Configured Grant Type 1 |  |  | 38.306, 4.2.7.2 |  |  | Rel-17 |  |  | pc_cg_SDT_r17 |  |  | UE supports SDT via Configured Grant Type 1 in RRC_INACTIVE state |  |  |
| 17 |  |  | Support of NR NTN access |  |  | 38.306, 4.2.2 |  |  | Rel-17 |  |  | pc_nonTerrestrialNetwork_r17 |  |  | UE supports NR NTN access. |  |  |
| 18 |  |  | Support of RRC INACTIVE state in NTN |  |  | 38.331, 6.3.3 |  |  | Rel-17 |  |  | pc_inactiveStateNTN_r17 |  |  | UE supports RRC INACTIVE state in NTN |  |  |
| 19 |  |  | Support of RA-SDT in NTN |  |  | 38.331, 6.3.3 |  |  | Rel-17 |  |  | pc_ra_SDT_NTN_r17 |  |  | UE supports RA-SDT in NTN |  |  |
| 20 |  |  | Support of SRB-SDT in NTN |  |  | 38.331, 6.3.3 |  |  | Rel-17 |  |  | pc_srb_SDT_NTN_r17 |  |  | UE supports SRB-SDT in NTN |  |  |
| xx |  |  | Support of storage and delivery of multiple CEF reports |  |  | 38.306, 4.2.18 |  |  | Rel-17 |  |  | pc_multiple_CEF |  |  | UE supports the storage and delivery of multiple CEF reports upon request from the network |  |  |